| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 17–28 Mar. 2019 | Document: JVET-N0475 |
| --- | --- |

| Title: | Non-CE3/6: Enabling Transform Skip for ISP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tsung-chuan Ma Xiao-yu Xiu Yi-Wen Chen Xianglin Wang | Email: | tsung-chuanma@kwai.com xiaoyuxiu@kwai.com yiwenchen@kwai.com xianglinwang@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In current VVC, it is asserted that transform skip is not enable for ISP mode, i.e., transform skip is exclusively applied with ISP mode. This contribution proposes to enable transform skip for ISP in TU level. Based on the proposed implementation, the result shows that the BD-rate increases are -0.01 %, 0.00% and 0.02% for AI, RA and LD, respectively.

# Introduction

In VTM-4.0 [1], the ISP is exclusively applied with transform skip. It is well-known that the two-dimensional transform may not benefit for coding the frames with sharp edge objects, e.g., screen content video with detailed structure of characters and sharp program window boundary. Applying transform to prediction residual might be inefficient and even worse for coding those types of content. The transform skip is introduced to bypass two-dimensional transform for the better coding efficiency and video quality. However, it’s currently not applicable to ISP mode, which may limit the potentiality of ISP mode in coding those type of contents. In other words, additional coding gain can be expected when enabling the combination of ISP and the transform skip.

# Proposed Implementation

In this proposal, it is proposed to apply transform skip with ISP mode in TU level.

# Results

The proposed modification is integrated into VTM-4.0 [1] and the results of the proposed implementations are shown below against the VTM4.0 under CTC [3].

Table 1. Result of proposed modification under All Intra

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.06% | -0.03% | 104% | 101% |
| Class A2 | -0.01% | 0.11% | -0.06% | 105% | 102% |
| Class B | -0.01% | 0.00% | 0.05% | 106% | 106% |
| Class C | -0.02% | -0.09% | -0.06% | 103% | 104% |
| Class E | -0.01% | 0.04% | 0.01% | 104% | 104% |
| Overall | -0.01% | 0.01% | -0.01% | 104.43% | 103.67% |
| Class D | -0.01% | 0.00% | -0.20% | 106% | 104% |
| Class F (optional) | -0.19% | -0.08% | -0.14% | 104% | 104% |
| Class SCC 1080p | -0.25% | -0.12% | -0.14% | 103% | 108% |

Table 2. Result of proposed modification under Random Access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.01% | -0.05% | 100% | 98% |
| Class A2 | -0.02% | -0.09% | 0.08% | 103% | 102% |
| Class B | 0.00% | -0.05% | -0.16% | 101% | 103% |
| Class C | -0.01% | -0.13% | -0.42% | 103% | 105% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.07% | -0.16% | 102% | 102% |
| Class D | -0.03% | -0.10% | -0.30% | 101% | 101% |
| Class F (optional) | -0.17% | -0.17% | -0.28% | 104% | 105% |
| Class SCC 1080p | -0.25% | -0.08% | -0.06% | 103% | 104% |

Table 3. Result of proposed modification under Low Delay B

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.19% | 0.03% | 97% | 96% |
| Class C | -0.05% | 0.31% | -0.05% | 100% | 98% |
| Class E | 0.13% | 0.73% | 1.33% | 104% | 103% |
| Overall | 0.02% | 0.21% | 0.33% | 100% | 99% |
| Class D | 0.00% | 0.09% | -1.01% | 98% | 105% |
| Class F (optional) | -0.19% | -0.01% | -0.11% | 102% | 101% |
| Class SCC 1080p | -0.03% | 0.05% | 0.04% | 102% | 98% |

# Conclusion

In this proposal, the transform skip flag is applied with ISP mode and the result reportedly shows that the BD-rate difference are -0.01 %, 0.00% and 0.02% for AI, RA and LD, respectively. Therefore, in order to improve the encoding efficiency and video quality for video with sharp edge objects, e.g., screen content video, it is recommended to adopt the proposed modifications into the VVC working draft.

# References

| [1] | "VTM-4.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM". |
| --- | --- |
| [2] | S. De-Luxán-Hernández, H. Schwarz, D. Marpe and T. Wiegand, "CE3: Line-based intra coding mode (Tests 1.1.1 and 1.1.2)," Document JVET-M0102, Marrakesh , MA, 2019. |
| [3] | F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring, "JVET common test conditions and software reference configurations for SDR video," 2018. |

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).