| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0476 |
| --- | --- |

| Title: | Non-CE4: Shared Merge List on the Constraints of Picture Sizes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen Xiaoyu Xiu Tsung-Chuan Ma Xianglin Wang | Email: | yiwenchen@kwai.com xiaoyuxiu@kwai.com tsung-chuanma@kwai.com xianglinwang@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In current VVC, the picture width and height have to be multiple of 4 (the smallest CU size). However, when a picture with resolution (8M+4) x (8N+4) is input to the VTM-4.0, the encoder crashes because ALF only supports 4x4 SIMD filtering and (8M+4) x (8N+4) picture results in 2xN/Nx2 chroma region which triggers the encoder crash. It could be fixed easily by supporting 2xN/Nx2 ALF filtering. Another issue is that the merge sharing node of the 4x4 block located at the bottom-right corner of a picture may have region located outside the picture and it may result in undefined operations for MVP derivation. To solve the issue, it is proposed that the picture width and picture height are not necessarily equal to the minimum CU width/height. Instead, the picture width and height are both constrained to be multiples of 8 which is aligned with HEVC; alternatively, to provide more flexibility, the picture width is constrained to be multiples of 8 and picture height is constrained to be multiples of 4. Another solution is that when a boundary of a merge sharing node is located outside a picture, applying a truncation to the boundary to move the boundary of the merge sharing node to be aligned with a picture boundary. The proposed modifications do not bring impact to the coding performance under CTC.

# Introduction

The constraints that picture width and height have to be multiples of the minimum CU width or height (which is 4 in current VVC) may cause some problems when some coding tools are enabled. As shown in Figure 1, when coding a picture of 388x260 resolution using VTM-4.0 [1], the remaining region has either width or height equal to 4 in the right and bottom areas of the picture. Such a picture resolution may cause some issues for certain VVC coding tools such as ALF and share merge list. The issue of ALF could be fixed easily by supporting 2xN/Nx2 SIMD filtering.

Figure 1 The remaining 4xN, Nx4 and 4x4 region in a (8M+4) x (8N+4) picture.

Regarding the issue of the shared merge list, as shown in Figure 2, the merge sharing node of the remaining 4x4 block located at the bottom-right corner of the picture may have region located outside the picture boundary,

Figure 2 The merge sharing node for the remaining 4x4 block has region located outside the picture boundary.

# Proposed Methods

Two solutions are proposed in this contribution to solve the issue related to the shared merge list.

In the first proposed scheme, the picture width and height are constrained to be multiples of 8 which is aligned with HEVC. Alternatively, to provide more flexible picture resolution, the picture width is constrained to be multiples of 8 while the picture height is constrained to be multiples of 4.

In the second proposed scheme, when a boundary of a merge sharing node is located outside a picture, applying a truncation to the boundary to move the boundary of the merge sharing node to be aligned with a picture boundary.

# Results

The proposed modification does not introduce impact to the coding performance under CTC [2].

# Conclusion

In this proposal, two solutions are proposed to solve the issue of shared merge list when the input picture resolution is of (8M+4) x (8N+4). It is recommended to adopt the proposed fix into the VVC to prevent undefined MVP derivation caused by the sharing merge node located outside the picture boundary.

# Reference

- VTM-4.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, doc. JVET-M1010, January 2019.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).