| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0477-v2 |
| --- | --- |

| Title: | On luma mapping with chroma scaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yan Ye, Jie Chen, Ruling Liao 525 Almanor Avenue Sunnyvale, CA 94085 USA | Tel: Email: | yan.ye@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

# Abstract

At the 13th JVET meeting, luma mapping with chroma scaling (LMCS) (previously referred to as adaptive in-loop reshaper in JVET-M0427) was adopted into the VVC draft 4. In this contribution, three modifications to the chroma scaling process are proposed. In the current LMCS method, the chroma scaling process is performed at the block level, and the scaling factor is determined based on the average value of the corresponding luma prediction block. In the first proposed modification, the scaling factor computation is modified to use only up to 4x4 samples of the corresponding luma block. It is reported that the performance impact is minimal. Under the CTC, the overall impact for {Y, U, V} is {0.00%, -0.02%, -0.01%} for RA and {-0.04%, 0.20%, -0.03%} for LDB. In the second modification, it is proposed to bypass the chroma scaling process if the cbf’s of Cb and Cr are both zero. The second modification produces bit-exact coding results, but is asserted to simplify the decoder.

In the v2 revision, a third modification is proposed. In the third modification, for intra coded blocks, the top-left reference sample is used to derive the chroma scaling factor. This modification is only applied to the conventional intra coding mode. For all other modes, the proposed simplified luma averaging is used. Under the CTC, the overall impact for {Y, U, V} is {0.00%, -0.02%, -0.01%} for RA and {-0.04%, 0.20%, -0.03%} for LDB.

# Chroma scaling in VVC draft 4

Chroma residual scaling depends on the average value of the corresponding luma prediction block. In VVC draft 4, the average value of the luma prediction block, avgY, is computed using all samples in the prediction block. Once the average luma value is determined, the chroma scaling factor is computed by finding the piece index of the piecewise linear model to which avgY belongs.

# Proposal

It is asserted that using the entire luma prediction block to calculate the average value is complicated because the luma block can be as large as 128x128 luma samples. It is also unnecessary because a small deviation from the actual average luma value will still lead to the same piece index of the piecewise linear model being identified. Therefore, it is proposed to use only up to 4x4 luma samples of the luma prediction block. The following modified averaging process is proposed:

- If a dimension of the luma block is no larger than 4, then all sample positions in that dimension are used in the averaging process;

- Otherwise (the dimension is larger than 4), then only 4 evenly-spaced sample positions in that dimension are used in the averaging process.

Further, because the chroma scaling process is applied on the chroma residual signal. When there is no non-zero residual coefficients for both Cb and Cr, the averaging process and the subsequent derivation of chroma scaling factor can be completely bypassed. Currently in VTM4.0, these processes are invoked without checking the tu_cbf_cb and tu_cbf_cr conditions. It is proposed to add these conditions to the VTM and the VVC draft text to ensure decoder does not perform unnecessary operations. It is noted that this proposed modification does not change RD performance.

In v2 revision, a third modification is proposed. For blocks coded using the conventional intra mode, the top-left reference sample is used to derive the chroma scaling factor. For all other blocks, the simplified luma averaging as proposed above is used to derive the chroma scaling factor. It is asserted that using reference sample instead of prediction sample can reduce intra processing latency, especially for the encoder.

# Simulation results

The proposed modifications are implemented on top of VTM4.0. Simulation results of the first proposed modifications (simplified luma averaging) for the RA and LDB configurations are provided in Table 1. Negligible performance impact is observed. The overall impact for {Y, U, V} is {0.00%, -0.02%, -0.01%} for the RA configuration and {-0.04%, 0.20%, -0.03%} for the LDB configuration. The second proposed modification does not impact RD performance.

Table 2 provides the simulation results for using the top-left reference sample for intra coded blocks and using the simplified luma averaging process for all other blocks (modification 3). The overall impact for {Y, U, V} is {0.00%, -0.02%, -0.01%} for the RA configuration and {-0.04%, 0.20%, -0.03%} for the LDB configuration.

Table 1. Simulation results of simplifying the luma averaging process (modification 1)

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.00% | -0.03% | 100% | 101% |
| Class A2 | 0.00% | -0.02% | -0.04% | 100% | 101% |
| Class B | 0.00% | -0.05% | -0.05% | 100% | 100% |
| Class C | 0.01% | 0.00% | 0.06% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.02% | -0.01% | 100% | 100% |
| Class D | 0.01% | -0.04% | -0.08% | 100% | 100% |
| Class F | 0.00% | 0.02% | -0.07% | 100% | 100% |

|  |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.05% | 0.24% | 0.14% | 100% | 102% |
| Class E | -0.06% | 0.31% | -0.28% | 99% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.01% | -0.03% | -0.33% | 100% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 2. Simulation results of using top-left reference sample for intra coded blocks

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.13% | 0.05% | 99% | 99% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.01% | -0.05% | -0.05% | 99% | 98% |
| Class C | 0.00% | 0.05% | -0.05% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.03% | 0.19% | 0.05% | 100% | 97% |
| Class F | 0.00% | 0.03% | -0.02% | 100% | 99% |

|  |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.06% | 0.42% | -0.04% | 99% | 95% |
| Class E | -0.03% | 0.35% | -0.59% | 99% | 95% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.03% | 0.18% | -0.85% | 100% | 93% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

| Random Access. Over VTM-4.0. Y; U; V; EncT; DecT. Class A1; 0.01%; -0.13%; 0.05%; 99%; 99%. Class A2; #VALUE!; #VALUE!; #VALUE!; #NUM!; #NUM!. Class B; 0.01%; -0.05%; -0.05%; 99%; 98%. Class C; 0.00%; 0.05%; -0.05%; 100%; 100%. Class E. Overall; #VALUE!; #VALUE!; #VALUE!; #NUM!; #NUM!. Class D; -0.03%; 0.19%; 0.05%; 100%; 97%. Class F; 0.00%; 0.03%; -0.02%; 100%; 99%. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

# Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).