| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0479-v3 |
| --- | --- |

| Title: | CE1-related: Hadamard transform domain in-loop filter also using bottom/right reconstructed samples |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sergey Ikonin Victor Stepin Alexander Karabutov Jianle Chen | Tel: Email: | +7-495-660-44-59 sergey.ikonin@huawei.com stepin.victor@huawei.com karabutov.alexander@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., LTD |  |  |

_____________________________

# Abstract

This document is a follow-up contribution to JVET-N0478 and test CE1-2.3 in which Hadamard transform domain in-loop filter was tested having access to top and left neighboring reconstructed samples within current CTU. However, for in-loop filters bottom and right samples are also available. This contribution reports results of Hadamard transform domain in-loop filter also having access to bottom/right reconstructed samples.

The simulation over VTM-4.0 shows -0.41%/-0.64% of the luma BD-rate change with 106%/103% of encoding time and 104%/101% of decoding time for AI/RA configurations correspondingly.

This contribution also considers aspect of using reconstructed samples from other than current CTU.

# Introduction

In CE1-2.3 the Hadamard transform domain filter was tested as in-loop filter having access to left and top neighboring reconstructed samples. However, for in-loop filter bottom and right samples are also available. This document reports performance of Hadamard transform based in-loop filter if bottom-right samples are also available. The aspect of using of reconstructed samples from other than current CTU is also studied.

# Test description

The proposed filter was tested in following conditions:

- Test 1. The filter has access to left, top, right and bottom neighboring samples if they belong to current CTU.

- Test 2. The filter has access to left, top, right and bottom neighboring samples if they belong to current, left and right CTU.

- Test 3. The filter has access to left, top, right and bottom neighboring samples if they belong to any neighboring CTU.

# Simulation results

Simulations were carried out on top of VTM-4.0 software under common test conditions described in JVET-M1010.

Table 1: Test 1 results (left, top, right and bottom samples of current CTU are available)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.23% | 0.51% | 0.43% | 105% | 104% |
| Class A2 | -0.40% | 0.45% | 0.30% | 105% | 104% |
| Class B | -0.33% | 0.48% | 0.52% | 105% | 104% |
| Class C | -0.64% | 0.33% | 0.23% | 106% | 104% |
| Class E | -0.44% | 0.42% | 0.36% | 106% | 101% |
| Overall | -0.41% | 0.44% | 0.38% | 106% | 104% |
| Class D | -0.55% | 0.50% | 0.19% | 105% | 99% |
| Class F | -0.84% | 0.31% | 0.32% | 103% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.35% | -0.13% | -0.01% | 103% | 101% |
| Class A2 | -0.88% | -0.51% | -0.31% | 103% | 101% |
| Class B | -0.65% | 0.20% | -0.13% | 103% | 102% |
| Class C | -0.64% | 0.07% | 0.06% | 102% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.64% | -0.04% | -0.09% | 103% | 101% |
| Class D | -0.65% | -0.01% | 0.00% | 101% | 98% |
| Class F | -0.90% | -0.02% | 0.00% | 101% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.69% | 1.03% | 0.69% | 103% | 102% |
| Class E | -0.36% | 1.21% | 1.55% | 100% | 102% |
| Overall |  |  |  |  |  |
| Class D | -0.85% | 0.80% | 0.70% | 103% | 104% |
| Class F | -0.82% | 0.00% | -0.28% | 102% | 102% |

Table 2: Test 2 results (same as Test1 but samples from left and right CTU are also available)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.24% | 0.51% | 0.38% | 107% | 105% |
| Class A2 | -0.41% | 0.43% | 0.30% | 107% | 104% |
| Class B | -0.33% | 0.46% | 0.53% | 107% | 105% |
| Class C | -0.65% | 0.31% | 0.41% | 107% | 108% |
| Class E | -0.46% | 0.44% | 0.31% | 107% | 107% |
| Overall | -0.42% | 0.43% | 0.40% | 107% | 106% |
| Class D | -0.55% | 0.44% | 0.21% | 107% | 106% |
| Class F |  |  |  |  |  |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.37% | -0.03% | -0.02% | 104% | 101% |
| Class A2 | -0.88% | -0.51% | -0.27% | 104% | 101% |
| Class B | -0.65% | 0.19% | -0.15% | 104% | 102% |
| Class C | -0.66% | 0.06% | -0.01% | 103% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.64% | -0.03% | -0.11% | 104% | 102% |
| Class D | -0.67% | 0.38% | -0.20% | 103% | 103% |
| Class F | -0.89% | 0.04% | 0.08% | 102% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.69% | 1.13% | 0.71% | 103% | 101% |
| Class E | -0.57% | 0.94% | 0.69% | 101% | 102% |
| Overall |  |  |  |  |  |
| Class D | -0.89% | 0.97% | 1.24% | 104% | 104% |
| Class F | -0.99% | 0.14% | -0.40% | 103% | 102% |

Table 3: Test 3 results (same as Test1 but samples from any CTU are available)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.25% | 0.47% | 0.41% | 106% | 105% |
| Class A2 | -0.42% | 0.48% | 0.35% | 106% | 104% |
| Class B | -0.35% | 0.49% | 0.49% | 106% | 105% |
| Class C | -0.66% | 0.28% | 0.35% | 107% | 106% |
| Class E | -0.46% | 0.34% | 0.36% | 107% | 106% |
| Overall | -0.43% | 0.42% | 0.40% | 107% | 105% |
| Class D | -0.55% | 0.35% | 0.26% | 107% | 107% |
| Class F | -0.87% | 0.24% | 0.23% | 104% | 104% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.35% | -0.07% | -0.09% | 104% | 101% |
| Class A2 | -0.89% | -0.45% | -0.30% | 103% | 101% |
| Class B | -0.66% | 0.22% | -0.29% | 104% | 101% |
| Class C | -0.66% | 0.11% | -0.11% | 103% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.64% | 0.00% | -0.20% | 103% | 102% |
| Class D | -0.66% | -0.11% | 0.12% | 103% | 102% |
| Class F | -0.88% | 0.10% | 0.00% | 102% | 102% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.71% | 1.10% | 0.83% | 103% | 102% |
| Class E | -0.47% | 0.32% | 1.07% | 100% | 101% |
| Overall |  |  |  |  |  |
| Class D | -0.86% | 0.80% | 0.84% | 104% | 103% |
| Class F | -0.87% | 0.12% | 0.11% | 103% | 102% |

# Conclusion

Having access to bottom-right reconstructed samples brings additional coding gain to Hadamard transform domain in-loop filter in comparison to CE1-2.3 which was limited by left-top samples only. Simulation over VVC4.0 reportedly shows -0.6% of luma BD-rate change for RA configuration. It is recommended to include Hadamard transform domain in-loop filter into next release of VVC.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).