| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, March 2019 | Document: JVET-N0480-V2 |
| --- | --- |

| Title: | CE13-1.2: Adaptive convolutional neural network loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hujun Yin, Rongzhen Yang, Xiaoran Fang, Shoujiang Ma, 2200 Mission College Blvd, Santa Clara, CA, 95054, USA | Tel: Email: | +1 408 410 5081 {Hujun.yin, Rongzhen.yang} @intel.com |
| Source: | Intel Corporation |  |  |

_____________________________

# Abstract

Adaptive Convolutional Neural Network Loop Filter (ACNNLF) is initially proposed in JVET-M0566 and evaluated in this document as Core Experiment 13-1.2a (CE13-1.2a) [1].

In this contribution, only the mandatory CE13-1.2a is tested and the optional CE13-1.2b is not tested due to lack of time.

For the purpose of the comparison with CE13-1.1, the ACNNLF parameter quantization method was aligned with M0159 [2]  to reduce quantization from 8-bits in JVET-M0566 to 6-bits for all weights. Compared to M0566 [3], the decoding time overhead was optimized and reduced to 27%, but performance loss due to quantization is significant.

After the v1 result to meet CE deadline, the quantized models was improved by applying different inner clipping thresholds to reduce some quantization errors. The later results were presented in this version with the original (v1) result kept as record.

Compared with VTM-4.0-RA, the proposed ACNNLF achieves -1.14%, -0.21%, and -1.18% BD-rates for Y, U, and V, respectively, for Class A1 video sequences; -0.98%, -14.37%, and -16.96% BD-rates for Y, U, and V, respectively, for Class A2 video sequences; -0.55%, -21.79%, and -20.04% BD-rates for Y, U, and V, respectively, for Class B video sequences; and 0.09%, -2.75%, and -1.43% BD-rates for Y, U, and V, respectively, for Class C video sequences.

# Introduction

In the 13th JVET meeting, an adaptive convolution neural network loop filter (ACNNLF) design is proposed in M0566 [3] and defined as CE13-1.2 in JVET-M1033 [1].

The tested ACNNLF has the following properties:

- Two layer CNN, as shown in the figure 2;

- Up to 3 ACNNLFs for luma and 3 ACNNLFs for chroma adaptively trained with video data;

- The encoder selects the best ACNNLF for each of luma and chroma in each CTB from the trained set of ACNNLFs, to perform encoding. The weights of the trained set of ACNNLFs are signaled in the tile group header and the index of selected ACNNLF is signaled in the coded stream in CTU level;

The decoder receives the ACNNLF weights (up to 3 sets for luma and 3 set for chroma) and the CTU level selected ACNNLF and uses the selected ACNNLF to perform the filtering process for each luma and chroma block during frame reconstruction.

Figure 1. ACNNLF for luma

The parameters of ACNNLF are signaled in the tile group header. In RA configuration, only the I-slice contains ACNNLF parameters of this RAS. The on/off control is applied at the tile group header level for luma and chroma components, and the acnnlf_luma_slice_enable_flag and acnnlf_chroma_slice_enable_flag syntax elements are used to indicate ACNNLF enable/disable. CTB level information indicates which ACNNLF is selected or if there is no ACNNLF applied in the CTB.

# Experimental results

The following sections describe the settings and results for the experiments using VTM 4.0 following the template recommended in JVET-M1006 [5].

## Training stage

Online training based on the video sequence itself is performed for each Random Access Segment. To align with CE13-1.1 (JVET-M0159), pictures with temporal ID equal to 0 or 1 in the RAS are used for training, QP values {22, 27, 32, 37} are used.

Table 1. ACNNLF Training Setup

| Mandatory | Information in Training Stage |  |  |
| --- | --- | --- | --- |
|  | Basic Settings: |  |  |
|  |  | learning rate: | 0.0055 |
|  |  | optimizer: | ADAM |
|  |  | batch size: | 128 |
|  |  | epoch: | 148* |
|  |  | loss function: | L1 |
|  |  | training GPU: | GTX 1080 Ti |
|  |  | training time: | 15 minutes* |
|  |  | framework: | TensorFlow |
| Optional | Other Settings: |  |  |
|  |  | Patch Size | 32x32 |
|  |  | Dataset | CTC Sequences |
|  |  | QP set for generating training dataset | 22, 27, 32, 37 |

* Measured with A1/A2 video streams, per RAS with training procedure defined in figure 2

## Inference stage

In inference stage, at encoder, all three ACNNLFs are tested to choose the best ACNNLF. At decoder, only the signaled ACNNLF for each CTB is used reconstruct the frame. Currently the ACNNLF inference module is aligned with M0159 with 6 bit fixed point for weights and 32 bits for other parameters.

Table 2. ACNNLF Inference Stage Setup

|  | Network Details |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Total Conv. Layers | Total FC Layers | Param. Num | Mem.P(MB) | Mem.T(MB) |
| ACNNLF | 2 | 0 | 692x3 (Luma) 402x3 (Chroma) | 0.0028 | 0.0448 |

## Performance Loss for Post Training Quantization

The performance loss due to new 6 bit quantization without optimization is significant, especially in some streams, some examples were evaluated here by using Tensor Flow 8 bits quantization and inference as comparison:

Table 3. Post Training Weight Quantization Performance Loss (Luma Only)

| Stream Name | 8 Bits Gain in TF-Only Evaluation | 6 bits Post Training Quantization Gain | Gain Loss (%) |
| --- | --- | --- | --- |
| A1 / Campfire | -5.87% | -3.54% | 39.7% |
| A2 / ParkRunning | -2.52% | -2.18% | 13.4% |
| B / BQTerrace | -5.21% | -3.96% | 24.0% |

The quantization needs to be taken into consideration during training to minimize the quantization loss. The results with quantization aware training will be updated in next meeting.

## Experimental Results

The proposed ACNNLF is implemented on top of VTM-4.0. The simulations were performed following the JVET common test conditions [4].  Only RA video sequence are tested, Table 3 shows the results of the proposed scheme on Random Access for Class A1/A2, B and C, as the first version provided to meet CE13 deadline:

Table 4.  Experimental Results of the Proposed Scheme (Random Access) (Version 1)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.27% | -0.17% | -1.29% | 99% | 126% |
| Class A2 | -0.39% | -8.69% | -9.95% | 104% | 148% |
| Class B | -0.57% | -15.37% | -14.95% | 104% | 134% |
| Class C | 0.05% | -1.03% | -0.46% | 102% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.31% | -7.17% | -7.35% | 102% | 126% |

The new result in this version is evaluated by using the new quantized models, which uses different inner clipping threshold in quantization, and no changes in the VTM implementation code:

Table 5.  Experimental Results of the Proposed Scheme (Random Access) (Version 2)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.14% | -0.21% | -1.18% | 101% | 128% |
| Class A2 | -0.98% | -14.37% | -16.96% | 106% | 151% |
| Class B | -0.55% | -21.79% | -20.04% | 104% | 134% |
| Class C | 0.09% | -2.75% | -1.43% | 102% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.58% | -10.91% | -10.69% | 103% | 127% |

In the experiment, training is done separately in Tensor Flow. Therefore, the VTM-4.0 runtime does not include ACNNLF training time, but includes additional inference time in decoding that is comparable to CE13-1.1.

# Conclusions

This contribution presents an ACNNLF design with 3 classes of CNN based loop filters, where each filter has only 2 CNN layers and 692 parameters. The 3 ACNNLFs are adaptively trained with video sequence data. The best ACNNLF is selected for luma and chroma respectively for each CTB at encoder and indicated to decoder in coded stream with 2 bit indicator at CTB level. Compared with VTM-4.0-RA, the proposed ACNNLF achieves -1.14%, -0.21%, and -1.18% BD-rates for Y, U, and V, respectively, for Class A1 video sequences; -0.98%, -14.37%, and -16.96% BD-rates for Y, U, and V, respectively, for Class A2 video sequences; -0.55%, -21.79%, and -20.04% BD-rates for Y, U, and V, respectively, for Class B video sequences; and 0.09%, -2.75%, and -1.43% BD-rates for Y, U, and V, respectively, for Class C video sequences. The decoding time in the RA is 127% on VTM 4.0.

# References

- Ying Ming Li, Shan Liu, and Kei Kawamura, " Description of Core Experiment 13 (CE13): Neural Network based Filter for Video Coding" JVET-M1033, Jan 2019

- Y.-L. Hsiao, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, “AHG9: Convolution neural network loop filter,” Document of Joint Video Experts Team, JVET-M0159. Jan 2019

- H. Yin, R. Yang, X. Fan, S.Ma, and Y.Yu, “Adaptive convolutional neural network loop filter”, Document of Joint Video Experts Team, JVET-M0566. Jan 2019

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-M1010

- S. L. Yiming Li, "Methodology and reporting template for neural network coding tool testing," JVET-M1006, 2019

# Patent rights declaration(s)

Intel may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).