| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-Nxxxx-v1 |
| --- | --- |

| Title: | Non-CE2: Gbi inheritance for constructed affine merge candidate |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Chen, Ruling Liao, Yan Ye | Tel: Email: | jiechen.cj@alibaba-inc.com ruling.lrl@alibaba-inc.com yan.ye@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

In current VVC, Generalized Bi-Prediction (GBi) and affine merge are both employed. With GBi, each inter-coded block is further assigned a bi-prediction weight index in addition to the motion information. In the current affine merge design, for inherited affine merge candidate, the GBi of neighboring block is inherited, while for constructed affine merge candidate, the GBi index is set to the default value (i.e. equal weight). To align the design of GBi index derivation between these two types of affine merge candidates, it is proposed to derive a GBi index for constructed affine merge candidate instead of always setting to the default value. Simulation results reportedly show that the harmonization achieves 0.02% and xx% BD rate reduction for RA Main10 and LDB Main 10 configurations, respectively.

.

Introduction

In the current VVC, there are three types of affine merge candidates: inherited affine merge candidates, constructed affine merge candidates and zero MVs. For inherited affine merge candidates, the Gbi index of the neighboring blocks is inherited together with motion information. So the candidates may have different Gbi indices depending on the neighboring blocks. However, for the constructed merge candidates and zero MVs, the Gbi index is fixed to default value (i.e. equal weight) regardless the GBi index of neighboring blocks’. This creates a design discrepancy in the affine merge mode.

Proposal

To align the Gbi design of inherited affine merge candidate and constructed affine merge candidate, it is proposed to adaptively generate a Gbi index for the constructed affine merge candidate. First, the Gbi index of the bottom-right CP is set to 2 (i. e. equal weight) since we don’t have temporal GBi information. And then a GBi index is derived from the GBi indices of two or three control points of the current candidate with the following methods:

- Method 1: if at least two control points have the same GBi index, then this GBi index value is assigned to the candidate; otherwise the GBi index of the current constructed candidate is set to 2.

- Method 2: if one of the control point GBi indices is 0 and the other one is 4 (0 and 4 are the mirroring weight pairs with negative weight value, (-1/4, 5/4) and (5/4, -1/4)), then the GBi index of the current constructed candidate is set to 2; otherwise a GBi index is derived with method 1 for the current constructed affine candidate.

- Method 3: Divide the range of GBi index {0,1,2,3,4} into three groups {0},{1,2,3} and {4}. If all of the control point’s GBi indices are from the same group, a GBi index is derived with method 1 for the current constructed candidate; otherwise, its GBi index is set to 2.

Simulation results

The proposed modification is implemented on top of VTM4.0. Simulation results for the RA and LDB configurations are provided in Table 1.

- Method 1:

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.03% | -0.07% | 100% | 101% |
| Class A2 | -0.02% | -0.07% | -0.10% | 101% | 101% |
| Class B | -0.02% | -0.09% | 0.05% | 100% | 99% |
| Class C | -0.01% | -0.04% | -0.12% | 100% | 96% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.06% | -0.05% | 100% | 99% |
| Class D | -0.01% | -0.05% | 0.10% | 100% | 94% |
| Class F | -0.03% | -0.02% | -0.07% | 99% | 97% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.05% | -0.20% | 102% | 106% |
| Class C | -0.02% | 0.13% | -0.29% | 100% | 101% |
| Class E | 0.05% | -0.23% | -0.41% | 100% | 103% |
| Overall | 0.00% | 0.01% | -0.29% | 101% | 103% |
| Class D | -0.03% | 0.27% | -0.60% | 100% | 96% |
| Class F | -0.22% | 0.30% | -0.28% | 99% | 100% |

- Method 2

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.15% | 0.01% | 102% | 102% |
| Class A2 | -0.03% | -0.08% | -0.02% | 102% | 102% |
| Class B | -0.02% | -0.04% | 0.05% | 102% | 101% |
| Class C | -0.01% | -0.01% | -0.13% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.06% | -0.02% | 102% | 102% |
| Class D | -0.02% | -0.04% | 0.10% | 101% | 102% |
| Class F | -0.04% | -0.01% | -0.03% | 101% | 101% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.05% | 0.03% | 0.23% | 102% | 102% |
| Class E | -0.04% | -0.06% | -0.31% | 102% | 103% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.01% | -0.03% | -0.18% | 101% | 102% |
| Class F | -0.09% | 0.04% | 0.05% | 102% | 103% |

- Method 3

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | -0.03% | -0.05% | -0.04% | 102% | 102% |
| Class B | -0.02% | -0.01% | 0.06% | 102% | 101% |
| Class C | 0.00% | -0.08% | -0.17% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.02% | 0.01% | 0.11% | 101% | 102% |
| Class F | -0.03% | -0.04% | -0.06% | 102% | 101% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.05% | 0.22% | -0.16% | 102% | 102% |
| Class E | -0.07% | 0.25% | 0.42% | 102% | 103% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.03% | -0.14% | -0.39% | 102% | 101% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Conclusion

It is proposed to align GBi index inheritance for both inherited candidate and constructed candidate. This removes the design discrepancy, and achieves BD-rate reduction of 0.02% in RA and xx% in LDB. It is suggested to adopt this proposal to next version of VVC draft.

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).