| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N481-v4 |
| --- | --- |

| Title: | Non-CE2: Gbi inheritance for constructed affine merge candidate |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Chen, Ruling Liao, Yan Ye | Tel: Email: | jiechen.cj@alibaba-inc.com ruling.lrl@alibaba-inc.com yan.ye@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

In current VVC, Bi-prediction with CU weights (BCW, or GBi) and affine merge are both employed. With BCW, each inter-coded block is further assigned a bi-prediction weight index in addition to the motion information. In the current affine merge design, for inherited affine merge candidate, the BCW of neighboring block is inherited, while for constructed affine merge candidate, the BCW index is set to the default value (i.e. equal weight). To align the design of BCW index derivation between these two types of affine merge candidates, it is proposed to derive a BCW index for constructed affine merge candidate instead of always setting to the default value. Simulation results reportedly show that the harmonization achieves 0.02% and 0.05% BD rate reduction for RA Main10 and LDB Main 10 configurations, respectively.

.

Introduction

In the current VVC, there are three types of affine merge candidates: inherited affine merge candidates, constructed affine merge candidates and zero MVs. For inherited affine merge candidates, the BCW index of the neighboring blocks is inherited together with motion information. So the candidates may have different BCW indices depending on the neighboring blocks. However, for the constructed merge candidates and zero MVs, the BCW index is fixed to default value (i.e. equal weight) regardless the BCW index of neighboring blocks’. This creates a design discrepancy in the affine merge mode.

Proposal

To align the BCW design of inherited affine merge candidate and constructed affine merge candidate, it is proposed to adaptively generate a BCW index for the constructed affine merge candidate. First, the Gb BCW index of the bottom-right CP is set to 2 (i. e. equal weight) since we don’t have temporal BCW information. And then a BCW index is derived from the BCW indices of two or three control points of the current candidate with the following methods:

- Method 1: if at least two control points have the same BCW index, then this BCW index value is assigned to the candidate; otherwise the BCW index of the current constructed candidate is set to 2.

- Method 2: if one of the control point BCW indices is 0 and the other one is 4 (0 and 4 are the mirroring weight pairs with negative weight value, (-1/4, 5/4) and (5/4, -1/4)), then the BCW index of the current constructed candidate is set to 2; otherwise a BCW index is derived with method 1 for the current constructed affine candidate.

- Method 3: Divide the range of BCW index {0,1,2,3,4} into three groups {0},{1,2,3} and {4}. If all of the control point’s BCW indices are from the same group, a BCW index is derived with method 1 for the current constructed candidate; otherwise, its BCW index is set to 2.

Simulation results

The proposed modification is implemented on top of VTM4.0. Simulation results for the RA and LDB configurations are provided in Table 1.

- Method 1:

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.03% | -0.07% | 100% | 101% |
| Class A2 | -0.02% | -0.07% | -0.10% | 101% | 101% |
| Class B | -0.02% | -0.09% | 0.05% | 100% | 99% |
| Class C | -0.01% | -0.04% | -0.12% | 100% | 96% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.06% | -0.05% | 100% | 99% |
| Class D | -0.01% | -0.05% | 0.10% | 100% | 94% |
| Class F | -0.03% | -0.02% | -0.07% | 99% | 97% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.05% | -0.20% | 102% | 106% |
| Class C | -0.02% | 0.13% | -0.29% | 100% | 101% |
| Class E | 0.05% | -0.23% | -0.41% | 100% | 103% |
| Overall | 0.00% | 0.01% | -0.29% | 101% | 103% |
| Class D | -0.03% | 0.27% | -0.60% | 100% | 96% |
| Class F | -0.22% | 0.30% | -0.28% | 99% | 100% |

- Method 2

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.03% | -0.06% | 102% | 101% |
| Class A2 | -0.02% | -0.08% | -0.11% | 102% | 101% |
| Class B | -0.03% | -0.07% | 0.06% | 102% | 101% |
| Class C | -0.01% | -0.04% | -0.12% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.06% | -0.04% | 102% | 101% |
| Class D | -0.01% | -0.05% | 0.09% | 101% | 102% |
| Class F | -0.03% | -0.02% | -0.07% | 101% | 101% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.19% | -0.22% | 101% | 103% |
| Class C | -0.02% | 0.23% | 0.08% | 102% | 101% |
| Class E | -0.01% | 0.02% | -0.55% | 102% | 102% |
| Overall | -0.02% | 0.00% | -0.20% | 101% | 102% |
| Class D | -0.04% | 0.14% | -0.64% | 102% | 101% |
| Class F | -0.11% | 0.23% | -0.41% | 102% | 102% |

- Method 3

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.02% | -0.09% | 102% | 101% |
| Class A2 | -0.02% | 0.01% | -0.12% | 102% | 101% |
| Class B | -0.02% | -0.14% | 0.13% | 102% | 101% |
| Class C | -0.01% | -0.01% | -0.14% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.05% | -0.03% | 101% | 101% |
| Class D | -0.01% | -0.02% | 0.13% | 101% | 102% |
| Class F | -0.04% | -0.05% | -0.09% | 101% | 101% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | -0.02% | -0.08% | 101% | 103% |
| Class C | -0.05% | 0.04% | 0.05% | 101% | 100% |
| Class E | -0.07% | -0.25% | -0.27% | 102% | 103% |
| Overall | -0.05% | -0.06% | -0.08% | 101% | 102% |
| Class D | -0.06% | 0.36% | -0.44% | 101% | 101% |
| Class F | -0.10% | -0.20% | -0.10% | 101% | 102% |

Conclusion

It is proposed to align BCW index inheritance for both inherited candidate and constructed candidate. This removes the design discrepancy, and achieves BD-rate reduction of 0.02% in RA and 0.05% in LDB. It is suggested to adopt this proposal to next version of VVC draft.

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).