| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0483-v1 |
| --- | --- |

| Title: | Non-CE4: Simplification of triangle partition and SBT combination |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao, Yan Ye, Jie Chen Radiance Jin Hui Tower, Building No.6, The 4 District Wangjing Dong Yuan, Chaoyang District, Beijing, China | Tel: Email: | ruling.lrl@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

In this contribution, it is proposed to disable sub-block transform when triangle mode is used. It is reported that, under the CTC, the overall impact for {Y, U, V} is {0.00%, -0.01%, -0.03%} for RA and {0.00%, 0.33%, -0.17%} for LDB.

Introduction

In VVC draft 4, sub-block transform (SBT) can be applied to a block coded with triangle mode. However, as illustrated in Figure 1, such combination creates two artificially intersecting boundaries within the CU. It is therefore proposed to disable SBT for triangle block.

Figure 1. An illustration of combining SBT and triangle mode

Simulation results

The proposed modification is implemented on top of VTM-4.0. Simulation results for the RA and LDB configurations are provided in Table 1. The overall impact for {RA, LDB} is {0.00%, 0.00%}. The maximum coding gain for {RA, LDB} is {0.09% and 0.32%} and the maximum coding loss for {RA, LDB} is {0.05%, 0.08%}. This could be due to the fact that such combination is seldom selected by the encoder during rate-distortion optimization even when allowed.

Table 1. Simulation results of proposed method

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.05% | 0.06% | 100% | 100% |
| Class A2 | -0.01% | -0.05% | 0.00% | 100% | 100% |
| Class B | 0.01% | -0.07% | -0.03% | 100% | 98% |
| Class C | 0.01% | 0.06% | -0.11% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | -0.03% | 100% | 99% |
| Class D | -0.02% | 0.13% | 0.16% | 100% | 101% |
| Class F | -0.02% | 0.01% | 0.01% | 100% | 99% |

|  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.23% | -0.11% | 100% | 101% |
| Class C | 0.01% | 0.44% | 0.06% | 100% | 99% |
| Class E | -0.04% | 0.35% | -0.58% | 100% | 98% |
| Overall | 0.00% | 0.33% | -0.17% | 100% | 99% |
| Class D | 0.01% | -0.23% | -0.39% | 100% | 97% |
| Class F | -0.13% | -0.35% | -0.09% | 100% | 99% |

Proposed text

The proposed syntax change is highlighted in yellow.

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && merge_flag[ x0 ][ y0 ] = = 0 ) |  |
| cu_cbf | ae(v) |
| if( cu_cbf ) { |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTER && sps_sbt_enabled_flag && !ciip_flag[ x0 ][ y0 ] && !merge_triangle_flag[ x0 ][ y0 ] ) { |  |
| if( cbWidth <= MaxSbtSize && cbHeight <= MaxSbtSize ) { |  |
| allowSbtVerH = cbWidth >= 8 |  |
| allowSbtVerQ = cbWidth >= 16 |  |
| allowSbtHorH = cbHeight >= 8 |  |
| allowSbtHorQ = cbHeight >= 16 |  |
| if( allowSbtVerH ¦ ¦ allowSbtHorH ¦ ¦ allowSbtVerQ ¦ ¦ allowSbtHorQ ) |  |
| cu_sbt_flag | ae(v) |
| } |  |
| … |  |
| } |  |

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).