| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0485 |
| --- | --- |

| Title: | Non-CE6: Efficient separable Multi-Transform-Signalling (MTS) without zero-out |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Amir Said, Hilmi E. Egilmez, Yung-Hsuan Chao, Vadim Seregin, Marta Karczewicz 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Email: | asaid@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

The “zeroing-out” of transform coefficients is a straightforward technique to reduce the computational complexity of large-dimension transforms. However, its apparent simplicity can hide the fact that, when it is used in different transforms and dimensions, it creates two logical types of transform blocks, resulting in very significant additional complications in the codec’s design and implementation code. To avoid those problems, proposal presents a new implementation of the 64-point DCT-2, together with Transform Adjustments for DCT-2-based computation of DCT-8 DST-7. Its complexity analysis and simulation results show that the propose method can reduce the complexity of the large separable transforms used by Multi-Transform-Signalling (MTS), to values very similar to the “zero-out,” without incurring in its coding loss.

# Introduction

For a transform of dimension N, “zeroing-out” consists in computing only a certain fraction N/K of transform coefficients (those corresponding to most important basis vectors, or lowest frequencies), and setting the remaining coefficients to zero. While this is theoretically sub-optimal, it is a very convenient and easy way to reduce the computational complexity by a factor of K, and has been shown to produce relatively small losses for large N.

However, to really evaluate its effects it is necessary to also consider factors like

- It causes losses that are not identified in low-resolution video sequences, but become important for sequences of high resolution.

- It forces the codec implementation to support two different logical types of transform blocks:

  - Residual and recovered residual blocks, which correspond to the input and output of the forward and inverse transforms, respectively;

  - Transform coefficient blocks (of possibly different shapes), which contain the sets of coefficients not required to be zero.

The first factor can be identified by simply analyzing the coding gains on videos of different resolutions, but the second is equally important but subtler, because it involves codec design and code complexity.

The existence of two types of logical transform blocks affect functions related to (a) transforms; (b) quantization; and (c) coefficient entropy coding. For that reason, the zero-out rules result in dozens of code “patches” on all those functions, in order to deal with all the cases where the two types of blocks have different dimensions and shapes.

Even in a codec that has been designed with an architecture to support such duality of transform blocks, those complications make the codec more expensive to implement, because it may be difficult to deal with the combinatorial number of possible cases and exceptions. Of course, those problems are much worse if the codec has not been originally designed to support different transform block types.

This proposal shows that there are alternatives to the zero-out technique that are, from a conceptual and computational perspective, only slightly more complex, but eliminate its problems

The proposal combines two techniques

- For the 64-point DCT-2 only, we propose a DCT-4 factorization that uses the same DCT-2 “partial-butterfly” that is part of the VTM code, plus one additional low-complexity computation, to halve again (besides the DCT-2 one level partial butterfly) matrix dimensions and number of multiplications. This is an alternative computation of the DCT-2, and not an an approximation.

- For supporting the DCT-8 and DST-7, we use the Transform Adjustment method of JVET-M0538, which approximates those transforms using the DCT-2 as main computation engine. Since those transforms use the DCT-2, they are independent of how it is computed, so it can be readily used together with the DCT-4 factorization.

# Summary of features

Since this proposal uses the same Transform Adjustment method as in JVET-M0538, we repeat here some of the features listed in that document.

- With the adoption of 8-bit coefficients for primary transforms in VTM-3, the use of the DCT-2 in this proposal now corresponds to current HEVC implementations, and previously optimized transform SW and HW can be readily re-used.

- No additional HW is needed for supporting the forward DCT at decoder-only implementations.

- Arithmetic precision is maintained:

  - The coefficients of the 8×8 adjustment matrix are represented with 8 bits.

  - After the adjustment stages the computed values are re-normalized and “clamped” to guarantee that inputs to the inverse DCT-2 are limited to 16-bit integers.

- The band-matrix previously used for adjustment computations is replaced by a single small matrix, which was indicated as the preferred form of computation.

- All computations of the DCT-2 and the adjustments have very regular patterns, greatly simplifying efficient parallel implementation using SIMD instructions or custom HW (a SIMD-based code is shared as example).

- The additional memory (beyond that of used for the DCT-2) needed for computing the MTS transforms of dimensions 16, 32, and 64, is only 64 bytes. This represents a very significant reduction compared to the memory required to store the DST-7/DCT-8 matrices (cf. Section 4).

- Due to the “zero-out” technique used in DCT-2 of dimension 64, the worst-case transform complexity in VTM-3, which defines the custom HW costs, boils down to the matrix-based DST-7 and DCT-8 on blocks of size 32×32. In Section 6 it is shown how the worst-case number of multiplications per transform coefficient is reduced from 64 to 36 (i.e., approximately a 40% reduction).

- A comparison of simulation results in CE6-1.4c and CE6-1.4d show that MTS transforms of dimension 64 now have a reasonable computational complexity, which is practically equal to the DCT-2 transform of dimension 64 (when using “zero-out”). So, additional gains from extending MTS to larger blocks do not increase encoding or decoding times.

- Transform coefficient throughput measurements, based on the throughput profiler proposed in JVET-M0540, show significant improvements, especially at decoder inverse transformations (leading from 2 to 5 times decoder speedups).

- A separate proposal [JVET-M0539] (exploiting special properties of the DCT-2 and the corresponding adjustments for DST-7 and DCT-8,) describes how it is possible to significantly increase the throughput at the encoder side, because all four MTS combinations (DST-7 or DCT-8 for horizontal or vertical passes) can be computed with complexity similar to that of computing a single separable transformation.

# Proposed DCT-2 and DCT-4 factorization

Using TN(C2) and TN(C4) to represent the matrices of the DCT-2 and DCT-4 transforms of dimension N, the “partial-butterfly” implementation of the DCT-2 in VTM is based on the factorization

TN(C2)=PTN/2(C2)00TN/4(C4) H,                                                           (1)  ,

where P is an even-odd permutation matrix and the matrix H, with Hadamard parallel rotations, is defined by

Hi,j=1,i=j<N/2,1,i=N-1-j<N/2,1,i=j+N/2,-1,i=3N/2-1-j,0,otherwise.                                                      (2)

The DCT-2 can be recursively factorized using equation (1), for a complete or incomplete “partial-butterfly” factorization, with the advantage that, because of the factors in H, all versions using integer arithmetic produce the same results.

The DCT-4 can also be recursively factorized, for example using

TN(C4)=GTN/2(C2)00TN/4(C2) H  ,

where G is compose of parallel Givens rotations, and H has parallel Hadamard rotations, but not as regular as in H.

In this proposal the following alternative (and less known) factorization of the DCT-4

TN(C4)=GTN/2(C4)00TN/4(C4) H  ,

is used only for the 64-point DCT-2, to enable reducing its complexity.

# Transform adjustments

The type of Transform Adjustments used in this proposal were proposed in JVET-M0538 and  as a method for replacing computationally the costly trigonometric transforms DST-7, and DCT-8, with approximations that could

- Provide the codec with the same type of transform diversity as that provided by the trigonometric transforms, and thus yield practically the same coding gains;

- Be computed using only an “adjustment” of very low complexity, plus the DCT-2 as the “main computational engine,” since it is the transform already used in HEVC, has been widely studied and optimized, and enables re-using previously developed hardware and software implementations.

A diagram with the new method is shown in Fig. 1, where the adjustments are computed after the forward DCT-2, and their inverse before the inverse DCT-2. In this new version those adjustments correspond to simply multiplying the vector with the 8 lowest-frequency transform coefficients by an 8×8 matrix. The same matrix is used for transforms of dimension 16, 32, and 64, and for the DST-7 and DCT-8 adjustments.

Fig. 1 – Implementation of transform adjustment that needs only the inverse DCT-2 at the decoder. For transforms of dimension 32 and 64, the adjustment blocks correspond to multiplying the 8 lowest frequency transform coefficients by the same 8×8 matrix.

# Implementation

For a transform of dimension N = 16, 32, or 64, if c(DCT-2) is the N-dimensional vector with coefficients of the DCT-2 transform (in increasing frequency), we can split this vector as

c(DCT-2) = [  x | y ],

where x has dimension 8 (i.e., the coefficients of the 8 lowest frequencies) and y has dimension N – 8. The transform adjustments corresponding to the DCT-8 and DST-7 MTS options are defined as

ĉ(DCT-8) = [  A(DCT-8) x | y ],

ĉ (DST-7) = [  A(DST-7) x | y ].

where A is an 8×8 matrix (symbol ĉ is used instead of c to indicate that those are not the true DCT-8 and DST-7 coefficients).

Matrix A should be orthogonal and designed to maximize coding gain. In this proposal the matrix used to compute the orthogonal transform associated with the DST-7 is scaled by 128, and defined by the C++ code

const TMatrixCoeff g_TAF_DST7[8][8] =

{

{ 118,-48,-12, -2, -2, -1, -1,  0 },

{  38,108,-54,-15, -8, -6, -3, -2 },

{  24, 36,107,-51,-16, -9, -4, -4 },

{  14, 23, 34,111,-44,-13, -8, -5 },

{  10, 16, 19, 29,114,-38,-14, -8 },

{   8, 13, 13, 14, 26,118,-33,-12 },

{   6,  9,  9, 11, 15, 24,120,-28 },

{   5,  8,  8,  8, 11, 14, 23,124 }

};

while the matrix corresponding to the DCT-8 computation is

const TMatrixCoeff g_TAF_DCT8[8][8] =

{

{ 118, 48,-12,  2, -2,  1, -1,  0 },

{ -38,108, 54,-15,  8, -6,  3, -2 },

{  24,-36,107, 51,-16,  9, -4,  4 },

{ -14, 23,-34,111, 44,-13,  8, -5 },

{  10,-16, 19,-29,114, 38,-14,  8 },

{  -8, 13,-13, 14,-26,118, 33,-12 },

{   6, -9,  9,-11, 15,-24,120, 28 },

{  -5,  8, -8,  8,-11, 14,-23,124 }

};

It can be observed that the elements of the two matrices have same magnitudes, but different signs, according to a chessboard pattern. This is not a coincidence and is a consequence of properties of the DCT-2, and DCT-8 and DST-7. Thus, only one of these matrices need to be stored, and used in the matrix-vector computations (as shown in the code for this CE). For various optimized HW or SW implementations it may be more useful to store both versions, yet it can be represented using a single 8x8 matrix requiring only 64 additional bytes of memory. Since the matrices above are scaled versions of orthogonal matrices, their inverses are defined by simply transposing (or left-side multiplication).

Note that in this proposal the same matrix coefficients are used for transforms of dimensions 16, 32, and 64, and thus the total memory requirement, beyond the DCT-2, is only 64 bytes.

# Simulation results

## CTC (inter MTS disabled)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.59% | -0.12% | -0.21% | 108% | 103% |
| Class A2 |  |  |  |  |  |
| Class B | -0.14% | 0.12% | 0.13% | 103% | 98% |
| Class C | -0.06% | -0.08% | -0.10% | 113% | 95% |
| Class E | -0.28% | 0.03% | -0.10% | 112% | 105% |
| Overall |  |  |  |  |  |
| Class D | -0.07% | 0.01% | -0.09% | 113% | 105% |
| Class F | 0.03% | 0.05% | -0.12% | 107% | 96% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.17% | 0.00% | -0.33% | 107% | 99% |
| Class C | -0.05% | -0.13% | -0.29% | 109% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.08% | -0.16% | 0.04% | 111% | 103% |
| Class F | -0.05% | -0.05% | -0.10% | 90% | 102% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.20% | 0.32% | -0.29% | 105% | 105% |
| Class E | 0.05% | 0.39% | 0.10% | 102% | 111% |
| Overall |  |  |  |  |  |
| Class D | -0.09% | -0.10% | -0.31% | 111% | 104% |
| Class F |  |  |  |  |  |

## CTC + inter MTS enabled

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 with InterMTS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.20% | 0.17% | -0.06% | 102% | 101% |
| Class C | -0.05% | -0.03% | -0.13% | 104% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.15% | 0.04% | -0.04% | 104% | 98% |
| Class F | -0.03% | 0.00% | -0.10% | 97% | 101% |

## Low QP test

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over Low QP VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | -0.12% | -0.18% | 100% | 96% |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.00% | 0.02% | 101% | 98% |
| Class C | 0.02% | 0.06% | 0.10% | 97% | 97% |
| Class E | -0.09% | 0.14% | -0.06% | 99% | 106% |
| Overall |  |  |  |  |  |
| Class D | -0.02% | 0.11% | 0.09% | 101% | 100% |
| Class F | 0.02% | -0.03% | 0.06% | 100% | 99% |

# References

- A. Said et al., “CE6-1.4:  Efficient Implementations of MTS with Transform Adjustments,” JVET-M0538, 13th meeting, Marrakech, MA, Jan. 2019.

- F. Bossen, J.Boyce, X. Li, V. Seregin and K. Suehring,  “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, 13th meeting, Marrakech, MA, Jan. 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

For consistency with signal processing notation, the matrix indexes are in the range 0, 1, …, N–1.