| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0486 |
| --- | --- |

| Title: | Non-CE4: Simplification for harmonization of HMVP and share merge list |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Suhong Wang1, Xiaozhen Zheng2, Shanshe Wang1, Siwei Ma1 Peking University1, DJI2 | Email: | suhong.wang@pku.edu.cn xiaozhen.zheng@dji.com |
| Source: | SZ DJI Technology Co., Ltd., Peking University |  |  |

_____________________________

- Abstract

At the 13th JVET meeting, share merge list is adopted to current VVC design, which proposes to share the same merge candidate list for all leaf coding units(CUs) of one ancestor node. When harmonized with HMVP, there exists some redundant operations when current CU is a ‘sharing merge’ state. In this contribution, simplification for harmonization of HMVP and share merge list is proposed. Simulation results show no performance change on average for both RA and LDB configurations.

Introduction

With shared merge list, it can enable parallel processing of small skip/merge-coded blocks. The shared merging list is generated at the merge sharing node pretending the merge sharing node is a leaf CU. In current design, when adding HMVP candidate into the merge list of current CU, firstly, the sharing merge state of current CU will be checked, then different look up tables (LUTs) will be chosen to derive HMVP candidate. The steps of the above process is illustrated in Fig.1.

Fig.1 Process of merge candidate list construction

During this process, if current CU is sharing state, a backup LUT will be introduced and all leaf CUs of a shared ancestor node will use the same backup HMVP LUT to derive HMVP candidate, which means there exists two HMVP LUTs, one is only for deriving HMVP candidate but not update, the other is only for updating but not use. This design is complicated and can be improved.

Proposed Method

In order to simplify the current design of harmonization for HMVP and share merge list, the process when adding HMVP merge candidate for sharing state CU is changed like follows:

- Use only one LUT for all CUs, whether it is sharing state or not.

- To enable parallel processing for these small sharing merge CUs, the same HMVP LUT is set for all leaf CUs of a shared ancestor node by only updating the motion information of the last coded leaf CU of the ancestor node to the HMVP LUT. In that way, coding gain of HMVP can be still maintained.

Fig.2 Process of merge candidate list construction with proposed method.

Figure 2 shows the merge candidate list construction process with proposed method. When updating the HMVP LUT, if current CU is a sharing state but not the last leaf CU (the right bottom one), the motion information of these kind of CU will not be added into HMVP LUT. For example, in Fig.3, the motion information of A and B will not be added into HMVP LUT, while C will.

Fig.3 Example of a shared merge CU sets.

Experimental Results

The experiments were performed under JVET common test conditions.

BD-Rate results of the tests on top of the VTM-4.0 compared to the anchor with RA and LDB configurations are shown in following tables. The full results are provided in the attached excel sheets. Due to the unstable performance of the computer resource, the encoding time is not reliable.

Table 1. Test results of the proposed method

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.01% | -0.03% | 107% | 98% |
| Class A2 | -0.01% | -0.06% | 0.03% | 106% | 99% |
| Class B | 0.01% | 0.01% | 0.06% | 81% | 98% |
| Class C | 0.01% | -0.08% | -0.12% | 82% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.03% | -0.01% | 91% | 98% |
| Class D | -0.02% | -0.01% | 0.06% | 82% | 96% |
| Class F | 0.02% | -0.09% | -0.16% | 100% | 97% |
|  |  |  | Low Delay B |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.09% | 0.00% | 80% | 97% |
| Class C | -0.01% | 0.26% | 0.02% | 82% | 99% |
| Class E | 0.03% | -0.01% | -0.70% | 81% | 98% |
| Overall | 0.00% | 0.12% | -0.17% | 81% | 98% |
| Class D | 0.00% | 0.02% | -0.64% | 84% | 97% |
| Class F | -0.15% | -0.28% | -0.46% | 58% | 99% |

Conclusion

This proposal suggests simplify the harmonization of HMVP and share merge list by using only one HMVP LUT for all CUs. Besides, to maintain the coding gain, motion information of the last coded leaf CU of a sharing ancestor node will be updated into the HMVP LUT. Experimental results shows that there’s no performance change for both RA and LDB configurations on average and minor decoding time decrease can be observed.

Reference

[1] C.-C. Chen, Y.-C. Lin, M.-S. Chiang, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei , JVET-M0170, CE4.3.1: Shared merging candidate list, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019

Patent rights declaration(s)

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SZ DJI Technology Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).