| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0486_v3 |
| --- | --- |

| Title: | CE4-related: Simplification for share merge list and HMVP harmonization process |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Suhong Wang1, Xiaozhen Zheng2, Shanshe Wang1, Siwei Ma1 Peking University1, DJI2 | Email: | suhong.wang@pku.edu.cn xiaozhen.zheng@dji.com |
| Source: | SZ DJI Technology Co., Ltd., Peking University |  |  |

_____________________________

- Abstract

At the 13th JVET meeting, share merge list is adopted to current VVC design, which proposes to share the same merge candidate list for all leaf coding units(CUs) of one ancestor node. When harmonized with HMVP, there exists some redundant operations when current CU is a ‘sharing merge’ state. In this contribution, simplification for harmonization of HMVP and share merge list is proposed. Simulation results show no performance change on average for both RA and LDB configurations.

Introduction

With shared merge list, it can enable parallel processing of small skip/merge-coded blocks. The shared merging list is generated at the merge sharing node pretending the merge sharing node is a leaf CU. In current design, when adding HMVP candidate into the merge list of current CU, firstly, the sharing merge state of current CU will be checked, then different look up tables (LUTs) will be chosen to derive HMVP candidate. The steps of the above process is illustrated in Fig.1.

Fig.1 Process of merge candidate list construction

During this process, if current CU is sharing state, a backup LUT will be introduced and all leaf CUs of a shared ancestor node will use the same backup HMVP LUT to derive HMVP candidate, which means there exists two HMVP LUTs, one is only for deriving HMVP candidate but not update, the other is only for updating but not use. This design is complicated and can be improved.

Proposed Method

In order to simplify the current design of harmonization for HMVP and share merge list, the process when adding HMVP merge candidate for sharing state CU is changed like follows:

- Use only one LUT for all CUs, whether it is sharing state or not.

- To enable parallel processing for these small sharing merge CUs, the same HMVP LUT is set for all leaf CUs of a shared ancestor node by only updating the motion information of the last coded leaf CU of the ancestor node to the HMVP LUT. In that way, coding gain of HMVP can be still maintained.

Fig.2 Process of merge candidate list construction with proposed method.

Figure 2 shows the merge candidate list construction process with proposed method. When updating the HMVP LUT, if current CU is a sharing state but not the last leaf CU (the right bottom one), the motion information of these kind of CU will not be added into HMVP LUT. For example, in Fig.3, the motion information of A and B will not be added into HMVP LUT, while C will.

Fig.3 Example of a shared merge CU sets.

To further simply HMVP LUT update design for shared merge list, this contribution also propose not allow to update the motion of the CUs in shared merge node to HMVP LUT.

Experimental Results

The experiments were performed under JVET common test conditions.

BD-Rate results of the tests on top of the VTM-4.0 compared to the anchor with RA and LDB configurations are shown in following tables. The full results are provided in the attached excel sheets. Due to the unstable performance of the computer resource, the encoding time is not reliable. The performance of using the last coded leaf CU of the ancestor node to HMVP LUT is shown at Table 1, and the performance that doesn’t update HMVP LUT in shared merge list node is shown at Table 2.

Table 1. Test results of the proposed method1

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.01% | -0.03% | 107% | 98% |
| Class A2 | -0.01% | -0.06% | 0.03% | 106% | 99% |
| Class B | 0.01% | 0.01% | 0.06% | 81% | 98% |
| Class C | 0.01% | -0.08% | -0.12% | 82% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.03% | -0.01% | 91% | 98% |
| Class D | -0.02% | -0.01% | 0.06% | 82% | 96% |
| Class F | 0.02% | -0.09% | -0.16% | 100% | 97% |
|  |  |  | Low Delay B |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.09% | 0.00% | 80% | 97% |
| Class C | -0.01% | 0.26% | 0.02% | 82% | 99% |
| Class E | 0.03% | -0.01% | -0.70% | 81% | 98% |
| Overall | 0.00% | 0.12% | -0.17% | 81% | 98% |
| Class D | 0.00% | 0.02% | -0.64% | 84% | 97% |
| Class F | -0.15% | -0.28% | -0.46% | 58% | 99% |

Table 2. Test results of the proposed method2

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.06% | 0.03% | 101% | 99% |
| Class A2 | -0.02% | -0.09% | 0.05% | 101% | 99% |
| Class B | 0.00% | -0.02% | -0.02% | 81% | 98% |
| Class C | -0.01% | -0.01% | -0.05% | 82% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | -0.01% | 89% | 98% |
| Class D | -0.03% | 0.02% | -0.04% | 81% | 96% |
| Class F | 0.01% | -0.08% | -0.14% | 83% | 97% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.00% | 0.09% | -0.11% | 81% | 98% |
| Class E | -0.06% | -0.16% | -0.25% | 82% | 96% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.11% | -0.03% | -0.35% | 82% | 94% |
| Class F | -0.28% | -0.01% | -0.41% | 57% | 96% |

Changes of Text

Method 1:

7.4.7.5

When all the following conditions are true, the history-based motion vector predictor list for the shared merging candidate list region is updated by setting NumHmvpSmrCand equal to the NumHmvpCand, and setting HmvpSmrCandList[ i ] equal to HmvpCandList[ i ] for i = 0..NumHmvpCand − 1:

- IsInSmr[ x0 ][ y0 ] is equal to TRUE

- SmrX[ x0 ][ y0 ]  is equal to x0

- SmrY[ x0 ][ y0 ]  is equal to y0

8.5.2.1 General

The updating process for the history-based motion vector predictor list as specified in clause 8.4.2.15 is invoked with luma motion vectors mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ], reference indices refIdxL0 and refIdxL1, and prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ], and IsInSmr[ xCb ][ yCb ], cbWidth, cbHeight, SmrW[ xCb ][ yCb ] , SmrH[ xCb ][ yCb ], xCb, yCb, SmrX[ xCb ][ yCb ] and SmrY[ xCb ][ yCb ].

8.5.2.2. Derivation process for luma motion vectors for merge mode

The variables xSmr, ySmr, smrWidth, smrHeight, and smrNumHmvpCand are derived as follows:

xSmr = IsInSmr[ xCb ][ yCb ]  ?  SmrX[ xCb ][ yCb ]  :  xCb	(8-294)

ySmr = IsInSmr[ xCb ][ yCb ]  ?  SmrY[ xCb ][ yCb ]  :  yCb	(8-295)

smrWidth = IsInSmr[ xCb ][ yCb ]  ?  SmrW[ xCb ][ yCb ]  :  cbWidth	(8-296)

smrHeight = IsInSmr[ xCb ][ yCb ]  ?  SmrH[ xCb ][ yCb ]  :  cbHeight	(8-297)

smrNumHmvpCand = IsInSmr[ xCb ][ yCb ]  ?  NumHmvpSmrCand  :  NumHmvpCand	(8-298)

- When numCurrMergeCand is less than (MaxNumMergeCand − 1) and smrNumHmvpCandNumHmvpCand is greater than 0, the following applies:

8.5.2.6 Derivation process for history-based merging candidates

Inputs to this process are:

- a merge candidate list mergeCandList,

- a variable isInSmr specifying whether the current coding unit is inside a shared mergign candidate region,

- the number of available merging candidates in the list numCurrMergeCand.

Outputs to this process are:

- the modified merging candidate list mergeCandList,

- the modified number of merging candidates in the list numCurrMergeCand.

The variables isPrunedA1 and isPrunedB1 are both set equal to FALSE.

The array smrHmvpCandList and the variable smrNumHmvpCand are derived as follows:

smrHmvpCandList = isInSmr  ?  HmvpSmrCandList  :  HmvpCandList	(8-376)

smrNumHmvpCand = isInSmr  ?  NumHmvpSmrCand  :  NumHmvpCand	(8-377)

For each candidate in smrHmvpCandListHmvpCandList [ hMvpIdx ] with index hMvpIdx = 1..smrNumHmvpCandNumHmvpCand, the following ordered steps are repeated until numCurrMergeCand is equal to ( MaxNumMergeCand − 1):

- The variable sameMotion is derived as follows:

  - If all of the following conditions are true for any i = 0..numOrigMergeCand − 1, sameMotion and isPruned[ i ] are both set equal to TRUE:

  - The candidate smrHmvpCandListHmvpCandList[ smrNumHmvpCand − hMvpIdx] is equal to the merging candidate mergeCandList[ i ].

  - isPruned[ i ] is equal to FALSE.

  - Otherwise, sameMotion is set equal to FALSE.

- When sameMotion is equal to FALSE, the candidate smrHmvpCandList[ smrNumHmvpCand − hMvpIdx] is added to the merging candidate list as follows:

mergeCandList[ numCurrMergeCand++ ] = smrHmvpCandList[ smrNumHmvpCand − hMvpIdx ] 	(8-378)

8.4.2.16 Updating process for the history-based motion vector predictor candidate list

Inputs to this process are:

- luma motion vectors in 1/16 fractional-sample accuracy mvL0 and mvL1,

- reference indices refIdxL0 and refIdxL1,

- prediction list utilization flags predFlagL0 and predFlagL1.

- IsInSmr

- Width and height of current coding unit cbWidth, cbHeight;

- Width and height of current shared merge region SmrW, SmrH;

- The left-top coordinator of current coding unit (xCb, yCb);

- The left-top coordinator of current shared merge region (SmrX, SmrY);

If IsInSmr is false or IsInSmr is true and (xCb + cbWidth) is equal to SmrW and (yCb + cbHeight) is equal to SmrH, the candidate list HmvpCandList is modified using the candidate hMvpCand by the following ordered steps:

- The variable identicalCandExist is set equal to FALSE and the variable removeIdx is set equal to 0.

- When NumHmvpCand is greater than 0, for each index hMvpIdx with hMvpIdx = 0..NumHmvpCand − 1, the following steps apply until identicalCandExist is equal to TRUE:

  - When hMvpCand is equal to HmvpCandList[ hMvpIdx ], identicalCandExist is set equal to TRUE and removeIdx is set equal to hMvpIdx.

- The candidate list HmvpCandList is updated as follows:

  - If identicalCandExist is equal to TRUE or NumHmvpCand is equal to 6, the following applies:

  - For each index i with i = ( removeIdx + 1 )..( NumHmvpCand − 1 ), HmvpCandList[ i − 1] is set equal to HmvpCandList[ i ].

  - HmvpCandList[ NumHmvpCand − 1 ] is set equal to mvCand.

  - Otherwise (identicalCandExist is equal to FALSE and NumHmvpCand is less than 6), the following applies:

  - HmvpCandList[ NumHmvpCand++ ] is set equal to mvCand.

Method 2:

The modifications for 7.4.5.6, 8.4.2.2, 8.4.2.6 is same as method 1. The modifications for 8.4.2.1 and 8.4.2.15 are shown below:

8.5.2.1 General

The updating process for the history-based motion vector predictor list as specified in clause 8.4.2.15 is invoked with luma motion vectors mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ], reference indices refIdxL0 and refIdxL1, and prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ], and IsInSmr[ xCb ][ yCb ].

8.4.2.16 Updating process for the history-based motion vector predictor candidate list

Inputs to this process are:

- luma motion vectors in 1/16 fractional-sample accuracy mvL0 and mvL1,

- reference indices refIdxL0 and refIdxL1,

- prediction list utilization flags predFlagL0 and predFlagL1.

- IsInSmr

If IsInSmr is false, the candidate list HmvpCandList is modified using the candidate hMvpCand by the following ordered steps:

- The variable identicalCandExist is set equal to FALSE and the variable removeIdx is set equal to 0.

- When NumHmvpCand is greater than 0, for each index hMvpIdx with hMvpIdx = 0..NumHmvpCand − 1, the following steps apply until identicalCandExist is equal to TRUE:

  - When hMvpCand is equal to HmvpCandList[ hMvpIdx ], identicalCandExist is set equal to TRUE and removeIdx is set equal to hMvpIdx.

- The candidate list HmvpCandList is updated as follows:

  - If identicalCandExist is equal to TRUE or NumHmvpCand is equal to 6, the following applies:

  - For each index i with i = ( removeIdx + 1 )..( NumHmvpCand − 1 ), HmvpCandList[ i − 1] is set equal to HmvpCandList[ i ].

  - HmvpCandList[ NumHmvpCand − 1 ] is set equal to mvCand.

  - Otherwise (identicalCandExist is equal to FALSE and NumHmvpCand is less than 6), the following applies:

  - HmvpCandList[ NumHmvpCand++ ] is set equal to mvCand.

Conclusion

This proposal suggests simplify the harmonization of HMVP and share merge list by using only one HMVP LUT for all CUs. Besides, to maintain the coding gain, motion information of the last coded leaf CU of a sharing ancestor node will be updated into the HMVP LUT. Experimental results shows that there’s no performance change for both RA and LDB configurations on average and minor decoding time decrease can be observed.

Reference

[1] C.-C. Chen, Y.-C. Lin, M.-S. Chiang, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei , JVET-M0170, CE4.3.1: Shared merging candidate list, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019

Patent rights declaration(s)

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SZ DJI Technology Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).