| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0488-v2 |
| --- | --- |

| Title: | CE5-related: Syntax redundancy removal in adaptive loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xuewei Meng, Xiaozhen Zheng, Chuanmin Jia, Shanshe Wang, Siwei Ma | Tel: Email: | xwmeng@pku.edu.cn xiaozhen.zheng@dji.com |
| Source: | Peking University, SZ DJI Technology Co., Ltd., |  |  |

_____________________________

- Abstract

This contribution proposes to remove syntax redundancy for adaptive loop filter coefficient signalling in VTM-4.0. The proposed method removes the alf_luma_coeff_delta_idx when the syntax alf_luma_num_filters_signalled_minus1 equals to NumAlfFilters-1. It is reported that as compared to VTM-4.0, xxx%. And in worst case, it can achieve xxx%, xxx% and xxx% coding gain in all intra, random access and low delay B configuration respectively.

Introduction

In current ALF design, alf_chroma_idc and alf_luma_num_filters_signalled_minus1, alf_luma_coeff_delta_idx are existed in current tile group header.

alf_luma_num_filters_signalled_minus1 plus 1 specifies the number of adpative loop filter classes for which luma coefficients can be signalled. The value of alf_luma_num_filters_signalled_minus1 shall be in the range of 0 to NumAlfFilters − 1, inclusive. The value of NumAlfFilters is set equal to 25 in VVC-4.0.

alf_luma_coeff_delta_idx[ filtIdx ] specifies the indices of the signalled adaptive loop filter luma coefficient deltas for the filter class indicated by filtIdx ranging from 0 to NumAlfFilters − 1. When alf_luma_coeff_delta_idx[ filtIdx ] is not present, it is inferred to be equal to 0.

However, when alf_luma_num_filters_signalled_minus1 is equal to NumAlfFilters – 1. The max number of pixel class is equal to the number of filter class. In this case, it is unnecessary to write alf_luma_coeff_delta_idx in the bitstream.

Proposed method

Encoder and Decoder optimization

This contribution proposed to remove the alf_luma_coeff_delta_idx signalling when the syntax alf_luma_num_filters_signalled_minus1 equals to NumAlfFilters-1 in encoder side. As for decoder side, if the syntax alf_luma_num_filters_signalled_minus1 equals to NumAlfFilters-1, the value of alf_luma_coeff_delta_idx[ filtIdx ] is set to  filtIdx and it is unnecessary to parse alf_luma_coeff_delta_idx.

Proposed syntax changes

7.3.3.2 Adaptive loop filter data syntax

| alf_data( ) { | Descriptor |
| --- | --- |
| alf_chroma_idc | tu(v) |
| alf_luma_num_filters_signalled_minus1 | tb(v) |
| if( alf_luma_num_filters_signalled_minus1 > 0 && alf_luma_num_filters_signalled_minus1 < NumAlfFilters - 1 ) { |  |
| for( filtIdx = 0; filtIdx < NumAlfFilters; filtIdx++ ) |  |
| alf_luma_coeff_delta_idx[ filtIdx ] | tb(v) |
| } |  |
| alf_luma_coeff_delta_flag | u(1) |
| if ( !alf_luma_coeff_delta_flag && alf_luma_num_filters_signalled_minus1 > 0 ) |  |
| alf_luma_coeff_delta_prediction_flag | u(1) |
| alf_luma_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 3; i++ ) |  |
| alf_luma_eg_order_increase_flag[ i ] | u(1) |
| if ( alf_luma_coeff_delta_flag ) { |  |
| for( sigFiltIdx = 0; sigFiltIdx <= alf_luma_num_filters_signalled_minus1; sigFiltIdx++ ) |  |
| alf_luma_coeff_flag[ sigFiltIdx ] | u(1) |
| } |  |
| for( sigFiltIdx = 0; sigFiltIdx <= alf_luma_num_filters_signalled_minus1; sigFiltIdx++ ) { |  |
| if ( alf_luma_coeff_flag[ sigFiltIdx ] ) { |  |
| for ( j = 0; j < 12; j++ ) { |  |
| alf_luma_coeff_delta_abs[ sigFiltIdx ][ j ] | uek(v) |
| if( alf_luma_coeff_delta_abs[ sigFiltIdx ][ j ] ) |  |
| alf_luma_coeff_delta_sign[ sigFiltIdx ][ j ] | u(1) |
| } |  |
| } |  |
| } |  |
| if ( alf_chroma_idc > 0 ) { |  |
| alf_chroma_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 2; i++ ) |  |
| alf_chroma_eg_order_increase_flag[ i ] | u(1) |
| for( j = 0; j < 6; j++ ) { |  |
| alf_chroma_coeff_abs[ j ] | uek(v) |
| if( alf_chroma_coeff_abs[ j ] > 0 ) |  |
| alf_chroma_coeff_sign[ j ] | u(1) |
| } |  |
| } |  |
| } |  |

7.4.4.2 Adaptive loop filter data semantics

alf_chroma_idc equal to 0 specifies that the adaptive loop filter is not applied to Cb and Cr colour components. alf_chroma_idc equal to 1 indicates that the adaptive loop filter is applied to the Cb colour component. alf_chroma_idc equal to 2 indicates that the adaptive loop filter is applied to the Cr colour component. alf_chroma_idc equal to 3 indicates that the adaptive loop filter is applied to Cb and Cr colour components.

The maximum value maxVal of the truncated unary binarization tu(v) is set equal to 3.

The variable NumAlfFilters specifying the number of different adaptive loop filters is set equal to 25.

alf_luma_num_filters_signalled_minus1 plus 1 specifies the number of adpative loop filter classes for which luma coefficients can be signalled. The value of alf_luma_num_filters_signalled_minus1 shall be in the range of 0 to NumAlfFilters − 1, inclusive.

The maximum value maxVal of the truncated binary binarization tb(v) is set equal to NumAlfFilters − 1.

alf_luma_coeff_delta_idx[ filtIdx ] specifies the indices of the signalled adaptive loop filter luma coefficient deltas for the filter class indicated by filtIdx ranging from 0 to NumAlfFilters − 1. When alf_luma_coeff_delta_idx[ filtIdx ] is not present and alf_luma_num_filters_signalled_minus1 is equal to 0, it is inferred to be equal to 0. When alf_luma_num_filters_signalled_minus1 is equal to NumAlfFilters − 1, alf_luma_coeff_delta_idx[ filtIdx ] is inferred to be equal to filtIdx.

Simulation results

The simulations are conducted following the common test conditions [1] on top of reference software VTM-4.0 (Table 1). The encoding time is unbelievable.

Table 1. Performance of proposed method over VTM4.0

|  | All intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 141% | 95% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class B | 0.00% | 0.00% | 0.00% | 97% | 102% |
| Class C | 0.00% | 0.00% | 0.00% | 113% | 112% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 104% |
| Overall | 0.00% | 0.00% | 0.00% | 108% | 103% |
| Class D | 0.00% | 0.00% | 0.00% | 112% | 105% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 109% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 114% |
| Class A2 | 0.00% | 0.00% | 0.00% | 80% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 112% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 109% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 96% | 109% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 108% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 115% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 81% | 111% |
| Class C | 0.00% | 0.00% | 0.00% | 82% | 105% |
| Class E | 0.00% | 0.00% | 0.00% | 82% | 105% |
| Overall | 0.00% | 0.00% | 0.00% | 81% | 108% |
| Class D | 0.00% | 0.00% | 0.00% | 81% | 107% |
| Class F | 0.00% | 0.00% | 0.00% | 58% | 106% |

Since the case that all of 25 filters are selected in encoder happens seldom, the coding results for the proposed method under CTC is almost same as anchor. The proposed method is also tested under the worst case which force encoder-side select 25 filters. The results is shown in Table2. The anchor of Table2 is VTM-4.0 with the worst case mentioned above.

Table 2. Performance of proposed method over VTM4.0

|  | All intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.02% | -0.02% | 101% | 103% |
| Class A2 | -0.01% | -0.01% | -0.01% | 101% | 102% |
| Class B | -0.03% | -0.02% | -0.02% | 99% | 102% |
| Class C | -0.07% | -0.06% | -0.06% | 100% | 103% |
| Class E | -0.08% | -0.07% | -0.08% | 100% | 103% |
| Overall | -0.04% | -0.04% | -0.04% | 100% | 102% |
| Class D | -0.32% | -0.17% | -0.18% | 100% | 105% |
| Class F | -0.08% | -0.05% | -0.05% | 99% | 105% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | -0.11% | -0.12% | 100% | 100% |
| Class A2 | -0.14% | -0.10% | -0.11% | 100% | 100% |
| Class B | -0.33% | -0.26% | -0.26% | 102% | 101% |
| Class C | -0.74% | -0.57% | -0.54% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.38% | -0.28% | -0.27% | 101% | 100% |
| Class D | -2.09% | -1.65% | -1.74% | 100% | 102% |
| Class F | -0.70% | -0.58% | -0.57% | 96% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.28% | -0.22% | -0.26% | 101% | 101% |
| Class C | -0.60% | -0.25% | -0.33% | 97% | 95% |
| Class E | -2.69% | -2.63% | -2.64% | 99% | 96% |
| Overall | -0.99% | -0.83% | -0.88% | 100% | 98% |
| Class D | -1.88% | -1.60% | -0.75% | 101% | 96% |
| Class F | -1.37% | -1.45% | -0.58% | 100% | 95% |

Conclusions

In this proposal, a syntax redundancy removal method of adaptive loop filter is proposed. It can achieve minor bits saving in VTM-4.0 and can achieve 0.04%/0.38%/0.99 coding gain for AI/RA/LDB in worst case that force encoder to select 25 filters.

References

[1] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET-M1010, “JVET common test conditions and software reference configurations for SDR video”, Marrakech, January 2019.

Patent rights declaration(s)

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SZ DJI Technology Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).