| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0490-r2 |
| --- | --- |

| Title: | CE6: Transform Simplification (CE6-2.3a-c) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christopher Hollmann, Davood Saffar, Jacob Ström, Per Wennersten Torshamnsgatan 23 SE-16480 Stockholm | Tel: Email: | +46107190000 {christopher.hollmann, davood.saffar, jacob.strom, per.wennersten} @ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This proposal describes the results of CE6-2.3a-c. It is proposed to reduce the number of allowed transform candidates by removing the combination (DCT-8, DCT-8) from consideration, replacing the size-32 DCT-8 with the existing DCT-2, and changing the CABAC coding for the combinations (DCT-8, DST-7) and (DST-7, DCT-8) to a what is alleged to be a more probable mode coding based on the intra prediction mode or the block shape. The original technology was proposed in JVET-M0366.

The three subtests a, b and c use the same software and differ in the configuration. In test a, the number of evaluated transform candidates is limited to 3, in test b to 4, and in test c to 5. It is reported that test (a) gives -0.03% / x% / x% for AI/RA/LDB, respectively. For test (b), the reported results are -0.02% / x% / x% for AI/RA/LDB. For test (c), the results are reported to be -0.02% / x% / x% for AI/RA/LDB. It is stated that for test (a), run times are not changed, whereas for tests (b) and (c) the encoder run times are reduced by ~10% for AI and ~3% for RA and LDB for the tests with MTS enabled for inter.

A bug was found close to the document deadline, invalidating the previously reported test results.

In version 2 of this document, partial results were updated.

# Technology Description

The foundation of the technology investigated in this CE was described in document JVET-M0366 [1]. The changes evaluated can be summarized as follows:

- Remove the combination (DCT-8, DCT-8) from the codec,

- Replace the size-32 DCT-8 with the existing size-32 DCT-2, and

- Change the encoding of the MTS index.

Due to change 2) the DCT-8 and DCT-2 in combination with DST-7 are marked in this document as DCT-X for the sake of clarity.

With the adoption of proposal M0464 [3], the coding of the MTS index was changed to a truncated unary. In this proposal the coding tree was modified as shown in Figure 1. A step to the left in the figure means coding a ‘0’ whereas a step to the right corresponds to coding a ‘1’.

Transform Skip

DCT-2 hor

DCT-2 ver

DST-7 hor

DST-7 ver

More probable mode

Less probable mode

Figure 1. Changed coding tree

The “more probable mode” and “less probable mode” are the combinations (DCT-X, DST-7) and (DST-7, DCT-X). The decision which one is more probable is made based on the intra prediction mode for intra-predicted blocks and width and height for inter-predicted blocks. The mapping is shown in the following table.

| Prediction | Condition | More probable candidate | Less probable candidate |
| --- | --- | --- | --- |
| Intra MTS | Intra mode > 34 | (DCT-X, DST-7) | (DST-7, DCT-X) |
|  | Intra mode <= 34 | (DST-7, DCT-X) | (DCT-X, DST-7) |
| Inter MTS | Height > Width | (DST-7, DCT-X) | (DCT-X, DST-7) |
|  | Height <= Width | (DCT-X, DST-7) | (DST-7, DCT-X) |

The more probable combination is coded as a ‘0’, whereas the less probable combination is coded as a ‘1’.

Additionally, the context selection is identical to VTM-3.0, which uses different contexts for intra- and inter-predicted blocks for the last two bins indicating whether to use (DST-7, DST-7), (DST-7, DCT-8) or (DCT-8, DST-7).

Note that the software also includes a bug fix regarding bug report #196 [4] which created an encoder decoder mismatch for class F in the Random Access configuration. This bug fix was applied to both anchor and test software.

# Test Results

## Test description

The CE consists of three different tests. All three tests use the same software as described above. There are five different transform modes besides (DCT-2, DCT-2) which the encoder can evaluate: Transform Skip and four combinations of DST-7 and DCT-8. The difference between the tests is the maximum number of modes which is evaluated. In test a, the encoder searches a maximum of three modes, in test b the encoder searches a maximum of four modes, and in test c the encoder can evaluate all five modes.

The results show that the compression efficiency is hardly influenced by the changes. The encoder run time, on the other hand is reduced in tests b and c by around 10% for the AI configuration. The impact on RA and LD is smaller as the number of intra-coded blocks is much smaller. However, when MTS is turned on for inter-coded blocks, the reduction in encoder run time becomes larger. While there is a slightly stronger negative impact on the compression efficiency, the reduction in run time makes up for it.

## Summary

The following table provides an overview of all results. Complete results can be found in section 2.2.

Per-sequence results can be found in the accompanying excel sheets.

We would like to thank Nokia for cross-checking the results.

| Test |  | MTS only for intra (MTS = 1) |  |  |  |  |  |  | MTS for intra and inter (MTS = 3) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate Y |  |  | enc/dec run times |  |  |  | BD-rate Y |  |  | enc/dec run times |  |  |
|  | AI |  | RA | LD |  | AI | RA | LD |  | RA | LD |  | RA | LD |
| CE6-2.3a | -0.03% |  |  |  |  | 100%/ 100% | 100%/ 99% | 100%/ 101% |  |  |  |  | 99%/ 99% | 99%/ 101% |
| CE6-2.3b | -0.02% |  |  |  |  | 90%/ 100% | 99%/ 99% | 100%/ 99% |  |  |  |  | 97%/ 100% | 97%/ 98% |
| CE6-2.3c | -0.02% |  |  |  |  | 90%/ 100% | 99%/ 100% | 99%/ 102% |  |  |  |  | 97%/ 100% | 97%/ 102% |

## Detailed Results for all tests

### CE6-2.3a without Inter MTS

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 + Fix196 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.03% |  | 0.00% |  | 100% |  | 99% |  |
| Class A2 | -0.03% | 0.11% |  | -0.02% |  | 99% |  | 101% |  |
| Class B | -0.04% | 0.03% |  | 0.06% |  | 100% |  | 100% |  |
| Class C | 0.02% | -0.08% |  | 0.01% |  | 101% |  | 99% |  |
| Class E | -0.03% | -0.07% |  | 0.08% |  | 99% |  | 100% |  |
| Overall | -0.03% | -0.01% |  | 0.03% |  | 100% |  | 100% |  |
| Class D | 0.00% | 0.03% |  | -0.18% |  | 100% |  | 100% |  |
| Class F | 0.02% | -0.02% |  | -0.07% |  | 100% |  | 101% |  |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 + Fix196 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  | 100% |  | 99% |  |
| Class A2 |  |  |  |  |  | 101% |  | 99% |  |
| Class B | -0.02% | 0.02% |  | -0.08% |  | 100% |  | 99% |  |
| Class C | 0.00% | -0.02% |  | -0.08% |  | 100% |  | 101% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  | 100% |  | 99% |  |
| Class D | 0.03% | -0.15% |  | -0.30% |  | 100% |  | 100% |  |
| Class F | 0.01% | -0.04% |  | 0.08% |  | 100% |  | 100% |  |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 + Fix196 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  | 100% |  | 102% |  |
| Class C | -0.06% | 0.40% |  | -0.18% |  | 100% |  | 98% |  |
| Class E | 0.08% | -0.32% |  | 1.11% |  | 100% |  | 101% |  |
| Overall |  |  |  |  |  | 100% |  | 101% |  |
| Class D | -0.03% | 0.42% |  | 0.43% |  | 100% |  | 102% |  |
| Class F | -0.11% | 0.46% |  | 0.40% |  | 101% |  | 103% |  |

### CE6-2.3a with Inter MTS

Note that AI results are omitted as they are identical to the ones without Inter MTS.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  | 99% | 100% |
| Class A2 |  |  |  | 99% | 102% |
| Class B |  |  |  | 99% | 97% |
| Class C |  |  |  | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall |  |  |  | 99% | 99% |
| Class D |  |  |  | 99% | 100% |
| Class F |  |  |  | 99% | 103% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  | 99% |  | 101% |  |
| Class C |  |  |  |  |  | 99% |  | 100% |  |
| Class E | 0.04% | 0.62% |  | 2.15% |  | 100% |  | 101% |  |
| Overall |  |  |  |  |  | 99% |  | 101% |  |
| Class D | 0.05% | -0.01% |  | 0.64% |  | 99% |  | 101% |  |
| Class F |  |  |  |  |  | 98% |  | 98% |  |

### CE6-2.3b without Inter MTS

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 + Fix196 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.02% | -0.03% | 90% | 102% |
| Class A2 | 0.00% | -0.05% | -0.05% | 91% | 98% |
| Class B | -0.03% | 0.03% | -0.02% | 90% | 100% |
| Class C | -0.01% | 0.00% | -0.04% | 91% | 101% |
| Class E | -0.03% | -0.05% | 0.10% | 91% | 102% |
| Overall | -0.02% | -0.01% | -0.01% | 90% | 100% |
| Class D | -0.01% | 0.00% | -0.05% | 91% | 101% |
| Class F | 0.02% | 0.00% | 0.13% | 95% | 102% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 + Fix196 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.10% |  | 0.01% |  | 99% |  | 97% |  |
| Class A2 |  |  |  |  |  | 99% |  | 101% |  |
| Class B | 0.01% | -0.10% |  | -0.25% |  | 99% |  | 99% |  |
| Class C | 0.00% | -0.04% |  | -0.07% |  | 99% |  | 98% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  | 99% |  | 99% |  |
| Class D | 0.02% | 0.18% |  | -0.41% |  | 99% |  | 100% |  |
| Class F | -0.08% | 0.11% |  | -0.01% |  | 101% |  | 100% |  |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 + Fix196 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  | 100% |  | 102% |  |
| Class C | 0.03% | 0.40% |  | -0.18% |  | 100% |  | 98% |  |
| Class E | 0.03% | -0.15% |  | 0.31% |  | 100% |  | 98% |  |
| Overall |  |  |  |  |  | 100% |  | 99% |  |
| Class D | 0.00% | -0.34% |  | -0.23% |  | 99% |  | 99% |  |
| Class F | -0.14% | 0.23% |  | 0.29% |  | 99% |  | 104% |  |

### CE6-2.3b with Inter MTS

Note that AI results are omitted as they are identical to the ones without Inter MTS.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 + Fix196 + Inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  | 97% | 99% |
| Class A2 |  |  |  | 97% | 99% |
| Class B |  |  |  | 97% | 100% |
| Class C |  |  |  | 97% | 103% |
| Class E |  |  |  |  |  |
| Overall |  |  |  | 97% | 100% |
| Class D |  |  |  | 97% | 99% |
| Class F |  |  |  | 97% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 + Fix196 + Inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  | 96% |  | 96% |  |
| Class C | 0.01% | 0.18% |  | 0.20% |  | 96% |  | 99% |  |
| Class E | 0.31% | 0.45% |  | 0.51% |  | 98% |  | 99% |  |
| Overall |  |  |  |  |  | 97% |  | 98% |  |
| Class D | -0.01% | 0.26% |  | 0.30% |  | 97% |  | 101% |  |
| Class F |  |  |  |  |  | 98% |  | 100% |  |

### CE6-2.3c without Inter MTS

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 + Fix 196 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.03% | -0.04% | 90% | 101% |
| Class A2 | 0.00% | -0.01% | -0.02% | 91% | 101% |
| Class B | -0.03% | 0.02% | -0.06% | 89% | 98% |
| Class C | 0.00% | 0.01% | -0.11% | 90% | 100% |
| Class E | -0.03% | -0.06% | 0.11% | 90% | 99% |
| Overall | -0.02% | -0.01% | -0.03% | 90% | 100% |
| Class D | -0.02% | 0.04% | -0.08% | 89% | 100% |
| Class F | 0.01% | 0.01% | 0.10% | 94% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 + Fix 196 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.15% |  | 0.02% |  | 99% |  | 101% |  |
| Class A2 |  |  |  |  |  | 99% |  | 101% |  |
| Class B | 0.00% | 0.00% |  | -0.28% |  | 99% |  | 98% |  |
| Class C | 0.00% | -0.04% |  | -0.06% |  | 100% |  | 101% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  | 99% |  | 100% |  |
| Class D | 0.01% | 0.15% |  | -0.32% |  | 99% |  | 100% |  |
| Class F | -0.07% | 0.22% |  | -0.03% |  | 99% |  | 100% |  |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 + Fix 196 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  | 99% |  | 102% |  |
| Class C | 0.01% | 0.16% |  | -0.01% |  | 100% |  | 102% |  |
| Class E | 0.03% | -0.24% |  | 0.31% |  | 100% |  | 104% |  |
| Overall |  |  |  |  |  | 99% |  | 102% |  |
| Class D | -0.01% | -0.14% |  | -0.66% |  | 100% |  | 100% |  |
| Class F | -0.09% | 0.11% |  | 0.35% |  | 99% |  | 99% |  |

### CE6-2.3c with Inter MTS

Note that AI results are omitted as they are identical to the ones without Inter MTS.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0rc1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  | 97% | 101% |
| Class A2 |  |  |  | 97% | 98% |
| Class B |  |  |  | 97% | 100% |
| Class C |  |  |  | 95% | 99% |
| Class E |  |  |  |  |  |
| Overall |  |  |  | 97% | 100% |
| Class D |  |  |  | 96% | 99% |
| Class F |  |  |  | 97% | 98% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0rc1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  | 96% |  | 103% |  |
| Class C | -0.01% | 0.19% |  | 0.46% |  | 96% |  | 101% |  |
| Class E | 0.26% | 0.09% |  | -0.15% |  | 99% |  | 101% |  |
| Overall |  |  |  |  |  | 97% |  | 102% |  |
| Class D | 0.03% | 0.34% |  | 0.04% |  | 96% |  | 100% |  |
| Class F |  |  |  |  |  | 97% |  | 102% |  |

# References

[1]	C. Hollmann, D. Saffar, P. Wennersten, J. Ström. CE-6 related: Transform Simplification, JVET-M0366, Marrakech, Morocco, January 2019

[2]	X. Zhao, H.E. Egilmez. Description of Core Experiment 6 (CE6): Transforms and transform signaling, JVET-M1026, Marrakech, Morocco, January 2019

[3]	B. Bross et al. Non-CE8: Unified Transform Type Signalling and Residual Coding for Transform Skip, JVET-M0464, Marrakech, Morocco, January 2019

[4] VVC Bug Tracker. Ticket #196: Enc/Dec mismatch for Inter MTS in class F, https://jvet.hhi.fraunhofer.de/trac/vvc/ticket/196, February 2019

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).