| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0492-v2 |
| --- | --- |

| Title: | CBF flags signalling in VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Roman Chernyak Yin Zhao Sergey Ikonin Jianle Chen | Tel: Email: | +7-925-008-12-86 chernyak.roman@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes a modification of VVC 4 specification and VTM 4.0.1 SW aimed to remove inconsistency in part of CBF flags signalling between them. The contribution consists of two aspects. The first aspect proposes to remove hierarchical chroma CBF signalling based on Transform Unit depth from the VTM SW, which is not presented in the specification; and include to the specification missing luma cbf flag deriving methods for normal TU and SBT TU based on chroma CBF flags, which is presented in the SW. The second aspect is proposed on top of the first and assumes unified design of all existing luma cbf flag deriving methods. Both aspects provide negligible coding performance difference in respect to VTM 4.0.1.

# Problem Statement

There is a mismatch between VVC 4 specification draft and VTM4.0 SW concerning CBFs flags signalling, which was discussed after 13th JVET meeting and submitted as issue to JVET bug tracker [1]. The differences are caused by presence in VTM SW HEVC concepts of 1) hierarchical chroma CBFs signalling mechanism; and 2) TU luma cbf flag deriving based on two chroma cbf flags method, which are not presented in the VTM 4 specification.

# Proposed solution

This contribution includes two aspects.

In the first aspect, both SW and specification modifications are proposed. Concerning the SW, it is proposed to remove from the hierarchical chroma CBFs signalling based on Transform Unit depth, which was inherited to VTM from NEXT/JEM times. Concerning the specification, it is proposed to include the luma cbf flag deriving method based on two chroma cbf flags.

The second aspect is applied on top of the first and proposes a unified scheme of luma cbf flag deriving for all TU partitioning methods, which are currently used.

## Aspect 1

There are four possibilities of TU representing in VVC 4:

- Normal TU equals to CU size (no split);

- SBT TUs partitioning;

- ISP TUs partitioning;

- TUs partitioning caused by maximal transform size restriction.

Following tables demonstrates these possibilities.

Table 1 – VVC 4 transform_tree syntax table

| transform_tree( x0, y0, tbWidth, tbHeight , treeType) { | Descriptor |
| --- | --- |
| InferTuCbfLuma = 1 |  |
| if( IntraSubPartSplitType = = NO_ISP_SPLIT ) { |  |
| if( tbWidth > MaxTbSizeY ¦ ¦ tbHeight > MaxTbSizeY ) { |  |
| trafoWidth = ( tbWidth > MaxTbSizeY ) ? (tbWidth / 2) : tbWidth |  |
| trafoHeight = ( tbHeight > MaxTbSizeY ) ? (tbHeight / 2) : tbHeight |  |
| transform_tree( x0, y0, trafoWidth, trafoHeight ) |  |
| if( tbWidth > MaxTbSizeY ) |  |
| transform_tree( x0 + trafoWidth, y0, trafoWidth, trafoHeight, treeType ) |  |
| if( tbHeight > MaxTbSizeY ) |  |
| transform_tree( x0, y0 + trafoHeight, trafoWidth, trafoHeight, treeType ) |  |
| if( tbWidth > MaxTbSizeY && tbHeight > MaxTbSizeY ) |  |
| transform_tree( x0 + trafoWidth, y0 + trafoHeight, trafoWidth, trafoHeight, treeType ) |  |
| } else { |  |
| transform_unit( x0, y0, tbWidth, tbHeight, treeType, 0 ) |  |
| } |  |
| } else if( cu_sbt_flag ) { |  |
| if( !cu_sbt_horizontal_flag ) { |  |
| trafoWidth = tbWidth * SbtNumFourthsTb0 / 4 |  |
| transform_unit( x0, y0, trafoWidth, tbHeight, treeType , 0 ) |  |
| transform_unit( x0 + trafoWidth, y0, tbWidth − trafoWidth , tbHeight, treeType , 1 ) |  |
| } else { |  |
| trafoHeight = tbHeight * SbtNumFourthsTb0 / 4 |  |
| transform_unit( x0, y0, tbWidth, trafoHeight, treeType , 0 ) |  |
| transform_unit( x0, y0 + trafoHeight, tbWidth, tbHeight − trafoHeight, treeType , 1 ) |  |
| } |  |
| } else if( IntraSubPartitionsSplitType = = ISP_HOR_SPLIT ) { |  |
| trafoHeight = tbHeight / NumIntraSubPartitions |  |
| for( partIdx = 0; partIdx < NumIntraSubPartitions; partIdx++ ) |  |
| transform_unit( x0, y0 + trafoHeight * partIdx, tbWidth, trafoHeight, treeType, partIdx ) |  |
| } else if( IntraSubPartitionsSplitType = = ISP_VER_SPLIT ) { |  |
| trafoWidth = tbWidth / NumIntraSubPartitions |  |
| for( partIdx = 0; partIdx < NumIntraSubPartitions; partIdx++ ) |  |
| transform_unit( x0 + trafoWidth * partIdx, y0, trafoWidth, tbHeight, treeType, partIdx ) |  |
| } |  |
| } |  |

Table 2 – VVC 4 transform_unit syntax table

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex ) { | Descriptor |
| --- | --- |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !( cu_sbt_flag && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex < NumIntraSubPartitions − 1 ¦ ¦ !InferTuCbfLuma ) ) ) |  |
| tu_cbf_luma[ x0 ][ y0 ] | ae(v) |
| if (IntraSubPartitionsSplitType != ISP_NO_SPLIT ) |  |
| InferTuCbfLuma = InferTuCbfLuma && !tu_cbf_luma[ x0 ][ y0 ] |  |
| } |  |
| if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) { |  |
| if( ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !( cu_sbt_flag && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex = = NumIntraSubPartitions − 1 ) ) ) { |  |
| tu_cbf_cb[ x0 ][ y0 ] | ae(v) |
| tu_cbf_cr[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| … |  |
| } |  |

It can be noted that according to Table 1 and Table 2, both luma and chroma CBFs signalling is performed independently from each other’s and there is no hierarchical dependencies for chroma CBF flags.

From the SW perspective, VTM4.0.1 includes both of abovementioned aspects.

The hierarchical chroma CBFs signalling method was tested in VTM4.0.1 and demonstrates negligible impact to coding efficiency. On the other hand, support of this feature requires additional non-trivial logic in both SW and specification. It is proposed to remove from the SW hierarchical chroma CBFs signalling method and include to the spec luma CBF deriving.

Following table demonstrates modified syntax table.

Table 3 – Proposed transform_unit syntax table (aspect 1)

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex ) { | Descriptor |
| --- | --- |
| if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) { |  |
| if( ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !( cu_sbt_flag && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex = = NumIntraSubPartitions − 1 ) ) ) { |  |
| tu_cbf_cb[ x0 ][ y0 ] | ae(v) |
| tu_cbf_cr[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !( cu_sbt_flag && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) && (CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ tu_cbf_cb[ x0 ][ y0 ] ¦ ¦ tu_cbf_cr[ x0 ][ y0 ] ) ) ¦ ¦( (IntraSubPartitionsSplitType != ISP_NO_SPLIT) && ( subTuIndex < NumIntraSubPartitions − 1 ¦ ¦ !InferTuCbfLuma ) ) ) |  |
| tu_cbf_luma[ x0 ][ y0 ] | ae(v) |
| … |  |
| } |  |

## Aspect 2

It can be noted, that current version of SW utilizes luma cbf deriving for all possible transform unit partitioning methods besides the partitioning caused by maximal transform size restriction. For design unification, aspect 2 proposes to apply similar method of luma cbf deriving for TUs divided in order to satisfy maximal TU restriction. This unification can share existing mechanism of last luma cbf deriving for ISP. The table below demonstrates proposed modification.

Table 4 - Proposed transform_unit syntax table (aspect 2)

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex ) { | Descriptor |
| --- | --- |
| if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) { |  |
| if( ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !( cu_sbt_flag && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex = = NumIntraSubPartitions − 1 ) ) ) { |  |
| tu_cbf_cb[ x0 ][ y0 ] | ae(v) |
| tu_cbf_cr[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !( cu_sbt_flag && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) && (CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ tu_cbf_cb[ x0 ][ y0 ] ¦ ¦ tu_cbf_cr[ x0 ][ y0 ] ) ) ¦ ¦( (IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ (CbWidth[ x0 ][ y0 ] > MaxTbSizeY ¦ ¦ CbHeight[ x0 ][ y0 ] > MaxTbSizeY )) && ( subTuIndex < NumIntraSubPartitions − 1 ¦ ¦ !InferTuCbfLuma ) ) ) |  |
| tu_cbf_luma[ x0 ][ y0 ] | ae(v) |
| if (CuPredMode[ x0 ][ y0 ] == MODE_INTRA) |  |
| if (IntraSubPartitionsSplitType != ISP_NO_SPLIT ) |  |
| InferTuCbfLuma = InferTuCbfLuma && !tu_cbf_luma[ x0 ][ y0 ] |  |
| else |  |
| InferTuCbfLuma = 0 |  |
| else |  |
| InferTuCbfLuma = InferTuCbfLuma && !tu_cbf_luma[ x0 ][ y0 ] && !tu_cbf_cb[ x0 ][ y0 ] && !tu_cbf_cr[ x0 ][ y0 ] |  |
| … |  |
| } |  |

## Simulation results

### Aspect 1

Table 5 - RA simulation results of aspect 1

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0.1 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | 0.00% | 0.00% |  | 0.00% |  | 98% |  | 98% |  |
| Class A2 | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 100% |  |
| Class B | 0.00% | 0.00% |  | 0.00% |  | 99% |  | 97% |  |
| Class C | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 100% |  |
| Class E | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 97% |  |
| Overall | 0.00% | 0.00% |  | 0.00% |  | 99% |  | 98% |  |
| Class D | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 97% |  |
| Class F | 0.00% | 0.00% |  | 0.00% |  | 99% |  | 94% |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |
|  | Over VTM4.0.1 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | -0.04% | 0.32% |  | 0.21% |  | 100% |  | 100% |  |
| Class A2 | 0.01% | 0.11% |  | 0.00% |  | 100% |  | 100% |  |
| Class B | 0.02% | 0.13% |  | 0.04% |  | 100% |  | 100% |  |
| Class C | 0.03% | 0.06% |  | -0.03% |  | 100% |  | 100% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.01% | 0.15% |  | 0.05% |  | 100% |  | 100% |  |
| Class D | 0.00% | 0.00% |  | 0.00% |  | 101% |  | 104% |  |
| Class F | 0.01% | 0.03% |  | 0.01% |  | 100% |  | 100% |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
|  | Over VTM4.0.1 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  |
| Class C | -0.01% | -0.29% |  | -0.12% |  | 100% |  | 100% |  |
| Class E | -0.01% | -0.51% |  | 0.30% |  | 99% |  | 99% |  |
| Overall | #VALUE! | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  |
| Class D | 0.01% | 0.04% |  | -0.14% |  | 99% |  | 98% |  |
| Class F | 0.05% | 0.42% |  | -0.08% |  | 100% |  | 99% |  |

Anchor: VTM4.0.1

Test: Aspect 1 modifications

### Aspect 2

Table 6 - RA simulation results of aspect 2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | 0.03% | 0.06% | -0.03% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Anchor: VTM4.0.1

Test: Aspect1 + Aspect 2 modifications

# References

- JVET VVC bug tracker. Ticket #203. URL: https://jvet.hhi.fraunhofer.de/trac/vvc/ticket/203

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).