| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0493_v2 |
| --- | --- |

| Title: | CE1-related: Multiplication-free bilateral loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jacob Ström, Per Wennersten, Jack Enhorn and Rickard Sjöberg Torshamnsgatan 23 164 80 Stockholm Sweden | Tel: Email: | +46702672192 {jacob, per, jack, rickard} @ {ericsson.com} |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution presents a version of the bilateral filter as a loop filter, placed directly before deblocking. It is reported that the shape of the filter is 3×3 samples, and that it uses five 16-item-long LUT tables, that together need 60 bytes of storage. It is stated that a total of 18 additions, 8 shifts and 8 checks are used per pixel, of which five of the additions are reported to be rounding operations. BDR figures for luma are -0.46% / -0.61%  for AI / RA respectively over VTM-4.0.

# Introduction

In CE1-1.5, bilateral filtering has been investigated as a conventional loop filter, i.e., at the same place in the decoding chain as deblocking, SAO and ALF. Whereas CE1-1.5 (JVET-N0489) provides a clear BD-rate performance improvement over VTM-4.0, (-0.16%/-0.44%/-0.26% for AI/RA/LD) there is still room for improvement. One problem is that CE1-1.5 uses two multiplications with input width 7 bits by 9 bits. This is expensive to implement in hardware in terms of silicon surface area, especially since four samples likely need to be filtered simultaneously. Another solution is to do Hadamard filtering instead, as proposed in CE1-2.3 (JVET-N0478) and JVET-N0479. However, this requires the computation of a Hadamard forward and inverse transform, which is also expensive. In this contribution we present a solution that needs neither a multiplication nor a transform to filter the samples, making it very efficient to implement in terms of silicon surface area.

# Filtering placement and constraints

The filter is placed right before deblocking in the decoding pipeline. All types of blocks are filtered, except blocks with a dimension smaller than 4, as well as blocks that do not have any transform coefficients, such as blocks with cbf=0 or the part of an SBT block that is without coefficients. When filtering a block, the input consists of the block itself plus a one-sample wide border around the block (above, below, to the left and to the right). The only exception is for samples above and below that belong to a different CTU, since that would require an additional line buffer, or blocks at the image border. Here samples are padded instead. Only luma samples are filtered.

# Filtering a sample

Filtering a sample is performed with the help of its 8-neighbors. These are shown in Figure 1, where the sample to be filtered is denoted IC for “center intensity”:

| INW | IA | INE |
| --- | --- | --- |
| IL | IC | IR |
| ISW | IB | ISE |

Figure 1. The sample in the center IC is filtered with the help of the surrounding samples.

Here, IR denotes the sample to the right, IL the one to the left, IA and IB denotes the samples above and below respectively. Furthermore, cardinal directions are used to denote the diagonal neighbors. Hence INW is short for “north-west” and denotes the sample one step up and one step to the left, etc.

During filtering, we only need to carry out calculations for the neighbors marked with gray in Figure 1. The reason is that the filter contribution from the remaining neighbors can be easily inferred from previously filtered samples. As an example, the contribution from IL for the current sample is equal to the negative of the contribution from IR when filtering the sample to the left.

Taking the right neighbor IR as an example, we first calculate the downshifted difference against the center sample IC:

ΔIR=IR-IC+4≫3        (Eqn 1)

This value is then used to obtain a modifier value mR using a look-up table (LUT). Only positive values are stored in the LUT, so if the value is negative, we negate the value of the LUT output. Furthermore, since it is difficult to implement look-up tables longer than 16 items using SIMD instructions, we clamp the value at 15:

cIR=min15, ΔIR                            (Eqn 2)
mR=LUTcIR        if ΔIR≥0    -LUTcIR    otherwise        (Eqn 3)

The modifier value mB for the neighbor below is calculated analogously. The LUT used depends on the qp:

LUT=0,4,4,4,3,2,1,2,1,1,1,1,0,1,1,-1                                       if 17<qp≤22
LUT=0,8,11,11,7,5,5,4,5,4,4,2,2,2,2,-2                                  if 22<qp≤27
LUT=0,9,16,19,22,22,20,15,12,12,11,9,9,7,8,-3                if 27<qp≤32       (Eqn 4)
LUT=0,12,21,28,33,36,40,40,40,36,29,22,19,17,15,-3    if 32<qp≤37
LUT=0,17,23,33,37,41,44,44,45,44,42,27,22,17,15,-3    if qp>37

For the diagonal neighbors, an extra shift by one is used to compensate for the fact that a diagonal neighbor is farther from the center than a neighbor to the right or below:

ΔISW=ISW-IC+4≫3                                       (Eqn 5)
cISW=min15, ΔISW                                             (Eqn 6)
mSW=LUTcISW≫1           if ΔIR≥0-LUTcISW≫1    otherwise             (Eqn 7)

Having calculated mR, mB, mSW and mSE for the current sample, and retrieved mA, mL, mNW and mNE from previously filtered neighbors, we can now form the sum of the modifier values.

msum=mA+mB+mL+mR+mNW+mNE+mSW+mSE.           (Eqn 8)

For small intra blocks, we want to filter more strongly, which corresponds to an msum value of larger magnitude. For large blocks, we want to filter more weakly. This can be achieved by calculating a corrected modifier value

cmsum=k1 &msum≪1+k2 & msum≫1,          (Eqn 9)

where k1 and k2 are one-bit numbers and the & operator calculates logical AND between the left operand and all of the bits of the right operand. Setting k1 and k2 according to Table 1 gives the listed effective strengths.

|  | D=min⁡(blockwidth,blockheight) |  |  |
| --- | --- | --- | --- |
|  | D≤4 | 4<D<16 | D≥16 |
| intra block | k1=1, k2=1 strength = 1.5 | k1=1, k2=0 strength = 1 | k1=0, k2=1 strength = 0.5 |
| inter block | k1=1, k2=0 strength = 1 | k1=1, k2=0 strength = 1 | k1=0, k2=1 strength = 0.5 |

Table 1: Using the following combinations of k1 and k2 it is possible to increase the filtering strength for small intra blocks and decrease it for large inter blocks.

In software, Equation 9 is likely more efficiently implemented using a multiplication cmsum=k*m_sum≫1, where k=1, 2 or 3, since multiplications are very inexpensive on modern CPUs.

The filtered sample IF is finally computed as

IF=IC+cmsum+4≫3     (Eqn 10)

## Hardware complexity

In this section we count the number of arithmetic operations (mult, add, shift, check) needed to filter a sample according to the method presented in the contribution. We also estimate the size of the LUT.

### Arithmetic operations

First, we count the number of operations for IB or IR:

| Step | add +(round add) | add width | shift | check |
| --- | --- | --- | --- | --- |
| ΔIR=IR-IC+4≫3 (Eqn 1) | 1 (1) | 10, 11 | 1 |  |
| cIR=min15, ΔIR (Eqn 2) |  |  |  | 2 |
| mR=LUTcIR if ΔIR≥0 -LUTcIR otherwise (Eqn 3) |  |  |  |  |
| sum | 1 (1) |  | 1 | 2 |

Table 2: Operations to process either IB or IR.

It should be noted that since the result mR is going to be used in an addition later, it is possible to invert the bits of LUT(cIR) and add 1 through the carry to get -LUT(cIR). Hence no addition is needed for the minus sign.

For the two diagonal neighbors ISW and ISE, we need an additional shift. We can now sum together the rest of the operations needed per sample:

| Step | add + (round add) | add width | shift | check |
| --- | --- | --- | --- | --- |
| mR and mB | 2 (2) | 10, 11 | 2 | 4 |
| mSW and mSE | 2 (2) | 10, 11 | 4 | 4 |
| msum=mA+mB+mL+mR+mNW+mNE+mSW+mSE (Eqn 8) | 7 | 6,7,8 |  |  |
| cmsum=k1 &msum≪1+k2 & msum≫1, (Eqn 9) | 1 | 10 | 1 |  |
| IF=IC+cmsum+4≫3 (Eqn 10) | 1 (1) | 10 | 1 |  |
| total per pixel | 13 (5) |  | 8 | 8 |

Table 3: Total number of operations per sample

The notation 13 (5) means that there are in total 18 additions, five of which are simple additions used for rounding, namely the one adding +4 in Equation 1 (of which there are four per sample) and the one adding four in Equation 10 (of which there is one). Adding four in this case is even less expensive than adding 1: As an example, the second term in Equation 10,

cmsum+4≫3          (Eqn 11)

can hardware be implemented in hardware as

cmsum≫2+1≫1        (Eqn 12)

which is an addition that is two bits smaller than if we had added 1 to cmsum. In hardware of course fixed shifts are free so it should not be important that the number of shifts have increased from one to two.

Hence it can be noted that the number of additions is 13 plus five 1-bit rounding additions.

### Addition width

The input widths of the marked additions are summarized in Table 4 below.

| Step | width | number |
| --- | --- | --- |
| ΔIR=IR-IC+4≫3 (Eqn 1) | 10 | 4 |
| ΔIR=IR-IC+4≫3 (Eqn 1) | 11 | 4 |
| msum=mA+mB+mL+mR+mNW+mNE+mSW+mSE (Eqn 8) | 6 | 2 |
| msum=mA+mB+mL+mR+mNW+mNE+mSW+mSE (Eqn 8) | 7 | 3 |
| msum=mA+mB+mL+mR+mNW+mNE+mSW+mSE (Eqn 8) | 8 | 1 |
| msum=mA+mB+mL+mR+mNW+mNE+mSW+mSE (Eqn 8) | 9 | 1 |
| cmsum=k1 &msum≪1+k2 & msum≫1, (Eqn 9) | 11 | 1 |
| IF=IC+cmsum+4≫3 (Eqn 10) | 11 | 1 |
| IF=IC+cmsum+4≫3 (Eqn 10) | 10 | 1 |

Table 4: Input width of the additions. If the input widths are different, the maximum is reported.

Hence in summary, there are 6 11-bit adders, 5 10-bit adders, 1 9-bit adder, 1 8-bit adder, 3 7-bit adders and 2 6-bit adders.

### Memory requirements

We also need to store the values of the LUTs:

LUT=0,4,4,4,3,2,1,2,1,1,1,1,0,1,1,-1                                       if 17<qp≤22
LUT=0,8,11,11,7,5,5,4,5,4,4,2,2,2,2,-2                                  if 22<qp≤27
LUT=0,9,16,19,22,22,20,15,12,12,11,9,9,7,8,-3                if 27<qp≤32       (Eqn 13)
LUT=0,12,21,28,33,36,40,40,40,36,29,22,19,17,15,-3    if 32<qp≤37
LUT=0,17,23,33,37,41,44,44,45,44,42,27,22,17,15,-3    if qp>37

These values between -3 and 45. Since some values are negative, normally this would require seven bits to store. However, it is possible to store these values in 6 bits and then recover the MSB (sign bit) by using an AND gate on the top most stored bits, as is shown here:

| stored bits | interpreted value | AND of top two stored bits | AND output plus stored bits |
| --- | --- | --- | --- |
| 000000 | 0 | 0 | 0000000 = 0 |
| 000001 | 1 | 0 | 0000001 = 1 |
| … | … | … | … |
| 101101 | 45 | 0 | 0101101 = 45 |
| 101101 | unused | unused | unused |
| … | … | … | … |
| 111100 | unused | unused | unused |
| 111101 | -3 | 1 | 1111101 = -3 |
| 111110 | -2 | 1 | 1111110 = -2 |
| 111111 | -1 | 1 | 1111111 = -1 |

Table 4: Recovering the seven-bit two-complement number from stored 6-bit value using an AND gate.

Thus, the number of stored values is 16*5*6=480 bits or 60 bytes.

# Results

Two sets of results are presented. Due to a late mistake, our first variant of the results allows neighboring samples outside the CTU to be accessed. For variant 2, this has been corrected so that reading samples outside the CTU is only allowed to the left and to the right, but not above and below. At the time of first upload, all results for variant 2 are not available. Therefore, we provide version 1 as information, since we expect that the difference between the two variants to be small.

The contribution has also been updated with a third variant, where also blocks width dimensions smaller than 4 are encoded, Here the qp22 results are still pending are still running for the A-class sequences, but an estimate can be made by using the results from variant 2 for these qps. Thjs estimate gives -0.65% for RA.

## Variant 1 (reading outside the CTU)

### Variant 1 AI

|  | Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.30% | 0.30% | 0.23% |  |  |
| Class A2 | -0.43% | 0.30% | 0.20% |  |  |
| Class B | -0.36% | 0.35% | 0.36% |  |  |
| Class C | -0.69% | 0.17% | 0.25% |  |  |
| Class E | -0.51% | 0.29% | 0.17% |  |  |
| Overall | -0.46% | 0.28% | 0.26% |  |  |
| Class D | -0.58% | 0.35% | 0.30% |  |  |
| Class F | -0.92% | 0.12% | -0.04% |  |  |

### Variant 1 RA

|  | Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.40% | 0.00% | 0.02% |  |  |
| Class A2 | -0.59% | 0.08% | 0.06% |  |  |
| Class B | -0.59% | 0.17% | 0.05% |  |  |
| Class C | -0.78% | 0.32% | 0.18% |  |  |
| Class E |  |  |  |  |  |
| Overall | -0.61% | 0.16% | 0.08% |  |  |
| Class D | -0.77% | 0.07% | -0.04% |  |  |
| Class F | -1.14% | 0.16% | 0.06% |  |  |

### Variant 1 LD

|  | Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.46% | 1.31% | 0.90% |  |  |
| Class C | -0.78% | 1.19% | 0.89% |  |  |
| Class E | -0.50% | 0.65% | 1.82% |  |  |
| Overall | -0.58% | 1.11% | 1.13% |  |  |
| Class D | -1.01% | 1.32% | 1.23% |  |  |
| Class F | -1.29% | 0.69% | 0.51% |  |  |

## Variant 2 (not reading outside CTU above and below)

### Variant 2 AI

First decoder time column is non-SIMD code and second column is SIMD code. For the non-SIMD the test and anchor decodings were not run on the same manchine and these figures are therefore less reliable. For the SIMD figures the anchor and test were run on the same manchine and these figures are therefore more reliable.

|  | Over VTM-4.0 |  |  |  |  | SIMD |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | DecT |
| Class A1 | -0.28% | 0.31% | 0.18% | 111% | 120% | 104% |
| Class A2 | -0.43% | 0.36%! | 0.19% | 109% | 114% | 107% |
| Class B | -0.35% | 0.32% | 0.38% | 111% | 114% | 105% |
| Class C | -0.69% | 0.12% | 0.27% | 112% | 110% | 107% |
| Class E | -0.52% | 0.30% | 0.09% | 110% | 113% | 106% |
| Overall | -0.46% | 0.28% | 0.24% | 111% | 114% | 106% |
| Class D | -0.57% | 0.24% | 0.27% | 111% | 105% | 105% |
| Class F | -0.95% | 0.05% | 0.10% | 108% | 114% | 109% |

### Variant 2 RA

First decoder time column is non-SIMD code and second column is SIMD code. For the non-SIMD the test and anchor decodings were not run on the same manchine and these figures are therefore less reliable. For the SIMD figures the anchor and test were run on the same manchine and these figures are therefore more reliable.

|  | Over VTM-4.0 |  |  |  |  | SIMD |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | DecT |
| Class A1 | -0.40% | 0.13% | 0.18% | 103% | 100% | 103% |
| Class A2 | -0.56% | 0.09% | 0.05% | 103% | 99% | 99% |
| Class B | -0.61% | 0.34% | 0.15% | 105% | 103% | 102% |
| Class C | -0.79% | 0.47% | 0.25% | 106% | 101% | 100% |
| Class E |  |  |  |  |  |  |
| Overall | -0.61% | 0.28% | 0.16% | 104% | 101% | 101% |
| Class D | -0.75% | 0.10% | 0.12% | 104% | 105% | 101% |
| Class F | -1.15% | 0.04% | 0.05% | 115% | 104% | 102% |

### Variant 2 LD

First decoder time column is non-SIMD code and second column is SIMD code. For the non-SIMD the test and anchor decodings were not run on the same manchine and these figures are therefore less reliable. For the SIMD figures the anchor and test were run on the same manchine and these figures are therefore more reliable.

|  | Over VTM-4.0 |  |  |  |  | SIMD |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |  |
| Class C | -0.73% | 1.37% | 0.99% | 101% | 98% | 100% |
| Class E | -0.34% | 1.10% | 0.70% | 102% | 96% | 98% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |  |
| Class D | -1.00% | 1.19% | 1.04% | 107% | 103% | 101% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | 99% |

## Variant 3

The difference between variant 2 and variant 3 is that the latter filters also smaller blocks.

### Variant 3 RA

QP 22 results are still missing for class A, but assuming these would be equivalent to the result in variant 2, the estimated BDR figure is -0.65% for RA. The run times were not run on the same machine and are therefore less reliable.

|  | Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.66% | 0.20% | -0.07% | 107% | 104% |
| Class C | -0.84% | 0.04% | -0.04% | 106% | 101% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.82% | -0.02% | -0.24% | 107% | 106% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Proposal

We propose that this contribution is adopted (spec language included in zip file) into the next version of VVC.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).