| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0494-v3 |
| --- | --- |

| Title: | AHG17: Dependent random access point pictures in VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Martin Pettersson Rickard Sjöberg Mitra Damghanian | Email: | martin.m.pettersson@ericsson.com rickard.sjoberg@ericsson.com mitra.damghanian@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes to include dependent random access point (DRAP) pictures to the VVC draft, similar to the DRAP pictures in HEVC. A DRAP picture is a RAP picture that may use the previous IRAP picture in decoding order for temporal prediction.

In contrast to DRAP pictures in HEVC, the following is proposed for VVC:

- To signal DRAP indication as a VCL NAL unit type in the NAL unit header

- To signal a flag in SPS whether the CVS may contain DRAP pictures or not

In v2 of the contribution the presented slide deck is added that includes some use cases for DRAP.

In v3 of the contribution, an option 2 is added with proposed specification text for a dependent random access point indication SEI message for VVC. Compared to the dependent random access point indication SEI message in HEVC, the following changes are proposed to adapt the HEVC text for VVC:

- TRAIL_R is replaced with TRAIL_NUT

- nuh_layer_id is removed

- RPS is replaced with RPL

# Introduction

HEVC specifies the dependent random access point (DRAP) indication SEI message that is used to indicate the presence of a DRAP picture in the bitstream. A DRAP picture is a RAP picture that may use the previous IRAP picture in decoding order for temporal prediction. A DRAP picture may be used to increase the frequency of random access points (RAPs) without significantly increasing the required bitrate. To perform a random access operation at a DRAP picture, the previous IRAP picture in the bitstream must be made available as illustrated in Figure 1.

Fig. 1. Random access at a DRAP picture is performed by first decoding the associated IRAP picture.

DRAP pictures may for instance be useful for fast random access in MPEG-DASH and in server-controlled streaming, fast channel switching for broadcasted TV and IPTV, searching in stored static content such as surveillance video and for tuning into video conferences with a massive number of participants [1].

The syntax and semantics for the DRAP indication SEI message in HEVC is as follows:

| dependent_rap_indication( payloadSize ) { | Descriptor |
| --- | --- |
| } |  |

The picture associated with a dependent random access point indication SEI message is referred to as a DRAP picture.

The presence of the dependent random access point indication SEI message indicates that the constraints on picture order and picture referencing specified in this subclause apply. These constraints can enable a decoder to properly decode the DRAP picture and the pictures that follow it in both decoding order and output order without needing to decode any other pictures except the associated IRAP picture.

The constraints indicated by the presence of the dependent random access point indication SEI message are as follows:

–	The DRAP picture shall be a TRAIL_R picture with TemporalId equal to 0 and nuh_layer_id equal to 0.

–	The DRAP picture shall not include any pictures in its RPS lists RefPicSetStCurrBefore, RefPicSetStCurrAfter, and RefPicSetLtCurr except its associated IRAP picture.

–	Any picture that follows the DRAP picture in both decoding order and output order shall not include, in its RPS, any picture that precedes the DRAP picture in decoding order or output order with the exception of the IRAP picture associated with the DRAP picture.

# Motivation

We believe DRAP pictures are useful in certain scenarios for decreasing the bitrate for bitstreams supporting random access. The frequency of random access points can alternatively be increased without significantly increasing the bitrate.

In HEVC, a DRAP picture is indicated with an SEI message. This is a valid solution also for VVC. On the other hand, having DRAP normatively specified in the VVC specification would have the following additional benefits:

-	Decoder support. An encoder would know that an unknown decoder could support the DRAP functionality.

-	Improved compression efficiency. When the encoder knows that DRAP is supported in the decoder, the frequency of IRAP pictures and potentially the bitrate can be reduced. An encoder may then replace some of the expensive IRAP pictures with less expensive DRAP pictures.

We therefore think it is a good solution to indicate the presence of a DRAP picture in a normative way and we propose to signal DRAP as a VCL NAL unit type in the NAL unit header.

We also believe it could be useful for a decoder to know if a DRAP picture may be expected in a bitstream, such that the associated IRAP picture could be made available. We therefore also propose to signal in the SPS whether a CVS may contain DRAP pictures or not.

# Option 1 proposal

It is proposed to include DRAP pictures to the VVC draft, similar to the DRAP pictures in HEVC. In contrast to DRAP pictures in HEVC, the following is proposed for VVC:

- To signal DRAP indication as a VCL NAL unit type in the NAL unit header

- To signal a flag in SPS whether the CVS may contain DRAP pictures or not

## Proposed changes to the VVC draft text

The following changes marked in red are proposed on top of the VVC draft JVET-M1001-v6.

#### 3	Definitions

…

3.34	dependent random access point (DRAP) access unit: An access unit in which the coded picture is a DRAP picture.

3.35	dependent random access point (DRAP) picture: A coded picture for which each VCL NAL unit has nal_unit_type equal to DRAP_NUT.

NOTE – A DRAP picture does not refer to any pictures other than its associated IRAP picture for inter prediction in its decoding process. Provided the necessary parameter sets are available when they need to be activated and that the associated IRAP picture is available, the DRAP picture and all subsequent pictures in decoding order can be correctly decoded without performing the decoding process of any pictures that precede the DRAP picture in decoding order, other than the associated IRAP picture.

…

# 4     Abbreviations

# …

# DRAP	Dependent Random Access Point

…

#### 7.3.3.2	General constraint information syntax

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| general_progressive_source_flag | u(1) |
| general_interlaced_source_flag | u(1) |
| general_non_packed_constraint_flag | u(1) |
| general_frame_only_constraint_flag | u(1) |
| intra_only_constraint_flag | u(1) |
| max_bitdepth_constraint_idc | u(4) |
| max_chroma_format_constraint_idc | u(2) |
| frame_only_constraint_flag | u(1) |
| no_dependent_random_access_point_flag | u(1) |
| no_qtbtt_dual_tree_intra constraint_flag | u(1) |
| no_sao_constraint_flag | u(1) |
| no_alf_constraint_flag | u(1) |
| no_pcm_constraint_flag | u(1) |
| … |  |
| } |  |

no_dependent_random_access_point_flag equal to 1 specifies that it is a requirement of bitstream conformance that no access unit in the CVS shall have nal_unit_type equal to DRAP_NUT. no_dependent_random_access_point_flag equal to 0 does not impose a constraint.

#### 7.4.2.2 	NAL unit header semantics

…

Table 7-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 | TRAIL_NUT | Coded tile group of a non- STSA trailing picture tile_group_layer_rbsp( ) | VCL |
| 1 | STSA_NUT | Coded tile group of an STSA picture tile_group_layer_rbsp( ) | VCL |
| 2 | RASL_NUT | Coded tile group of a RASL picture tile_group_layer_rbsp( ) | VCL |
| 3 | RADL_NUT | Coded tile group of a RADL picture tile_group_layer_rbsp( ) | VCL |
| 4 | DRAP_NUT | Coded tile group of a DRAP picture tile_group_layer_rbsp( ) | VCL |
| 5..7 | RSV_VCL_5.. RSV_VCL_7 | Reserved non-IRAP VCL NAL unit types | VCL |
| 8 9 | IDR_W_RADL IDR_N_LP | Coded tile group of an IDR picture tile_group_layer_rbsp( ) | VCL |
| 10 | CRA_NUT | Coded tile group of a CRA picture tile_group_layer_rbsp( ) | VCL |
| 11 12 13 | RSV_IRAP_VCL11 RSV_IRAP_VCL12 RSV_IRAP_VCL13 | Reserved IRAP VCL NAL unit types | VCL |
| 14..15 | RSV_VCL14.. RSV_VCL15 | Reserved non-IRAP VCL NAL unit types | VCL |
| 16 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 17 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 18 | APS_NUT | Adaptation parameter set adaptation_parameter_set_rbsp( ) | non-VCL |
| 19 | AUD_NUT | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 20 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | non-VCL |
| 21 | EOB_NUT | End of bitstream end_of_bitstream_rbsp( ) | non-VCL |
| 22, 23 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) | non-VCL |
| 24..27 | RSV_NVCL24.. RSV_NVCL27 | Reserved non-VCL NAL unit types | non-VCL |
| 28..31 | UNSPEC28.. UNSPEC31 | Unspecified non-VCL NAL unit types | non-VCL |

NOTE 3 – A clean random access (CRA) picture may have associated RASL or RADL pictures present in the bitstream.

NOTE 4 – An instantaneous decoding refresh (IDR) picture having nal_unit_type equal to IDR_N_LP does not have associated leading pictures present in the bitstream. An IDR picture having nal_unit_type equal to IDR_W_RADL does not have associated RASL pictures present in the bitstream, but may have associated RADL pictures in the bitstream.

The constraints on a DRAP picture with nal_unit_type equal to DRAP_NUT are as follows:

- The DRAP picture shall be a trailing picture with TemporalId equal to 0.

- The DRAP picture shall not include any pictures in its RPL except its associated IRAP picture.

- Any picture that follows the DRAP picture in both decoding order and output order shall not include, in its RPL, any picture that precedes the DRAP picture in decoding order or output order with the exception of the IRAP picture associated with the DRAP picture.

NOTE 5 - These constraints enables a decoder to properly decode the DRAP picture and the pictures that follow it in both decoding order and output order without needing to decode any other pictures except the associated IRAP picture.

# Option 2 proposal

The HLS BoG (JVET-N0724v6) recommended to copy the HEVC dependent random access indication SEI message (with modifications as appropriate to adapt to VVC). Option 2 of the contribution proposes, as recommended by the HLS BoG, to signal the dependent random access point indication in an SEI message for VVC.

## Proposed additions to the VVC draft text

The following text is proposed to be added to the next version of the VVC draft. Changes compared to HEVC version 5 is highlighted in yellow.

# Abbreviations and acronyms

…

DRAP	Dependent Random Access Point

…

Dependent random access point indication SEI message syntax

| dependent_rap_indication( payloadSize ) { | Descriptor |
| --- | --- |
| } |  |

Dependent random access point indication SEI message semantics

The picture associated with a dependent random access point indication SEI message is referred to as a DRAP picture.

The presence of the dependent random access point indication SEI message indicates that the constraints on picture order and picture referencing specified in this subclause apply. These constraints can enable a decoder to properly decode the DRAP picture and the pictures that follow it in both decoding order and output order without needing to decode any other pictures except the associated IRAP picture.

The constraints indicated by the presence of the dependent random access point indication SEI message are as follows:

–	The DRAP picture shall be a TRAIL_NUTTRAIL_R picture with TemporalId equal to 0 and nuh_layer_id equal to 0.

–	The DRAP picture shall not include any pictures in its RPLRPS lists RefPicList[ 0 ] and RefPicList[ 1 ] RefPicSetStCurrBefore, RefPicSetStCurrAfter, and RefPicSetLtCurr except its associated IRAP picture.

–	Any picture that follows the DRAP picture in both decoding order and output order shall not include, in its RPLRPS, any picture that precedes the DRAP picture in decoding order or output order with the exception of the IRAP picture associated with the DRAP picture.

# Results for HEVC

This section provides experimental results comparing compression efficiency for IRAP and DRAP pictures for HEVC. The experiments are based on the HEVC reference software HM16.2 and have been run on the RA Main test configuration of the JCT-VC HEVC common test conditions (CTC) described in JCTVC-J1100.

Modifications were made to the HM16.2 for the DRAP simulations. Long-term pictures were implemented in the encoder to enable referencing associated IRAP pictures outside the current GOP. It was also made sure that the reference picture set (RPS) for each DRAP picture did not contain any pictures in RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetLtCurr except the associated IRAP picture. For the pictures following a DRAP picture in output order it was made sure that their RPS did not contain any pictures preceding the DRAP picture except the IRAP picture associated with the DRAP picture.

In a first experiment all IRAP pictures except the first one in each CTC sequence are replaced by DRAP pictures. This experiment simulates having the IRAP pictures far apart in the bitstream, which may be beneficial for use cases such as searching in static video surveillance recordings. The results are shown in Figure 2 where CTC is the anchor. Class F that comprises sequences with low motion benefits the most from using DRAP pictures in this experiment. Having IRAP pictures far apart is less useful for high-motion sequences and sequences with zoom such as in class D.

Fig. 2. Replacing all IRAP pictures but the first one with DRAP pictures. The anchor for comparison is the compressed CTC sequences.

In a second experiment the bitrate savings of using DRAP pictures in combination with IRAP pictures are investigated. These results, shown in Figure 3, indicate the potential for the fast channel switching use cases.

Fig. 3.  Bitrate differences by using periodic DRAP pictures in combination with periodic IRAP pictures when comparing to using only periodic IRAP pictures.

Experimental data from the second experiment is also used to measure the bitrate impact from increasing the random access point periodicity with and without using DRAP pictures. Figure 4 shows the additional bitrate cost for decreasing the IRAP period compared to using IRAP period 64 and Figure 5 shows the additional bitrate cost for decreasing the random access point period by introducing DRAP pictures at intervals 8, 16 and 32 in a bitstream with IRAP period 64. In a random access operation, the average extra overhead for accessing a single random access point in an IRAP/DRAP configuration compared to an IRAP only configuration were 54.7%, 47.1% and 31.6% for random access point periods of 8, 16 and 32 respectively.

Fig. 4. Additional bitrate cost of decreasing IRAP period compared to IRAP period 64.

Fig. 5. Additional bitrate cost of decreasing random access point period by inserting DRAP pictures at intervals 8, 16 and 32, compared to IRAP period 64.

# Reference

[1]	M. Pettersson, R. Sjöberg, J. Samuelsson, “Dependent Random Access Point Pictures in HEVC”, IEEE International Conference on Image Processing (ICIP), Sept. 2015

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).