| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0496-v2 |
| --- | --- |

| Title: | AHG12: Rectangular tile group address signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Mitra Damghanian Martin Pettersson | Email: | rickard.sjoberg@ericsson.com mitra.damghanian@ericsson.com martin.m.pettersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution noted that support for rectangular tile groups adopted at the 13th JVET meeting in Marrakech includes a loop over tile groups, and in the loop there are two tile index code words for signaling the top left tile index and the bottom right tile index for each tile group as follows:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) { |  |
| if( i > 0) |  |
| top_left_tile_idx[ i ] | u(v) |
| bottom_right_tile_idx[ i ] | u(v) |
| } |  |
| ... |  |
| } |  |

This contribution proposes to remove the top_left_tile_idx[ i ] syntax element and the if( i > 0 ) statement and instead derive the value of top_left_tile_idx[ i ] as equal to the first tile index in raster scan order that is not yet included in any tile group.

In the v2 version of the contribution, a modified semantics text based on off-line discussions at the Geneva meeting was added in a new section 3 as an option 2.

# Introduction and problem statement

In the VVC v6 draft,  when rectangular tile groups are enabled, the signaling of tiles that are included in each tile group is done in the PPS as shown in Table 1. This signaling includes a loop over tile groups, and in the loop, there are two tile index code words for signaling the top left tile index and the bottom right tile index for the tile group. Both code words are fixed length coded using Ceil(Log2(NumTilesInPic)) bits.

Table 1 – PPS elements from JVET-M1001-v6

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| single_tile_per_tile_group_flag | u(1) |
| if(!single_tile_per_tile_group_flag ) |  |
| rect_tile_group_flag | u(1) |
| if( rect_tile_group_flag && !single_tile_per_tile_group_flag ) { |  |
| num_tile_groups_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) { |  |
| if( i > 0) |  |
| top_left_tile_idx[ i ] | u(v) |
| bottom_right_tile_idx[ i ] | u(v) |
| } |  |
| } |  |
| ... |  |
| } |  |

The meeting notes from the 13th JVET meeting in Marrakech states that the tile groups need to be in raster-scan order. If that design choice is also applied to the signaling of the tile groups in the PPS, it can be noted that the top_left_tile_idx[ i ] syntax element can be derived instead of being signaled.

# Proposal

We propose that the top_left_tile_idx[ i ] syntax element is removed. Instead, we propose to derive it from the next tile index in raster scan order that is not yet included in any tile group.

The proposed text changes are specified below. Added text is shown using highlighted red text and removed text is shown as highlighted red strikethrough text.

Table 2 – Proposed PPS change relative JVET-M1001-v6

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| single_tile_per_tile_group_flag | u(1) |
| if(!single_tile_per_tile_group_flag ) |  |
| rect_tile_group_flag | u(1) |
| if( rect_tile_group_flag && !single_tile_per_tile_group_flag ) { |  |
| num_tile_groups_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) { |  |
| if( i > 0) |  |
| top_left_tile_idx[ i ] | u(v) |
| bottom_right_tile_idx[ i ] | u(v) |
| } |  |
| } |  |
| ... |  |
| } |  |

top_left_tile_idx[ i ] specifies the tile index of the tile located at the top-left corner of the i-th tile group. The value of top_left_tile_idx[ i ] shall not be equal to the value of top_left_tile_idx[ j ] for any i not equal to j. When not present, the value of top_left_tile_idx[ i ] is inferred to be equal to i. The length of the top_left_tile_idx[ i ] syntax element is Ceil( Log2( NumTilesInPic ) bits.

bottom_right_tile_idx[ i ] specifies the tile index of the tile located at the bottom-right corner of the i-th tile group. When single_tile_per_tile_group_flag is equal to 1 bottom_right_tile_idx[ i ] is inferred to be equal to top_left_tile_idx[ i ]. When not present, the value of  bottom_right_tile_idx[ i ] is inferred to be equal to i. The length of the bottom_right_tile_idx[ i ] syntax element is Ceil( Log2( NumTilesInPic) ) bits.

It is a requirement of bitstream conformance that any particular tile shall only be included in one tile group.

The variable NumTilesInTileGroup[ i ], which specifies the number of tiles in the i-th tile group, and related variables, are derived as follows:

if( i == 0 )
	for (j = 0; j <= num_tiles_minus1; j++ )
		TileInTileGroupFlag[ j ] = 0
for( j = 0; j <= num_tiles_minus1; j++ )
	if( TileInTileGroupFlag[ j ] == 0 )
		break
TopLeftTileIdx[ i ] = j 
deltaTileIdx = bottom_right_tile_idx[ i ] – top_left_tile_idx[ i ] TopLeftTileIdx[ i ]
NumTileRowsInTileGroupMinus1[ i ] = deltaTileIdx / ( num_tile_columns_minus1 + 1 )
NumTileColumnsInTileGroupMinus1[ i ]  = deltaTileIdx % ( num_tile_columns_minus1 + 1 )
NumTilesInTileGroup[ i ] = ( NumTileRowsInTileGroupMinus1[ i ] + 1 ) *
	  ( NumTileColumnsInTileGroupMinus1[ i ] + 1 )
for (y = 0; y <= NumTileRowsInTileGroupMinus1[ i ]; y++)
	for (x = 0; x <= NumTileColumnsInTileGroupMinus1[ i ]; x++)
		TileInTileGroupFlag[ TopLeftTileIdx [ i ] + y * ( num_tile_columns_minus1 + 1 ) + x ] = 1

It can be noted that it might be beneficial to replace NumTileRowsInTileGroupMinus1 and NumTileColumnsInTileGroupMinus1 by NumTileRowsInTileGroup and NumTileColumnsInTileGroup and change the derivation of them and the associated semantics in the VVC draft, but that would be purely editorial.

# Semantics option 2

Based on off-line discussions at the Geneva meeting, the following semantics is proposed instead of the one above. The new semantics does not have the break statement that was questioned in one of the Geneva meeting BoG sessions on Coded Picture Regions. The yellow highlights below show the difference to the semantics above.

if( i  = =  0 )
	for (j = 0; j <= num_tiles_minus1; j++ )
		TileInTileGroupFlag[ j ] = 0
nextTGIdx = (i  = =  0 ) ? 0 : TopLeftTileIdx[ i−1 ] + NumTileColumnsInTileGroupMinus1[ i−1 ] + 1
while( nextTGIdx <= num_tiles_minus1  &&  TileInTileGroupFlag[ nextTGIdx ] )
	nextTGIdx++
TopLeftTileIdx[ i ] = nextTGIdx
deltaTileIdx = bottom_right_tile_idx[ i ] – top_left_tile_idx[ i ] TopLeftTileIdx[ i ]
NumTileRowsInTileGroupMinus1[ i ] = deltaTileIdx / ( num_tile_columns_minus1 + 1 )
NumTileColumnsInTileGroupMinus1[ i ]  = deltaTileIdx % ( num_tile_columns_minus1 + 1 )
NumTilesInTileGroup[ i ] = ( NumTileRowsInTileGroupMinus1[ i ] + 1 ) *
	  ( NumTileColumnsInTileGroupMinus1[ i ] + 1 )
for (y = 0; y <= NumTileRowsInTileGroupMinus1[ i ]; y++)
	for (x = 0; x <= NumTileColumnsInTileGroupMinus1[ i ]; x++)
		TileInTileGroupFlag[ TopLeftTileIdx [ i ] + y * ( num_tile_columns_minus1 + 1 ) + x ] = 1

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).