| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0497 |
| --- | --- |

| Title: | AHG12: Dependent tiles |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Martin Pettersson Mitra Damghanian | Email: | rickard.sjoberg@ericsson.com martin.m.pettersson@ericsson.com mitra.damghanian@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The current tiles in VVC is seen as independent by the authors since there is no prediction between tiles and CABAC is initialized for each tile such that all tiles can be decoded in parallel. Dependent tiles, according to this proposal, would enable prediction across tiles within a tile group and only initialize CABAC for the first tile in the tile group. It is proposed that CABAC is byte-aligned for each dependent tile but that CABAC states are continuously updated within a tile group.

The contribution proposes that a flag is added for each tile group that specifies whether the tiles within a tile group are dependent or independent. For rectangular tile groups, where currently the tile group structure is signaled in the PPS, it is proposed that the flags are added to the PPS. For raster scan tile groups, it is proposed to add a single flag in the tile group header. The tile groups are always independent according to the proposal, it is only the internal tile boundaries within a tile group that are affected. The scan order of CTUs in a picture would according to the proposal stay the same regardless if tiles are made dependent or not.

It is claimed that the proposal would effectively add support for tiles of different sizes, something that is claimed to be useful for e.g. multi-resolution 360 video streaming. The authors claim that the required VVC text changes are small.

# Introduction and problem statement

A tile design including a segmentation of tiles into VCL NAL units consisting of a tile group header was adopted at the 13th meeting in Marrakech. The design only supports NAL units consisting of an integer number of tiles.

Tiles in its current form in VVC (and HEVC) are restricted by the use of tile rows and tile columns. This means that all tile boundaries are required to span across the entire picture; from left to right or from top to bottom. This limits the flexibility of tiles. For example, the left tile partition in Figure 1 is not supported in VVC.

Figure 1 – An example of a flexible tile partition (left) and the closest tile partition to this flexible tile partition in the current VVC draft (right)

To virtually support such a tile structure using VVC, one may specify the tile partition shown to the right in Figure 1. However, that tile structure contains a lot more tile boundaries which harms compression efficiency and may cause unnecessary tiling artifacts.

VVC allows for enabling or disabling in-loop filtering dependencies for all tile boundaries using loop_filter_across_tiles_enabled_flag and for the tile group boundaries using loop_filter_across_tile_group_boundaries_enabled_flag. So, one may allow in-loop filtering across all the tile boundaries but disallow in-loop filtering across all the tile group boundaries. This can bring the result closer to the flexible tile structure shown in the left of Figure 1, but still at the cost of extra complexity and potentially remaining artifacts.

Figure 2 shows a CMP partitioning example from section 2.5 of JVET-K0300-v1. The JVET-K0300-v1 document reports that the example is included into clauses D.6.3 and D.6.4 of OMAF and adopted in the VR Industry Forum Guidelines. This and other 360 video streaming solutions use segments of different resolutions which in HEVC requires a mix of tiles and slices. As can be seen to the right of Figure 2, the rightmost tile column consisting of small tiles is realized by splitting tiles using HEVC slices.

Since a mix of slices and tiles is not supported in VVC, the example in Figure 2 is not supported.

Figure 2 – Example MCTS-based RWMR viewport-dependent 360° streaming

# Proposal

We propose that a flag is added for each tile group that specifies whether the tiles in a tile group are dependent or independent. Independent tiles work the same as tiles in the current VVC draft. Dependent tiles allow prediction across tiles and CABAC states to be kept across tile boundaries rather than being initialized. For rectangular tile groups, where currently the tile group structure is signaled in the PPS, we propose that the flags are added to the PPS. For raster scan tile groups, we propose to add a flag in the tile group header.

## Proposed PPS syntax and semantics

Table 1 – Proposed PPS syntax shown using highlighted text

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| transform_skip_enabled_flag | u(1) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_tile_spacing_flag | u(1) |
| if( !uniform_tile_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| single_tile_per_tile_group_flag | u(1) |
| if(!single_tile_per_tile_group_flag ) |  |
| rect_tile_group_flag | u(1) |
| if( rect_tile_group_flag && !single_tile_per_tile_group_flag ) { |  |
| num_tile_groups_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) { |  |
| if( i > 0) |  |
| top_left_tile_idx[ i ] | u(v) |
| bottom_right_tile_idx[ i ] | u(v) |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| if( loop_filter_across_tiles_enabled_flag ) |  |
| loop_filter_across_tile_groups_enabled_flag | u(1) |
| } |  |
| if( rect_tile_group_flag ) { |  |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) |  |
| dependent_tiles_in_rect_tile_group_flag[ i ] | u(1) |
| signalled_tile_group_id_flag | u(1) |
| if( signalled_tile_group_id_flag ) { |  |
| signalled_tile_group_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) |  |
| tile_group_id[ i ] | u(v) |
| } |  |
| … |  |
| rbsp_trailing_bits( ) |  |
| } |  |

dependent_tiles_in_rect_tile_group_flag[ i ] equal to 0 specifies that the i-th tile group consists of independent tiles. dependent_tiles_in_rect_tile_group_flag[ i ] equal to 1 specifies that the i-th tile group consists of dependent tiles.

## Proposed tile group header syntax and semantics

Table 1 – Proposed tile group header syntax shown using highlighted text

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_parameter_set_id | ue(v) |
| if( rect_tile_group_flag ¦ ¦ NumTilesInPic > 1 ) |  |
| tile_group_address | u(v) |
| if( !rect_tile_group_flag && !single_tile_per_tile_group_flag ) |  |
| num_tiles_in_tile_group_minus1 | ue(v) |
| if( !rect_tile_group_flag ) |  |
| dependent_tiles_in_tile_group_flag | u(1) |
| tile_group_type | ue(v) |
| ... |  |
| byte_alignment( ) |  |
| } |  |

dependent_tiles_in_tile_group_flag equal to 0 specifies that the tile group consists of independent tiles. dependent_tiles_in_tile_group_flag equal to 1 specifies that the tile group consists of dependent tiles. When not present, the value of dependent_tiles_in_tile_group_flag is inferred to be equal to dependent_tiles_in_rect_tile_group_flag[ tileGroupIdx ].

## Proposed availability process

We propose is to add a condition that the value of dependent_tiles_in_tile_group_flag is equal to 0 for the availability to be false when the current and neighboring blocks belong to different tiles in the same tile group. The following text changes to JVET-M1001-v6 is shown by yellow highlights are proposed:

### 6.4.4 Derivation process for neighbouring block availability

Inputs to this process are:

- the luma location ( xCurr, yCurr ) of the top-left sample of the current block relative to the top-left luma sample of the current picture,

- the luma location ( xNbY, yNbY ) covered by a neighbouring block relative to the top-left luma sample of the current picture.

Output of this process is the availability of the neighbouring block covering the location ( xNbY, yNbY ), denoted as availableN.

The neighbouring block availability availableN is derived as follows:

–	If the neighbouring block is contained in a different tile group than the current block, availableN is set equal to FALSE.

–	Otherwise, if the the neighbouring block is contained in a different tile than the current block and the value of dependent_tiles_in_tile_group_flag for the tile is equal to 0, availableN is set equal to FALSE.

–	Otherwise, [Ed. (CJ): the derivation process of availableN is to be specified.]

## Proposed CABAC behavior

We propose that initialization of CABAC is not done for tiles that are not first in a tile group when dependent_tiles_in_tile_group_flag is equal to 1. Proposed text changes relative M1001-v6 is shown below.

## 9.5	CABAC parsing process for tile group data

### 9.5.1	General

Inputs to this process are a request for a value of a syntax element and values of prior parsed syntax elements.

Output of this process is the value of the syntax element.

The initialization process as specified in clause 9.5.2 is invoked when starting the parsing of the CTU syntax specified in clause 7.3.6.2 and the CTU is either the first CTU in a tile group or the first CTU in a tile for which the value of dependent_tiles_in_tile_group_flag is equal to 0.

…

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).