| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0499 |
| --- | --- |

| Title: | CE4-related: Simplification for merge list derivation in triangular prediction mode on top of test CE4-4.6.b |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Timofey Solovyev, Semih Esenlik, Sergey Ikonin, Jianle Chen | Tel: Email: | +7-925-008-9652 solovyev.timofey@huawei.com semih.esenlik@huawei.com sergey.ikonin@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

# Abstract

This contribution presents modification of triangular merge list construction method tested in core experiment 4-4.6.b. As well as in the core experiment, regular merge list construction process is used as a basic part of triangular merge list construction process. Due to invoking regular merge list construction process with maximum number of merge candidates is equal to five instead of six, a pairwise candidate is added more often that cause the performance improvement in comparison to the method tested in core experiment.

The method shows on average 0.0% BD-rate change for RA configuration and 0.1% luma BD-rate loss for LDB configuration without significant change in encoding and decoding times.

# Introduction

During the 12th JVET meeting in Macau, the triangular prediction mode has been adopted into the VVC specification draft [1]. If triangular mode is selected by a CU, it is split diagonally and each resulting triangular part is predicted using a different motion vector. The predictions generated for each triangular part is restricted to be uni-prediction.

The motion candidates used to predict the triangular parts are selected based on a uni-prediction candidate list. The uni-prediction candidate list consists of five candidates. It is derived from seven neighboring blocks including five spatial neighboring blocks (1 to 5) and two temporal co-located blocks (6 to 7). The motion vectors are collected and put into the uni-prediction candidate list according the following order:

- uni-prediction motion vectors

- L0 motion vector of bi-prediction motion vectors

- L1 motion vector of bi-prediction motion vectors

- averaged motion vectors of the L0 and L1 motion vectors of bi-prediction motion vectors.

If the number of candidates is less than five, zero motion vector(s) is added to the list.

# Proposed method

As well as in core experiment 4-4.6.b [2], the additional uni-prediction merge candidate list dedicated to triangular prediction mode is removed. Instead, regular merge list is used by converting bi-prediction candidates to uni-prediction candidates.

The uni-prediction candidate list is constructed according to the steps of:

- Invoke the regular merge candidate list construction process with maximum number of merge candidates equal to 5.

- Starting from the beginning of the constructed candidate list, the following is performed:

  - If a candidate is uni-predictive, insert it into uni-prediction candidate list.

  - If a candidate is bi-predictive:

    - Insert L0 motion information into uni-prediction candidate list,

    - Insert L1 motion information if it is not identical to the L0 motion information as additional candidate to the uni-prediction candidate list.

Motion information here means motion vector and reference index.

In this method L1 motion information of a bi-prediction candidate is compared with the L0 motion information before insertion into the uni-prediction candidate list. The L0-L1 motion information comparison can be performed in parallel to the other pruning checks of the merge list construction process.

The number of candidates in the uni-prediction candidate list is set as 5.

Step 1 of this method differs from corresponding step of method 2, described in core experiment 4-4.6, in value of parameter “maximum number of merge candidates” that is used by regular merge candidate list construction process. In 4-4.6, value 6 is used if Common Test Conditions [3] case is considered. Candidate construction process is stopped as first five candidates are obtained. In proposed method, value 5 is used as maximum number of merge candidates when regular merge candidate construction process is invoked. Finally in both 4-4.6 and the proposed method only 5 candidates is used as input for the step 2.

Step 2 of proposed method is the same as corresponding step in method 2, described in core experiment 4-4.6.

Using different value of maximum number of merge candidates (5 instead of 6) while invoking regular merge list construction process entails adding the pairwise candidate in a list of first 5 candidates more often which in turn cause the performance improvement in comparison to the method tested in core experiment.

# Complexity analysis

Complexity analysis is performed according to the description of Core Experiment 4 [3]. Summary of complexity analysis is presented in Table 1. In this analysis it is assumed that regular merge list is already constructed, so complexity of deriving triangular merge candidates list based on the regular merge candidates list is analysed.

Table 1. Summary of complexity analysis for the proposed method

| Merge list size | Max number of potential candidates | Max number of candidate comparison | Max number of MV scaling | Max number of temporal candidates | Additional local buffer | Max number of memory access | Others |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 5*2 | 4 | 0 | 1 | 0 | 5 | – |

# Simulation results

The proposed method is evaluated for random-access (RA) and low delay B (LDB) configurations using VTM-4.0 software, according to the common test conditions in JVET-M1010 [3] and the description of Core Experiment 4 [3]. The detailed experimental results are provided in Table 2 – Table 3.

Table 2. Coding performance of the proposed method over VTM-4.0 for RA configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | 0.02% | -0.02% | -0.10% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.04% | -0.10% | -0.16% | 99% | 97% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table 3. Coding performance of the proposed method over VTM-4.0 for LDB configuration

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

# Conclusion

The proposed method is a modification of triangular merge list construction method explored in core experiment 4-4.6.b. Due to invoking regular merge list construction process with different value of maximum number of merge candidates, performance improvement in comparison to the method tested in core experiment is achieved.

Simulation results for the proposed method reportedly show on average 0.0%, 0.1% luma BD-rate loss for RA and LDB configurations respectively.

# References

- R.-L. Liao, C. S. Lim, “CE10.3.1.b: Triangular prediction unit mode”, JVET-L0124, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 12th Meeting: Macao, CN, 3–12 Oct. 2018

- T. Solovyev, S. Esenlik, S. Ikonin, J. Chen, “CE4-4.6: Simplification for merge list derivation in triangular prediction mode”, JVET-M0454, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 14th Meeting: Geneva, CH, 19–27 March 2019

- H. Yang, G. Li, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding,” JVET-M1024, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 13th Meeting: , Marrakech, MA, January, 2019.

- F. Bossen, J. Boyce, K. Sühring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 13th Meeting: , Marrakech, MA, January, 2019

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).