| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0500 |
| --- | --- |

| Title: | Non-CE4: Restrictions on triangular merge list size |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Timofey Solovyev, Semih Esenlik, Sergey Ikonin, Jianle Chen | Tel: Email: | +7-925-008-9652 solovyev.timofey@huawei.com semih.esenlik@huawei.com sergey.ikonin@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

# Abstract

The number of tringular merge candidates in current VTM is static and equal to five. To better support low-complexity encoder use case different methods for controlling maximum number of candidates in triangular merge list are proposed. The first method is to signal this maximum number by a new syntax element in tile group header. The second method is to derive maximum number of triangular merge candidates based on maximum number of regular merge candidates. Different options for this deriving are proposed.

# Introduction

In current VVC maximum number of candidates in regular merge list can be flexibly set using syntax element six_minus_max_num_merge_cand in tile group header. Adding high-level syntax element for controlling regular merge list size was previously proposed to HEVC and was motivated by the need to support low-complexity encoder use case [1].

During the 12th JVET meeting in Macau, the triangular prediction mode has been adopted into the VVC specification draft [2]. This mode introduce new merge list (triangular prediction mode merge list) that always contains five candidates. This cause problems with support of low-complexity encoder use case in VVC. This issue was firstly addressed in contribution JVET-M0286 during the 13th JVET meeting in Marrakech [2]. Current contribution propose more options for controlling maximum number of candidates in triangular merge list.

# Proposed method

To control maximum number of candidates in triangular merge list two methods are proposed.

Method 1

This method is the signaling of maximum number of candidates in triangular merge list by a new syntax element in tile group header. As well as for regular merge list, greater values of this list size look more frequently used. So difference between five and maximum triangular merge list size is signaled. The following change in tile group syntax are proposed:

| tile_group_header( ) { | Descriptor |
| --- | --- |
| … | ue(v) |
| if ( tile_group_type != I ) { |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| tile_group_temporal_mvp_enabled_flag | u(1) |
| if( tile_group_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( tile_group_temporal_mvp_enabled_flag ) { |  |
| if( tile_group_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| } |  |
| if( ( weighted_pred_flag && tile_group_type = = P ) ¦ ¦ ( weighted_bipred_flag && tile_group = = B ) ) |  |
| pred_weight_table( ) |  |
| six_minus_max_num_merge_cand | ue(v) |
| if( sps_affine_enabled_flag ) |  |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| if( sps_triangle_enabled_flag ) |  |
| five_minus_max_num_triangle_merge_cand | ue(v) |
| if( sps_fpel_mmvd_enabled_flag ) |  |
| tile_group_fpel_mmvd_enabled_flag | u(1) |
| } else if ( sps_ibc_enabled_flag ) |  |
| six_minus_max_num_merge_cand | ue(v) |
| … |  |
| } |  |

## Method 2

The second method is the deriving of triangular merge list size based on regular merge list size. This provide some support for low-complexity encoder use case and at the other hand does not require adding new syntax element to the tile group header.

Two options for deriving triangular merge list size from regular merge list size are proposed. The first option is described in Table 1. Triangular prediction mode merge list size equal to zero in this table means that triangular prediction mode is not used and not signaled.

Table 1. The relationship between triangular merge list size and regular merge list size: option 1

| Regular merge list size | Triangular prediction mode merge list size |
| --- | --- |
| 1 | 0 |
| 2 | 2 |
| 3 | 3 |
| 4 | 4 |
| 5 | 5 |
| 6 | 5 |

To avoid the disabling of triangular prediction mode for maximum regular merge list size equal to 1, the second option of relationship between triangular merge list size and regular merge list size is proposed. This option is described in Table 2.

Table 2. The relationship between triangular merge list size and regular merge list size: option 2

| Regular merge list size | Triangular prediction mode merge list size |
| --- | --- |
| 1 | 2 |
| 2 | 2 |
| 3 | 3 |
| 4 | 4 |
| 5 | 5 |
| 6 | 5 |

# Conclusion

In this contribution two methods for controlling maximum number of candidates in triangular merge list are proposed. The first method is to signal this maximum number by a new syntax element in tile group header. The second method is to derive maximum number of triangular merge candidates based on maximum number of regular merge candidates.

Both methods addressed to low-complexity encoder support issue in VVC.

The first method provide more flexible triangular merge list size controlling, but the new syntax element need to be signaled in tile group header. The second method also solve low-complexity encoder support issue and not requires any additional syntax elements.

# References

- M. Zhou “CE13: Test results on maxNumMergeCand signaling and simplification of merge MVP list pruning process”, JCT-VC Document, JCTVC-G091, Geneva, CH, 19-30 November, 2011

- R.-L. Liao, C. S. Lim, “CE10.3.1.b: Triangular prediction unit mode”, JVET-L0124, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 12th Meeting: Macao, CN, 3–12 Oct. 2018

- T. Solovyev, S. Esenlik, S. Ikonin, J. Chen “Non-CE4: Simplifications for triangular prediction mode”, JVET-M0286, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 13th Meeting: , Marrakech, MA, January, 2019

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).