| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0505-v2 |
| --- | --- |

| Title: | Non-CE9: Simplification of parametric motion vector refinement in DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Eiichi Sasaki Tianyang Zhou Tomohiro Ikai | Tel: Email: | +81-50-5213-6277 eiichi.sasaki@sharp.co.jp zhou.tianyang@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

In Decoder-side Motion Vector Refinement (DMVR), a parametric motion vector refinement process is applied to derive a fractional refined motion vector using integer-pel precision SADs. In the current method, the fractional refined motion vector is estimated by the quadratic function approximated by 3 point SADs where division or multiple cascade comparisons is required. In this contribution, we propose a division-free parametric motion vector refinement process. It is reported the BDrate loss is 0.02 % on average in CTC RA condition.

Proposed specification

The change of working draft is following. Proposed change is also basis on the approximate quadratic function.

- 8.5.3.5  Parametric motion vector refinement process

Inputs to this process are:

- a list of sum of absolute differences sadList[ i ] with i = 0..8,

- the delta luma motion vector dMvL0 given in 1/16-luma-sample units.

Output of this process is the modified delta luma motion vector dMvL0.

The variable dX is derived as follows:

- If Abs( sadList[ 3 ] – sadList[ 5 ] ) is less than or equal to Min( sadList[ 3 ], sadList[ 5 ] ) – sadList[ 4 ],dX is set equal to 0.

- Otherwise, dX is set equal to Sign( sadList[ 3 ] – sadList[ 5 ] ) * 3.

The variable dY is derived as follows:

- If Abs( sadList[ 1 ] – sadList[ 7 ] ) is less than or equal to Min( sadList[ 1 ], sadList[ 7 ] ) – sadList[ 4 ],dY is set equal to 0.

- Otherwise, dY is set equal to Sign( sadList[ 1 ] – sadList[ 7 ] ) * 3.

The delta luma motion vector dMvL0 is modified as follows:

dMvL0[ 0 ] += dX 			(8-479)

dMvL0[ 1 ] += dY			(8-480)

Experimental Results

Proposed simplification is implemented on VTM-4.0 and Table 1 shows the results under CTC[2].

Table 1. Simulation results under CTC

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.06% | -0.02% | 100% | 101% |
| Class A2 | 0.05% | 0.10% | 0.05% | 100% | 100% |
| Class B | 0.03% | 0.03% | 0.02% | 100% | 100% |
| Class C | 0.01% | -0.02% | -0.06% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.04% | 0.00% | 100% | 100% |
| Class D | 0.05% | 0.04% | -0.03% | 100% | 100% |
| Class F | -0.04% | 0.03% | 0.00% | 94% | 100% |

Conclusion

A simplified parametric motion vector refinement in DMVR is proposed.

Reference

- S. Sethuraman, “CE9: Results of DMVR related Tests CE9.2.1 and CE9.2.2,” JVET-L0054, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 13th Meeting: Marrakech, MA, 9–18 Jan. 2019.

- F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 13th Meeting: Marrakech, MA, 9–18 Jan. 2019.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).