| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0506 |
| --- | --- |

| Title: | Disabling raster search in integer-pel ME for common test condition |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sang-hyo Park Je-Won Kang | Email: | sanghyo.park@ewha.ac.kr jewonk@ewha.ac.kr |
| Source: | Ewha Womans University |  |  |

_____________________________

- Abstract

This contribution reports the performance change in VTM when raster search is turned off. When turning it off, a 0.15 % increase is observed for RA case in Y BD-rate, but the encoding time is reduced to 95 % for RA, on average, in comparison with VTM 4.0. Since turning off raster search as a default can reduce encoding burden for experiments, this contribution recommends disabling raster search for CTC.

Introduction

Decreasing the encoding complexity of VTM can save time for reporting evaluation experiments. Herein, a simple parameter evaluation was conducted and reported. Particularly, traditional motion estimation (ME) process was evaluated. Since search range has been increased from 64 to 384 for random access (RA) main configuration, it is obvious that ME time can be much increased. We assumed that raster search is the most complex part in ME and thereby, evaluated the performance of VTM when raster search was turned off.

Proposed option

There are several cases that VTM can choose to search integer-pixel motion vector. The default option in VTM has been diamond search (i.e., MESEARCH_DIAMOND in source code) which uses diamond search pattern. In addition to the diamond search, VTM default option is conducting raster search when the result of diamond search is quite far from the starting point. Raster search “tests blocks along a sparse grid within the defined search window, testing blocks at every five luma samples horizontally and vertically” as written in JCTVC-AB1002 [1].

Although the search range was increased from 64 to 384 for RA case, the number of search point for diamond search may not be increased in the certain situation. It is possible because for the first diamond search, there exist stop criterion that determine the best search point and skip remaining points upon the given small distance. However, there is no such stop criterion on raster search when it is enabled and the best distance of the first diamond search is longer than five. Furthermore, it has not been reported that how much raster search impacts on VTM in view of coding efficiency or on encoding complexity.

For HM, it was recently reported that turning off raster search can reduce encoding time to 84% with a 0.3% coding loss [2]. When considering the comparison with HM, we assume that turning off raster search can reduce encoding burden of VTM as well as HM for experiments in standardization work. Thus, we propose CTC for VTM turn off raster search by simply disabling a parameter at VTM encoder. In addition, it would be useful to let the configuration file have an option to turn on/off such ME tool.

Experiments

Tests were conducted based on common test conditions (CTCs) in JVET-M1010 [3], but class A and class F sequences were not tested in this contribution. As an initial report for the performance check of raster search in VTM 4.0, one configuration was selected: RA configuration. Test material in this experiments is Class C, Class D and a few of Class B sequences. Overall results for RA is shown in Table 1. It was reported on average that a 0.15% increase was occurred in BD-rate of Y and a 5% decrease was occurred in encoding time, when raster search was turned off. Detailed result can be found in the attached excel sheet.

Table 1. BD-rates of raster search off against VTM 4.0 in RA case

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #DIV/0! | #DIV/0! | #DIV/0! | #NUM! | #NUM! |
| Class A2 | #DIV/0! | #DIV/0! | #DIV/0! | #NUM! | #NUM! |
| Class B | 0.09% | -0.01% | 0.37% | 93% | #DIV/0! |
| Class C | 0.16% | 0.31% | 0.24% | 96% | #DIV/0! |
| Class E |  |  |  |  |  |
| Overall | 0.15% | 0.24% | 0.27% | 95% | #DIV/0! |
| Class D | 0.07% | 0.20% | 0.10% | 98% | #DIV/0! |
| Class F (optional) | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

Conclusion

It is reported that on average, a 0.15% increase was occurred in BD-rate of Y and a 4% decrease was occurred in encoding time, when raster search was turned off. In addition, as reported in JCTVC-AF0022, disabling raster search can also reduce the encoding time of HM to 84% with a little coding loss. In conclusion, disabling raster search as a default for CTC can reduce the encoding time of both VTM and HM. Thus, we recommend that raster search be turned off as a default for VTM and HM to relieve the encoding burden of lots of experiments for comparison.

References

[1] “High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Encoder Description Update 9,” JCTVC-AB1002, Torino, IT, 15-21 July, 2017.

[2] S. Park, “Additional report on the performance of raster search with regard to JCTVC-AE0029,” JCTVC-AF0022, Ljubljana, SI, 12-18 July, 2018.

[3] “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, Marrakech, MA, 9-18 Jan. 2019.

Patent rights declaration(s)

Ewha Womans University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).