| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0507 |
| --- | --- |

| Title: | CE9-related: Early termination of BDOF with DMVR cost |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenji Kondo, Masaru Ikeda, Teruhiko Suzuki 2-10-1 Osaki, Shinagawa-ku, Tokyo Japan | Tel: Email: | +81-50-3750-2744 kenji.kondo@sony.com, Masaru.Ikeda@sony.com, teruhiko.s@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

In this contribution, an early termination technique for BDOF is introduced that a DMVR cost is used to decide early exit. In current BDOF design, the early termination using SAD between references L0 and L1 is already equipped. On the other hand, a SAD value as same as BDOF is calculated in the DMVR process to refine motion vectors. The introduced technique in this contribution re-uses DMVR cost for an early termination in BDOF process. If we re-use the SAD value, our simulation results show reducing decoding run time and almost no drop in BD performance i.e. 0.02%, 0.01% and -0.02% BD-rate for Y, U and V component, 100% and 99% for encoding and decoding time in Random Access (RA) configuration.

Introduction

As shown in Figure 1, DMVR and BDOF process are processed if those processes are enabled in the condition check in the current VTM-4.0 design. DMVR refines motion vectors (MVs), and then BDOF uses the MVs to predict current prediction block. In BDOF process, a sum of difference (SAD) between references L0 and L1 is calculated to be used an early termination.

Figure 1: Current VTM-4.0 design

In a motion estimation of DMVR process, SAD is calculated as a cost to find refinement MVs. In this cascade DMVR and BDOF, SAD is calculated twice. In this contribution, it’s proposed that BDOF re-uses the SAD of DMVR to reduce computational complexity.

The proposed frame work is shown in Figure 2. The DMVR outputs not only MVs also SAD value. In BDOF, if the SAD value of DMVR was gotten, it can skip to compute SAD, and use the SAD value from DMVR to decide the early termination.

Figure 2: CU boundary and early termination region

However, a SAD value is used for also 4x4 sub-block level early termination in BDOF process as shown in in Figure 3. In current design of BDOF, a usual bi-prediction and a prediction with optical flow technique are selected. If a SAD value for 4x4 is less than the threshold, the usual bi-prediction is used to reduce complexity. But, it means SAD value for 4x4 is necessary.

Figure 3: 4x4 sub-block early termination in BDOF process

In this proposal, 4x4 level early termination is not used to avoid the SAD calculation in BDOF process. The frame work is shown in Figure 4.

Figure 4: Proposed method for 4x4 early termination

Experiment

To check the performance, the introduced method is implemented into VTM-4.0. And the test is carried out. JVET common test condition [1] is followed. For the simulations, same platform machines as below in PC cluster were used.

- Encode

  - OS: CentOS 6.10

  - Compiler: g++ (GCC) 6.3.1

  - CPU: Intel Core i7-7700K 4.2GHz

    - SIMD: SSE42.

  - Memory: 32 GB

- Decode

  - OS: CentOS 6.10

  - Compiler: g++ (GCC) 6.3.1

  - CPU: Intel Core i5-2500K 3.3 GHz

    - SIMD: SSE42

  - Memory: 24 GB

For a runtime measurement, the user time in the output log was used. For a random-access case and an all intra case, a parallel encoding is used. To measure encoding time in parallel encoding, the total of all user time for each frame were calculated.

Test variation

This contribution proposes to reuse DMVR cost of DMVR for an early termination in BDOF. However, if DMVR cost was not available, there are two schemes about the early termination in BDOF. First one is to use current scheme in VTM-4.0. The other one is to always enable BDOF without an early termination. In other words, the early termination in BDOF is removed for 1st and 2nd early termination.

In also a contribution [2], it proposes to use DMVR cost for early termination of BDOF. Conceptually, the technique is the same as this contribution. In [2], also above two schemes are tested to check the impact.

In this contribution, two tests are prepared as below to investigate above two schemes.

SAD calculation is needed when DMVR cost was not available (e.g. DMVR was disabled),

The test variation is prepared as below.

- Test-1 (Reuse SAD cost of DMVR)

  - If DMVR was disabled, the early termination of current design in VTM-4.0 is used.

- Test-2 (Additional test on Test-1, Remove SAD calculation in BDOF)

  - If DMVR was enabled, the early termination is checked using SAD cost of DMVR.

  - If DMVR was disabled, BDOF is always processed without early termination.

The summary of test results

Test results are shown in Table 1 and Table 2 for Test-1 and Test-2.

Table 1 Test results for Test-1

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | -0.02% | 100% | 98% |
| Class A2 | 0.04% | 0.01% | 0.01% | 100% | 98% |
| Class B | 0.02% | 0.01% | -0.02% | 100% | 99% |
| Class C | 0.02% | 0.00% | -0.04% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.00% | -0.02% | 100% | 99% |
| Class D | 0.02% | 0.06% | -0.03% | 100% | 99% |
| Class F | 0.02% | 0.02% | 0.01% | 99% | 98% |

Table 2: Test results for Test-2

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.08% | 0.01% | 100% | 98% |
| Class A2 | 0.02% | 0.03% | 0.05% | 101% | 98% |
| Class B | 0.01% | 0.04% | -0.02% | 101% | 99% |
| Class C | 0.00% | 0.02% | -0.03% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.04% | 0.00% | 101% | 99% |
| Class D | 0.02% | -0.03% | -0.08% | 100% | 99% |
| Class F | 0.01% | 0.02% | 0.00% | 101% | 98% |

As shown in test results, the impact of coding efficiency is negligible. For the decoder run time, is slightly faster than anchor. The decoding run time of Test-2 is almost the same as Test-1 although the early termination of current design was not used. Probably, in a lot of CU, DMVR is enabled when BDOF was enabled. The condition to apply DMVR is similar to BDOF. Consequently, it’s expected that the decoding time did not increase compared with Test-1.

Recommendation

Based on the experimental results, it is recommended to study proposed technique in core experiment (CE).

References

[1]	F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR”, Document: JVET-M0449

[2] T. Chujoh, T. Ikai, “CE9-related: Early termination for BDOF”, JVET 14th Meeting: Geneva, CH, March 2019, JVET-N0148

Draft Specification Changes

Decoder side motion vector refinement process

General

Inputs to this process are:

- a luma location ( xSb, ySb ) specifying the top-left sample of the current coding subblock relative to the top-left luma sample of the current picture,

- a variable sbWidth specifying the width of the current coding subblock in luma samples,

- a variable sbHeight specifying the height of the current coding subblock in luma samples,

- the luma motion vectors in 1/16 fractional-sample accuracy mvL0 and mvL1,

- the selected luma reference picture sample arrays refPicL0L and refPicL1L.

Outputs of this process are:

- delta luma motion vectors dMvL0 and dMvL1.

- a variable best SAD cost bestSadInDmvr.

The variable subPelFlag is set to 0. and the variables srRange, offsetH0, offsetH1, offsetV0, and offsetV1 are all set equal to 2.

Both components of the delta luma motion vectors dMvL0 and dMvL1 are set equal to zero and modified as follows:

- For each X being 0 or 1, the ( sbWidth + 2 * srRange ) x ( sbHeight + 2 * srRange ) array predSamplesLXL of prediction luma sample values is derived by invoking the fractional sample bilinear interpolation process specified in 8.5.3.2.1 with the luma location ( xSb, ySb ), the prediction block width set equal to ( sbWidth + 2 * srRange ), the prediction block height set equal to ( sbHeight + 2 * srRange ), the reference picture sample array refPicLXL, the motion vector mvLX and the refinement search range srRange as inputs.

- The list sadList[ i ] with i = 0..8 is derived by invoking the sum of absolute differences calculation process specified in 8.5.3.3 with sbWidth, sbHeight, offsetH0, offsetH1, offsetV0, offsetV1, predSamplesL0L and predSamplesL1L as inputs.

- When sadList[ 4 ] is greater than or equal to sbHeight * sbWidth, the following applies:

- The variable bestIdx is derived by invoking the array entry selection process specified in clause 8.5.3.4 with the list sadList[ i ] with i = 0..8 as input.

- If bestIdx is equal to 4, subPelFlag is set equal to 1.

- Otherwise, the following applies:

dX = bestIdx % 3 − 1		(8-436)

dY = bestIdx / 3 − 1		(8-437)

dMvL0[ 0 ] += 16 * dX		(8-438)

dMvL0[ 1 ] += 16 * dY		(8-439)

offsetH0 += dX		(8-440)

offsetV0 += dY		(8-441)

offsetH1 −= dX		(8-442)

offsetV1 −= dY		(8-443)

- The list sadList[ i ] with i = 0..8 is modifed by invoking the sum of absolute differences calculation process specified in 8.5.3.3 with sbWidth, sbHeight, offsetH0, offsetH1, offsetV0, offsetV1, predSamplesL0L and predSamplesL1L as inputs.

- The variable bestIdx is modified by invoking the array entry selection process specified in clause 8.5.3.4 with the list sadList[ i ] with i = 0..8 as input.

- If bestIdx is equal to 4, subPelFlag is set equal to 1

- Otherwise (bestIdx is not equal to 4), the following applies:

dMvL0[ 0 ] += 16 * ( bestIdx % 3 − 1 )		(8-444)

dMvL0[ 1 ] += 16 * ( bestIdx / 3 − 1 )		(8-445)

- When subPelFlag is equal to 1, the parametric motion vector refinement process specified in clause 8.5.3.5 is invoked with the list sadList[ i ] with i = 0..8, and the delta motion vector dMvL0 as inputs and the modified dMvL0 as output.

- The delta motion vector dMvL1 is derived as follows:

dMvL1[ 0 ] = −dMvL0[ 0 ]		(8-446)

dMvL1[ 1 ] = −dMvL0[ 1 ]		(8-447)

- A variable bestSadInDmvr is set to sadList[bestIdx]

Decoding process for inter blocks

General

This process is invoked when decoding a coding unit coded in inter prediction mode.

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- variables numSbX and numSbY specifying the number of luma coding subblocks in horizontal and vertical direction,

- the motion vectors mvL0[ xSbIdx ][ ySbIdx ] and mvL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0 .. numSbX − 1, and ySbIdx = 0 .. numSbY − 1,

- the refined motion vectors refMvL0[ xSbIdx ][ ySbIdx ] and refMvL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0 .. numSbX − 1, and ySbIdx = 0 .. numSbY − 1,

- the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flags predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0 .. numSbX − 1, and ySbIdx = 0 .. numSbY − 1,

- the bi-prediction weight index gbiIdx,

- a variable cIdx specifying the colour component index of the current block.

- a variable bestSadInDmvr specifying the best SAD cost which is found in clause 8.5.3.1

Outputs of this process are:

- an array predSamples of prediction samples.

Let predSamplesL0L, predSamplesL1L and predSamplesIntraL be (cbWidth)x(cbHeight) arrays of predicted luma sample values and, predSamplesL0Cb, predSamplesL1Cb, predSamplesL0Cr and predSamplesL1Cr, predSamplesIntraCb, and predSamplesIntraCr be (cbWidth / 2)x(cbHeight / 2) arrays of predicted chroma sample values.

The width and the height of the current coding sublock subCbWidth, subCbHeight in luma samples are derived as follows:

sbWidth  =  cbWidth / numSbX		(8-793)

sbHeight  =  cbHeight / numSbY		(8-794)

For each coding subblock at subblock index ( xSbIdx, ySbIdx ) with xSbIdx = 0 .. numSbX − 1, and ySbIdx = 0 .. numSbY − 1, the following applies:

- The luma location ( xSb, ySb ) specifying the top-left sample of the current coding subblock relative to the top-left luma sample of the current picture is derived as follows:

( xSb, ySb )  =  ( xCb + xSbIdx * sbWidth, yCb + ySbIdx * sbHeight )	(8-795)

- The variable currPic specifies the current picture and the variable bdofFlag is derived as follows:

- If all of the following conditions are true, bdofFlag is set equal to TRUE.

- sps_bdof_enabled_flag is equal to 1.

- predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] are both equal to 1.

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] ) * DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] ) is less than 0.

- MotionModelIdc[ xCb ][ yCb ] is equal to 0.

- merge_subblock_flag[ xCb ][ yCb ] is equal to 0.

- GbiIdx[ xCb ][ yCb ] is equal to 0.

- cIdx is equal to 0.

- Otherwise, bdofFlag is set equal to FALSE.

- For X being each of 0 and 1, when predFlagLX[ xSbIdx ][ ySbIdx ] is equal to 1, the following applies:

  - The reference picture consisting of an ordered two-dimensional array refPicLXL of luma samples and two ordered two-dimensional arrays refPicLXCb and refPicLXCr of chroma samples is derived by invoking the process specified in clause 8.5.7.2 with X and refIdxLX as inputs.

  - The motion vector offset mvOffset is set equal to refMvLX[ xSbIdx ][ xSbIdx ] − mvLX[ xSbIdx ][ ySbIdx ].

  - When one or more of the following conditions is true, mvOffset[ 0 ] is set equal to 0:

    - xSb is not equal to xCb and mvOffset[ 0 ] is less than 0

    - ( xSb + sbWidth ) is not equal to ( xCb + cbWidth) and mvOffset[ 0 ] is greater than 0

  - When one or more of the following conditions is true, mvOffset[ 1 ] is set equal to 0:

    - ySb is not equal to yCb and mvOffset[ 1 ] is less than 0

    - ( ySb + sbHeight ) is not equal to ( yCb + cbHeight ) and mvOffset[ 1 ] is greater than 0

  - If cIdx is equal to 0, the following applies:

    - The array predSamplesLXL  is derived by invoking the fractional sample interpolation process specified in clause 8.5.7.3 with the luma location ( xCb, yCb ), the coding subblock width sbWidth, the coding subblock height sbHeight in luma samples, the luma motion vector offset mvOffset, the refined luma motion vector refMvLX[ xSb ][ xSb ], the reference array refPicLXL, bdofFlag, and cIdx as inputs.

  - Otherwise if cIdx is equal to 1, the following applies:

    - The array predSamplesLXCb is derived by invoking the fractional sample interpolation process specified in clause 8.5.7.3 with the luma location ( xCb, yCb ), the coding subblock width sbWidth / 2, the coding subblock height sbHeight / 2, the chroma motion vector offset mvOffset, the refined chroma motion vector refMvLX[ xSb ][ xSb ], the reference array refPicLXCb, bdofFlag, and cIdx as inputs.

  - Otherwise (cIdx is equal to 2), the following applies:

    - The array predSamplesLXCr is derived by invoking the fractional sample interpolation process specified in clause 8.5.7.3 with the luma location ( xCb, yCb ), the coding subblock width sbWidth / 2, the coding subblock height sbHeight / 2, the chroma motion vector offset mvOffset, the refined chroma motion vector refMvLX[ xSb ][ xSb ], the reference array refPicLXCr, bdofFlag, and cIdx as inputs.

- If bdofFlag is equal to TRUE, the following applies:

  - If dmvrFlag is equal to FALSE, the following applies:

- The variable shift is set equal to Max( 2, 14 − BitDepthY ).

- The variables cuDiffThres, bdofBlkDiffThres, and cuSumDiff are derived as follows:

cuDiffThres  =  ( 1 << ( BitDepthY − 8 + shift ) ) * cbWidth*cbHeight 	(8-796)

bdofBlkDiffThres  =  1 << ( BitDepthY − 3 + shift )	(8-797)

cuSumDiff =  0		(8-798)

- For xIdx = 0..(sbWidth >> 2 ) − 1 and yIdx = 0..( sbHeight >> 2 ) − 1, the variables bdofBlkSumDiff and the bidirectional optical flow utilization flag bdofUtilizationFlag[ xIdx ][ yIdx ] are derived as follows:

bdofBlkSumDiff = i=03j=03Abs( predSamplesL0L[ ( xIdx << 2 ) + 1 + i ][ ( yIdx << 2 ) + 1 + j ] − 	(8-799)
		predSamplesL1L[ ( xIdx << 2 ) + 1 + i ][ ( yIdx << 2 ) + 1 + j] )

bdofUtilizationFlag[ xIdx ][ yIdx ] = 	bdofBlkSumDiff >= bdofBlkDiffThres	 (8-800)

cuSumDiff += bdofBlkSumDiff		(8-801)

- When cuSumDiff is less than cuDiffThres, bdofFlag is set equal to FALSE.

  - Otherwise, the following applies:

    - If bestSadInDmvr is equal or larger than (cuDiffThres<<1),  the following applies:

      - For xIdx = 0..(sbWidth >> 2 ) – 1 and yIdx = 0..( sbHeight >> 2 ) − 1, bdofUtilizationFlag[ xIdx ][ yIdx ] arederived as follows:

bdofUtilizationFlag[ xIdx ][ yIdx ] = TRUE     (8-802)

    - Otherwise, bdofFlag is set equal to FALSE

- The array predSamples of prediction samples is derived as follows:

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).