| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0508 |
| --- | --- |

| Title: | CE10-related: A combination of JVET-N0302 (CE10.1.1) and JVET-N0327 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luong Pham Van, Geert Van der Auwera, Adarsh K. Ramasubramonian, Vadim Seregin, Marta Karczewicz (Qualcomm) Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Xianglin Wang (Kwai) | Tel: Email: | 1-858-209-3851 {lphamvan, geertv, aramasub, vseregin, martak }@qti.qualcomm.com {xiaoyuxiu, yiwenchen, tsung-chuanma, xianglinwang }@kwai.com |
| Source: | Qualcomm Inc. and Kwai Inc. |  |  |

_____________________________

# Abstract

In this contribution reports the results of the combination test of CE10-1.1 and JVET-N0327 on simplification for the combined inter and intra prediction (CIIP). In CE10-1.1, the number of intra modes allowed in CIIP is reduced from 4 to only 1, i.e., planar mode. In addition, an adaptive position-independent weight scheme based on the number of intra-coded neighbouring blocks is used. Test 1 of JVET-N0327 proposes to exclude decoder-side motion vector derivation (DMVR) and bi-directional optical flow (BDOF) when generating the inter prediction samples in the CIIP mode. Test 2 of JVET-N0327 proposes to restrict the inter prediction of the CIIP mode to always be uni-prediction when generating the inter prediction samples. The combination tests reportedly lead to {Y, U, V} BD-rate variations compared to VTM4.0 as follows:

- CE10-1.1+Test1: {-0.12%, -0.10%, -0.13%} and {-0.08%, 0.06%%, -0.19%} in RA and LDB, respectively.

- CE10-1.1+Test2: {-0.04% %, -0.11%, -0.13%} and {-0.06%, 0.09%, -0.22%} in RA and LDB, respectively.

# Introduction

In this CE test (CE10-1.1), the CIIP mode is simplified by reducing the number of intra modes allowed from 4 to only 1, i.e., planar mode, and the CIIP MPM list construction is therefore also removed. The weight for intra and inter predicted samples are adaptively selected based on the number of neighbouring intra-coded blocks (Figure 1). The (wIntra, wInter) weights are adaptively set as follows. If both top and left neighbours are intra-coded, (wIntra, wInter) is set equal to (3, 1). Otherwise, if one of these blocks is intra-coded, these weights are identical, i.e., (2, 2), else the weights are set equal to (1, 3).

Figure 1 The position of the top and left neighbouring blocks that are checked to decide the weights for intra and inter predicted samples in blending.

For better decoding throughput, JVET-N0327 proposes two methods to reduce the computational complexity of the CIIP mode, as described as follows:

Test 1:

For the CIIP design in the VVC working draft, decoder-side motion vector derivation (DMVR) and bidirectional optical flow (BDOF) are always enabled to generate the inter prediction samples when the current CIIP CU is bi-predicted. Given the additional computations resulting from the DMVR and BDOF, it is proposed in the first method to always disable those two tools for the inter prediction of the CIIP mode.

Test 2:

In the second method, it is proposed to restrict the inter prediction that is used to generate the combined prediction for the CIIP to always be uni-directional. To convert bi-prediction to uni-prediction, the proposed method always selects the uni-prediction from list L0 when the current CU is bi-predicted.

# Proposal

This proposal reports the experimental results of the combinations of CE10-1.1 and JVET-N0237.

# Simulation results

The proposed technique has been implemented on top of VTM4.0. The experimental results under the RA and LDB configuration are summarized in Table 1 and Table 2 for the combinations tests.

The combination tests reportedly lead to {Y, U, V} BD-rate variations compared to VTM4.0 as follows:

CE10-1.1+Test1: {-0.12%, -0.10%, -0.13%} and {-0.08%, 0.06%%, -0.19%%} in RA and LDB, respectively.

CE10-1.1+Test2: {-0.04% %, -0.11%, -0.13%} and {-0.06%, 0.09%, -0.22%} in RA and LDB, respectively.

Table 1 CE10-1.1 + Test1 (JVET-N0327)

Table 2 CE10-1.1 + Test2 (JVET-N0327)

# Conclusions

This proposal reports the experimental results of the combinations of CE10-1.1 and JVET-N0237. The combination tests reportedly lead to {Y, U, V} BD-rate variations compared to VTM4.0 as follows: CE10-1.1+Test1: {-0.12%, -0.10%, -0.13%} and {-0.08%, 0.06%%, -0.19%%} in RA and LDB, respectively and CE10-1.1+Test2: {-0.04% %, -0.11%, -0.13%} and {-0.06%, 0.09%, -0.22%} in RA and LDB, respectively.

# References

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 4)”, JVET-M1001, Jan. 2019, Marrakech, Morocco.

VTM-4.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, Jan. 2019, Marrakech, Morocco.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).