| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0509-v2 |
| --- | --- |

| Title: | CE6-Related: NSST with 8 Coefficients Computation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) Karam Naser Gagan Rath Philipe de Lagrange | Tel: Email: | karam.naser@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

NSST is a powerful transform coding tool that is reported to provide a significant coding gain. Recently, a simplified version of NSST was proposed in JVET-N0193 to limit the overall complexity by reducing the number of transform sets, as well as reducing the number of computed NSST coefficients. The solution is based on computing of the first N secondary transform coefficients and forcing the rest to be zero, where  N depends on the considered block dimension. Despite this simplification, the design is reported still complex in terms of memory and computational power. This contribution proposes further simplification to this design by fixing N value to 8. It is reported that the present method halves both memory and number of operations for computing NSST.

# Introduction

NSST is a secondary transform applied after the primary transform in order to provide a more compact representation of the coefficients for better coding efficiency. The recent design, called reduced secondary transform (RST) [1], considers applying NSST to the top-left area of the primary transform coefficient and retain the first N coefficients and set the rest to zero. The primary coefficients outside this area remains un-altered.

Depending on the primary transform block dimensions, either 16x16 RST transform matrix or 64x16 is used. For some dimensions, RST is applied twice to the first two 4x4 primary coefficients area. It should be mentioned that there are several proposed configurations of RST, in which the number of transform matrices and the computed number of coefficients differ.

In [1], a configuration that results in “worst-case” multiplication per coefficient of 8 is considered. The details about the NSST design in this configuration is provided in the following table:

| Dimension | RST Kernel dimension | Num RST | Retained Coefficients |
| --- | --- | --- | --- |
| 4x4 | 16x16 | 1 | 8 |
| 4x8 or 8x4 | 16x16 | 1 | 16 |
| 4xN or Nx4 (N>8) | 16x16 | 2 | 16 |
| 8x8 | 64x16 | 1 | 8 |
| else | 64x64 | 1 | 16 |

Table 1. Implementation details of simplified RST.

# Proposed Method

In current contribution, a method is proposed to further reduce the overall complexity associated with RST. This is achieved by fixing the number of retained coefficient to 8 regardless of the transform block size, or for only large block sizes. By doing so, the memory required for storing the RST matrices are halved.

Specifically, to compute N secondary coefficients with a transform size of MxM, a transform matrix of size NxM2 is required. In RST, the maximum number of computed secondary coefficient is 16, the 16xM2 matrices are stored, where M is 4 and 8. In the current proposed method, the matrices are reduced to the dimension of 8xM2. It is also worth mentioning that there 4 RST sets. Depending on the configuration, either one transform per set or two are used. For 8-bit representation, the overall memory for one transform matrix per set is 4*(16*64+16*4) = 4352 Bytes. This is reduced to 2176 bytes in the current proposed method.

Additionally, the computational complexity is also halved. For computing N RST coefficients, NxM2 multiplications and Nx(M2 – 1) additions are required. In general, the proposed method retains half of the coefficients, and both the multiplications and additions are halved.

In addition, in the proposed method, the coding of NSST flag coding is simplified. Specifically, the RST algorithm in JVET-N0193 checks the value of the secondary coefficients outside the retained region. If any of these coefficients is not zero, NSST flag is not coded but is inferred to be zero. This method requires that the decoder performs additional step of coefficients scanning. In the current contribution, it is proposed that this step is skipped to reduce the decoder complexity. Therefore, the NSST flag is always encoded.

In summary, the proposal consists of two simplifications of the RST design from JVET-N0193:

- fixing the number of retained coefficient to 8 regardless of the transform block size (test1);

- fixing the number of retained coefficient to 8 for transform block size greater or equal to 8x8 (test2);

# Experimental Results

The proposed method was implemented on top of [1] software, with the following settings:

#define CE6_3_1A                                          0

#define CE6_3_1B                                          0

#define CE6_3_1C                                          0

#define CE6_3_1D                                          0

#define CE6_3_1E                                          1

The following macro was disabled:

#define WORST_CASE_HANDLING_OFF                           0

That is, Test E (CE6-3.1) is considered. In this test, a single transform matrix per set is used. With VTM4.0 as an anchor, the common testing conditions [2] were ran and the following results are obtained (also reported in attached excel file).

# Test1: 8 Coefficients Computation Regardless the Block Size

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.73% | 0.31% | 0.66% | 101% | 99% |
| Class A2 | -0.59% ! | 0.93% | 1.57% ! | 102% | 100% |
| Class B | -0.21% | 0.75% | 0.07% | 100% | 101% |
| Class C | -0.43% | 0.32% | 0.02% | 99% | 101% |
| Class E | -0.41% | 0.11% | -0.85% | 94% | 101% |
| Overall | -0.44% | 0.50% | 0.25% | 99% | 101% |
| Class D | -0.17% | 0.53% | 0.07% | 100% | 101% |
| Class F | -0.58% | 0.42% | 0.19% | 100% | 101% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.39% | -0.75% | -0.78% | 106% | 100% |
| Class A2 | -0.20% | -0.72% | -0.29% | 104% | 100% |
| Class B | -0.10% | -0.73% | -1.50% | 105% | 101% |
| Class C | -0.10% | -0.25% | -0.74% | 104% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.18% | -0.60% | -0.91% | 105% | 100% |
| Class D | 0.00% | -0.29% | -0.83% | 104% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.13% | -0.05% | -0.29% | 104% | 2% |
| Class E | 0.02% | 1.31% | 0.70% | 102% | 43% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.11% | -0.17% | -0.79% | 104% | 6% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

The results show an average gain of xx without any increase in the encoder and decoder time. The method requires solely extra 2 Kbyte to be implemented compared to VTM4.0, with a small added computational power.

## Test1: 8 Coefficients Computation for Block Size > 8x8

## With the same conditions as in Test1, Test 2 applies the method only for block sizes greater than 8x8. In this way, the NSST of length 64 will always use 8x64 length matrix.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.76% | 0.36% | 0.73% | 102% | 100% |
| Class A2 | -0.64% | 0.93% | 1.46% | 103% | 102% |
| Class B | -0.29% | 0.64% | -0.02% | 101% | 102% |
| Class C | -0.64% | 0.23% | -0.15% | 102% | 102% |
| Class E | -0.62% | 0.11% | -0.76% | 95% | 101% |
| Overall | -0.56% | 0.46% | 0.20% | 101% | 102% |
| Class D | -0.42% | 0.38% | -0.20% | 102% | 103% |
| Class F | -0.74% | 0.25% | 0.03% | 101% | 103% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.44% | -0.81% | -0.84% | 106% | 100% |
| Class A2 | -0.22% | -0.77% | -0.38% | 103% | 100% |
| Class B | -0.14% | -0.71% | -1.63% | 103% | 99% |
| Class C | -0.20% | -0.25% | -0.62% | 98% | 85% |
| Class E |  |  |  |  |  |
| Overall | -0.23% | -0.62% | -0.95% | 102% | 95% |
| Class D | -0.12% | -0.74% | -1.07% | 97% | 88% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.07% | 0.01% | -0.38% | 104% | 102% |
| Class E | 0.04% | 0.89% | -0.97% | 102% | 102% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.05% | -0.22% | -0.73% | 104% | 102% |
| Class F | -0.32% | -0.32% | -0.58% | 104% | 101% |

# Comparison with the different configurations of RST

In [1], 5 configurations of RST are studied.

The following table provides the details about the memory requirement for each configuration, including the configuration rom the current proposal

| Test | No. Tr per Set | Total Memory (Byte) |
| --- | --- | --- |
| CE6-3.1A | 2 | 4*2*(16*16+16*64)= 10240 |
| CE6-3.1B | 2 | 4*2*(16*16+16*64)= 10240 |
| CE6-3.1C | 2 | 4*2*(16*16+16*64)= 10240 |
| CE6-3.1D | 2 | 4*2*(16*16+16*48)= 8192 |
| CE6-3.1E | 2 | 4*1*(16*16+16*64)= 5120 |
| Current proposal (Test1) | 1 | 4*(16*8+8*64)= 2560 |
| Current proposal (Test2) | 1 | 4*(16*16+8*64)= 3072 |

Table 2. Total memory requirement for RST in comparison with the proposed method

Clearly, the memory is reduced from 10 KBytes (CE6-3.1A) to 2.5 Kbytes in the proposed method (Tes1) and 3 Kbytes (Test2). It is worth mentioning that the total memory required for storing the MTS coefficients is equal 4*4+8*8+16*16+32*32 = 2720 Bytes (2.7KBytes). Therefore, the proposed method restricts the memory of NSST to be equivalent to MTS.

The number of multiplications is also compared in the following table.

| Block size | 4x4 | 4x8 or 8x4 | 4xN or Nx4 (N>8) | 8x8 | Other sizes |
| --- | --- | --- | --- | --- | --- |
| CE6-3.1A | 16*16=256 | 2*16*16=512 | 2*16*16=512 | 16*64=1024 | 16*64=1024 |
| CE6-3.1B | 8*16=128 | 16*16=128 | 2*16*16=512 | 8*64=512 | 16*64=1024 |
| CE6-3.1C | disabled | 16*16=128 | 2*16*16=512 | 8*64=512 | 16*64=1024 |
| CE6-3.1D | 8*16=128 | 16*16=128 | 2*16*16=512 | 8*48=384 | 16*48=768 |
| CE6-3.1E | 16*16=256 | 2*16*16=512 | 2*16*16=512 | 16*64=1024 | 16*64=1024 |
| Current proposal (Test1) | 8*16=128 | 8*16=128 | 2*8*16=256 | 8*64=512 | 8*64=512 |
| Current proposal (Test2) | 8*16=128 | 16*16=256 | 2*16*16=512 | 8*64=512 | 8*64=512 |

Table 3. multiplication count for RST in comparison with the proposed method

As can be seen, the proposed method achieves the minimum number of operations among the different configurations. The only exception is the case of 8x8 transform block size, where CE6-3.1D achieves the minimum. It should be noted that the present method can be applied with the configuration of CE6-3.1D with comparable results. However, this is not tested due to time limitation.

# Conclusion

The present contribution provides a very simplified version of a secondary transform that is based on RST method. The memory and number of operations is halved with respect to the least complex RST (CE6-3.1E), and the number of operations is highly reduced. Further, the secondary flag coding is also simplified by skipping the check of significant coefficient inside the secondary transform region.

It is proposed to adopt the concept of NSST with 8 coefficients only. It is because it provides a significant gain without an increase in the encoder/decoder time and with a memory requirement that is equivalent to MTS.

# Reference

- M. Salehifar et al., “CE6: Reduced Secondary Transform (RST) (CE6-3.1)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 14 JVET-N0193, 11th meeting, Geneve.

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, 13th JVET meeting, Marrakech, MA, 9–18 Jan. 2019.

# Patent rights declaration

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).