| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0510-v1 |
| --- | --- |

| Title: | CE2-related: Phase-variant affine subblock motion compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Ching-Yeh Chen, Chen-Yen Lai, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {peter.chuang, chingyeh.chen, jenny.lai, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, phase-variant affine subblock motion compensation (MC) is proposed. The conventional two stage horizontal-vertical interpolation is applied. However, unlike the phase-invariant block-based MC that uses the same horizontal filter for all sample rows and the same vertical filter for all sample columns, different phases of filters can be applied to different sample rows and different sample columns in the affine subblock. The used interpolation filter sets in the proposed method are the same as those in VTM4.0. The number of filtering operations for each affine subblock in the proposed method is also the same as that in VTM4.0. Simulation results reportedly show that the luma BD-rates are -0.xx% and -0.xx% in RA and LB cases, respectively.

# Introduction

In VTM4.0, when affine mode is applied, the coding unit (CU) is divided into 4×4 subblocks. The motion vector (MV) of each subblock is derived by using an affine motion model. For affine mode MC, the 4×4 block-based translational motion model MC is applied. The 4×4 block-based translational motion model MC can be divided into two stages. The first stage is to filter an (4+L–1)×(4+L–1) reference block window with (4+L–1) rows of horizontal filtering, where the L is the filter tap length of the interpolation filer. In VTM4.0, the L is eight for luma component and four for chroma components. For each row, four times of horizontal filtering are performed to generate 4×1 horizontally filtered samples. Therefore, in the first stage, in total 4×(4+L–1) times of horizontal filtering are performed, and 4×(4+L–1) horizontally filtered samples are generated. The horizontal filter phases of all the sample rows in the (4+L–1)×(4+L–1) reference block window are the same. In the second stage, the 4×(4+L–1) filtered samples are further vertically filtered for the four sample columns. For each column, four times of vertical filtering are performed to generate 1×4 vertically filtered samples. Therefore, in the second stage, in total 4×4 times of vertical filtering are performed, and 4×4 vertically filtered samples are generated as the MC results. The vertical filter phases of all the sample columns in the horizontally filtered block of 4×(4+L–1) samples are also the same. The horizontal filter phase and the vertical filter phase are derived from the MV of the 4×4 subblock. In each 4×4 subblock, the translational motion model is applied using phase-invariant MC.

# Proposed methods

To better approximate the affine motion model in an affine subblock, phase-variant MC is applied to the subblock. In the proposed method, the affine coded block is also divided into 4×4 subblocks, and a subblock MV is derived for each subblock as done in VTM4.0. The MC of each subblock is divided into two stages. The first stage is to filter an (4+L–1)×(4+L–1) reference block window with (4+L–1) rows of horizontal filtering, where the L is the filter tap length of the interpolation filer. However, different from the translational MC, in the proposed phase-variant affine subblock MC, the filter phase of each sample row is different. For each sample row, MVx is derived as follows.

MVx = (subblockMVx<<7 + dMvVerX×(rowIdx – L/2 – 2)) >>7                   (Equation 1)

The filter phase of each sample row is derived from the MVx. The subblockMVx is the x component of the MV of the derived subblock MV as done in VTM4.0. The rowIdx is the sample row index. The dMvVerX is (cuBottomLeftCPMVx – cuTopLeftCPMVx) << (7 – log2LumaCbHeight), where cuBottomLeftCPMVx is the x component of the CU bottom left control point MV, cuTopLeftCPMVx is the x component of the CU top left control point MV, and LumaCbHeight is the log2 of the height of the luma coding block (CB).

After the horizontal filtering, a 4×(4+L–1) horizontally filtered samples are generated. Figure 1 shows the concept of the proposed horizontal filtering. The gray points are the samples of the reference block window, and the orange points denote the horizontally filtered samples. A blue tube of 8×1 samples denotes applying one time of 8-tap horizontal filtering, as shown in Figure 1 and Figure 2. Each sample row needs four times of horizontal filtering. The filter phases on a sample row are the same. However, the filter phases on different rows are different. Skewed 4×11 samples are generated.

Figure 1.  Phase-variant horizontal filtering

Figure 2.  Illustration of applying one time of 8-tap horizontal filtering

In the second stage, the 4×(4+L–1) horizontally filtered samples (orange samples in Figure 1) are further vertically filtered. For each sample column, MVy is derived as follows.

MVy = (subblockMVy<<7 + dMvHorY×(columnIdx – 2)) >>7                    (Equation 2)

The filter phase of each sample column is derived from the MVy. The subblockMVy is the y component of the MV of the derived subblock MV as done in VTM4.0. The columnIdx is the sample column index. The dMvHorY is (cuTopRightCPMVy – cuTopLeftCPMVy) << (7 – log2LumaCbWidth), where cuTopRightCPMVy is the y component of the CU top right control point MV, cuTopLeftCPMVy is the y component of the CU top left control point MV, and log2LumaCbWidth is the log2 of the width of the luma CB.

After the vertical filtering, 4×4 affine subblock prediction samples are generated. Figure 3 shows the concept of the proposed vertical filtering. The light orange points are the horizontally filtered samples from first stage. The red points are the vertically filtered samples as the final prediction samples.

Figure 3. Non-uniform phase vertical filtering

In this proposal, the used interpolation filter sets are the same as those in VTM4.0. The only difference is that the horizontal filter phases are row-dependent and the vertical filter phases are column-dependent in this proposal, whereas they are invariant within the affine subblock in VTM4.0. As for the number of filtering operations for each affine subblock in the proposed method, it is the same as that in VTM4.0.

# Simulation results

The common test conditions [1] are used for evaluation. The results are shown in Table 1. Simulation results reportedly show that the average luma BD-rates are -0.xx% and -0.xx% in RA and LB cases, respectively.

Table 1.  Results of the proposed phase-variant affine subblock MC

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.21% | -0.28% | -0.34% | 101% |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.20% | -0.02% | -0.32% | 101% | 100% |
| Class E | -0.11% | -0.46% | -0.68% | 101% | 96% |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

# Conclusions

In this contribution, phase-variant affine subblock MC is proposed. Different phases of filters can be applied to different sample rows for horizontal filtering and different sample columns for vertical filtering. Simulation results reportedly show that the average luma BD-rates are -0.xx% and -0.xx% in RA and LB cases, respectively.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-M1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).