| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, Switzerland, 19–27 March 2019 | Document: JVET-N0511-v1 |
| --- | --- |

| Title: | AHG17: Signalling leading picture information in the NAL unit header structure |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lulin Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd, Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {lulin.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution is a continuation of JVET-N0072 to signal the leading picture related information in the NAL unit header structure. The information can be used by file writer, middle box, or system applications. The suggested syntax and semantics are provided.

# Introduction

In JVET-M1001-v6 [1], NAL unit type codes and NAL unit type classes are in Table 7-1. The leading picture and IRAP picture related NAL unit types are added as highlighted.

Table 7-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 | TRAIL_NUT | Coded tile group of a non- STSA trailing picture tile_group_layer_rbsp( ) | VCL |
| 1 | STSA_NUT | Coded tile group of an STSA picture tile_group_layer_rbsp( ) | VCL |
| 2 | RASL_NUT | Coded tile group of a RASL picture tile_group_layer_rbsp( ) | VCL |
| 3 | RADL_NUT | Coded tile group of a RADL picture tile_group_layer_rbsp( ) | VCL |
| 4..7 | RSV_VCL_4.. RSV_VCL_7 | Reserved non-IRAP VCL NAL unit types | VCL |
| 8 9 | IDR_W_RADL IDR_N_LP | Coded tile group of an IDR picture tile_group_layer_rbsp( ) | VCL |
| 10 | CRA_NUT | Coded tile group of a CRA picture tile_group_layer_rbsp( ) | VCL |
| 11 12 13 | RSV_IRAP_VCL11 RSV_IRAP_VCL12 RSV_IRAP_VCL13 | Reserved IRAP VCL NAL unit types | VCL |
| 14..15 | RSV_VCL14.. RSV_VCL15 | Reserved non-IRAP VCL NAL unit types | VCL |
| 16 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 17 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 18 | APS_NUT | Adaptation parameter set adaptation_parameter_set_rbsp( ) | non-VCL |
| 19 | AUD_NUT | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 20 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | non-VCL |
| 21 | EOB_NUT | End of bitstream end_of_bitstream_rbsp( ) | non-VCL |
| 22, 23 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) | non-VCL |
| 24..27 | RSV_NVCL24.. RSV_NVCL27 | Reserved non-VCL NAL unit types | non-VCL |
| 28..31 | UNSPEC28.. UNSPEC31 | Unspecified non-VCL NAL unit types | non-VCL |

In MPEG base media file format (ISOBMFF) [3], the six stream access point (SAP) types are specified in Annex I. Additionally, a leading sample (corresponding to a leading picture in HEVC [2]) and the related information, e.g. num_leading_samples, are also specified for use of a file writer.

# Proposed method

The first three SAP types in ISOBMFF are addressed accordingly with the picture properties of IDR, CRA, RADL, and RASL as in Table 7-1. This contribution proposes to expose the RADL and RASL related information in the NAL unit header structure to be friendly used by file writer, middle box, or system applications.

A RADL or a RASL picture may be associated with an IDR, CRA, or other IRAP picture. The information of its position to the associated picture may be useful for the level of middle box or system application and that of the number of leading pictures associated with an IDR, CRA, or other IRAP picture may be useful as well. Actually, the syntax element num_leading_samples in ISOBMFF stores the information as a general property.

The corresponding signalling is in the NAL unit header structure as below while keeping the header length of 16 bits.

      - NAL unit header syntax

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(5) |
| if( nal_unit_type == DRAP_NUT ){ |  |
| drap_poc_cnt | u(7) |
| nuh_reserved_zero_3bits | u(3) |
| } else if ( nal_unit_type == GDR_NUT ){ |  |
| refreshed_region_flag | f(1) |
| recovery_poc_cnt | u(6) |
| nuh_reserved_zero_3bits | u(3) |
| } else if ( nal_unit_type == IDR_ W_RADL ¦¦ nal_unit_type == CRA_NUT ¦¦ nal_unit_type == IRAP_NUT ) { |  |
| num_leading_pictures | u(4) |
| nuh_temporal_id_plus1 | u(3) |
| nuh_reserved_zero_3bits | u(3) |
| } else if ( nal_unit_type == RADL_NUT ¦¦ nal_unit_type == RASL_NUT ) { |  |
| leading_picture_poc_cnt | u(5) |
| nuh_temporal_id_plus1 | u(3) |
| nuh_reserved_zero_3bits | u(2) |
| } else { |  |
| nuh_temporal_id_plus1 | u(3) |
| nuh_reserved_zero_7bits | u(7) |
| } |  |
| } |  |

NOTE – IRAP_NUT is one of the reserved IRAP VCL NAL unit types

NAL unit header Semantics

…

num_leading_picture specifies the number of leading pictures of the associated picture

leading_picture_poc_cnt specifies the leading picture position to its associated picture in output order

…

# References

- “Versatile Video Coding (Draft 4),” JVET-M1001-v6, January 2019, Marrakesh, Morocco.

- “High Efficiency Video Coding,” ITU-T H.265, February 2018.

- “ISO base media file format,” ISO/IEC 14496-12, W17277, February 2015.

- “AHG17: New NAL unit types for VVC,” JVET-N0072, March 2019, Geneva, Switzerland.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).