| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0512 |
| --- | --- |

| Title: | AhG2 : Editorial Suggestion on the text specification for INTRA with interaction part between INTRA and IBC |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposed draft |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Jin Heo Junghak Nam Sunmi Yoo Jangwon Choi 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seunghwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution introduces editorial suggestions to VVC WD specification for clarifying derivation processes for INTRA and corner case handling when Intra DM for IBC mode is enabled at dual tree structure (i.e., tile_group_type is equal to Intra).

Problem statements

- Reference sample filtering process for intra sample prediction

– There is a mismatch between spec text and reference software in conditions to set filterFlag

- Specification of INTRA_ANGULAR2..INTRA_ANGULAR66 intra prediction modes

– Derivation process for cubicFilterFlag

3) Current spec text doesn’t support for the Intra_DM when current chroma block refer a luma block which is coded MODE_IBC in dual tree structure.

Proposed method

This contribution suggests editorial specification changes in order to align with reference SW.

The suggested editorial changes are described as bellow briefly.

1) Reference sample filtering process for intra sample prediction

Reference sample filtering process Inputs to this process are: - a variable refIdx specifying the intra prediction reference line index, - a variable nTbW specifying the transform block width, - a variable nTbH specifying the transform block height, - a variable refW specifying the reference samples width, - a variable refH specifying the reference samples height, - the (unfiltered) neighbouring samples refUnfilt[ x ][ y ], with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx, - a variable cIdx specifying the colour component of the current block. Outputs of this process are: - The variable filterFlag specifying the reference sample filter flag. - the reference samples p[ x ][ y ], with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx. The variable filterFlag is derived as follows: - If all of the following conditions are true, filterFlag is set equal to 1: - refIdx is equal to 0 - cIdx is equal to 0 - predModeIntra is not equal to INTRA_DC - IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT - one or more of the following conditions is true: - predModeIntra is equal to INTRA_PLANAR and nTbW * nTbH is greater than 32 - the variable whRatio is set equal to Abs( Log2( nW / nH ) ). nTbW is greater than nTbH and predModeIntra is greater than or euqal to 2 and predModeIntra is less than (whRatio > 1) ? ( 8 + 2*whRatio) : 8 - the variable whRatio is set equal to Abs( Log2( nW / nH ) ). nTbH is greater than nTbW and predModeIntra is less than or euqal to 66 and predModeIntra is less than (whRatio > 1) ? ( 60 - 2*whRatio) : 60 - The variable minDistVerHor is set equal to Min( Abs( predModeIntra − 50 ), Abs( predModeIntra − 18 ) ). The variable intraHorVerDistThres[ nTbS ] is specified in Table 8-4. minDistVerHor is greater than intraHorVerDistThres[ nTbS Otherwise, filterFlag is set equal to 0. Table 8-4 – Specification of intraHorVerDistThres[ nTbS ] for various transform block sizes nTbS nTbS = 2; nTbS = 3; nTbS = 4; nTbS = 5; nTbS = 6; nTbS = 7. intraHorVerDistThres[ nTbS ]; 20; 14; 2; 0; 0; 0.

2) Specification of INTRA_ANGULAR2..INTRA_ANGULAR66 intra prediction modes

Specification of INTRA_ANGULAR2..INTRA_ANGULAR66 intra prediction modes Inputs to this process are: - the intra prediction mode predModeIntra, - a variable refIdx specifying the intra prediction reference line index, - a variable nTbW specifying the transform block width, - a variable nTbH specifying the transform block height, - a variable refW specifying the reference samples width, - a variable refH specifying the reference samples height, - a variable nCbW specifying the coding block width, - a variable nCbH specifying the coding block height, - a variable cIdx specifying the colour component of the current block, - a variable filterFlag specifying the reference sample filter flag, - the neighbouring samples p[ x ][ y ], with x = −1− refIdx, y = −1− refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1− refIdx. …. The variable cubicFilterFlag is derived as follows: - If one or more of the following conditions is true, cubicFilterFlag is set equal to 1. - predModeIntra is equal to INTRA_ANGULAR2, INTRA_ANGULAR34 or INTRA_ANGULAR66 - refIdx is not equal to 0 - filterFlag is equal to 0 - IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT and cIdx is equal to 0 and predModeIntra is greater than or equal to INTRA_ANGULAR34 and nW is greater than 8 - IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT and cIdx is equal to 0 and predModeIntra is less than INTRA_ANGULAR34 and nH is greater than 8. Otherwise, cubicFilterFlag is set equal to 0.… The values of the prediction samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows: - If predModeIntra is greater than or equal to 34, the following ordered steps apply: - The reference sample array ref[ x ] is specified as follows: - The following applies: ref[ x ] = p[ −1 − refIdx + x ][ −1 − refIdx ], with x = 0..nTbW + refIdx (8-130) - If intraPredAngle is less than 0, the main reference sample array is extended as follows: - When ( nTbH * intraPredAngle ) >> 5 is less than −1, ref[ x ] = p[ −1 − refIdx ][ −1 − refIdx + ( ( x * invAngle + 128 ) >> 8 ) ], with x = −1..( nTbH * intraPredAngle ) >> 5 (8-131) ref[ ( ( nTbH * intraPredAngle ) >> 5 ) − 1 ] = ref[ ( nTbH * intraPredAngle ) >> 5 ] (8-132) ref[ nTbW + 1 + refIdx ] = ref[ nTbW + refIdx ] (8-133) - Otherwise, ref[ x ] = p[ −1 − refIdx + x ][ −1 − refIdx ], with x = nTbW + 1 + refIdx..refW + refIdx (8-134) ref[ −1 ] = ref[ 0 ] (8-135) - The additional samples ref[ refW + refIdx +x ] with x = 1..( Max( 1, nTbW / nTbH ) * refIdx + 1) are derived as follows: ref[ refW + refIdx +x ] = p[ −1 + refW ][ −1 − refIdx ] (8-136) - The values of the prediction samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows: - If intraPredAngle is equal to 0, the following applies: predSamples[ x ][ y ] = ref[ x + 1 ] - Otherwise, the follwing applies: - The index variable iIdx and the multiplication factor iFact are derived as follows: iIdx = ( ( y + 1 + refIdx ) * intraPredAngle ) >> 5 + refIdx (8-137) iFact = ( ( y + 1 + refIdx ) * intraPredAngle ) & 31 (8-138) - If cIdx is equal to 0, the following applies: - The interpolation filter coefficients fT[ j ] with j = 0..3 are derived as follows: fT[ j ] = cubicFilterFlag ? fC[ iFact ][ j ] : fG[ iFact ][ j ] (8-139) - The value of the prediction samples predSamples[ x ][ y ] is derived as follows: predSamples[ x ][ y ] = Clip1Y( ( ( i=03fT[ i ]*ref[ x+iIdx+i ] ) + 32 ) >> 6 ) (8-140) - Otherwise (cIdx is not equal to 0), depending on the value of iFact, the following applies: predSamples[ x ][ y ] = ( ( 32 − iFact ) * ref[ x + iIdx + 1 ] + iFact * ref[ x + iIdx + 2 ] + 16 ) >> 5 (8-141) - Otherwise (predModeIntra is less than 34), the following ordered steps apply: - The reference sample array ref[ x ] is specified as follows: - The following applies: ref[ x ] = p[ −1 − refIdx ][ −1 − refIdx + x ], with x = 0..nTbH + refIdx (8-143) - If intraPredAngle is less than 0, the main reference sample array is extended as follows: - When ( nTbW * intraPredAngle ) >> 5 is less than −1, ref[ x ] = p[ −1 − refIdx + ( ( x * invAngle + 128 ) >> 8 ) ][ −1 − refIdx ], with x = −1..( nTbW * intraPredAngle ) >> 5 (8-144) ref[ ( ( nTbW * intraPredAngle ) >> 5 ) − 1 ] = ref[ ( nTbW * intraPredAngle ) >> 5 ] (8-145) ref[ nTbG + 1 + refIdx ] = ref[ nTbH + refIdx ] (8-146) - Otherwise, ref[ x ] = p[ −1 − refIdx ][ −1 − refIdx + x ], with x = nTbH + 1 + refIdx..refH + refIdx (8-147) ref[ −1 ] = ref[ 0 ] (8-148) - The additional samples ref[ refH + refIdx +x ] with x = 1..( Max( 1, nTbW / nTbH ) * refIdx + 1) are derived as follows: ref[ refH + refIdx +x ] = p[ −1 + refH ][ −1 − refIdx ] (8-149) - The values of the prediction samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows: - If intraPredAngle is not equal to 0, the following applies: predSamples[ x ][ y ] = ref[ y + 1 ] - Otherwise, the follwing applies: - The index variable iIdx and the multiplication factor iFact are derived as follows: iIdx = ( ( x + 1 + refIdx ) * intraPredAngle ) >> 5 (8-150) iFact = ( ( x + 1 + refIdx ) * intraPredAngle ) & 31 (8-151) - If cIdx is equal to 0, the following applies: - The interpolation filter coefficients fT[ j ] with j = 0..3 are derived as follows: fT[ j ] = cubicFilterFlag ? fC[ iFact ][ j ] : fG[ iFact ][ j ] (8-152) - The value of the prediction samples predSamples[ x ][ y ] is derived as follows: predSamples[ x ][ y ] = Clip1Y( ( ( i=03fT[ i ]*ref[ y+iIdx+i ] ) + 32 ) >> 6 ) (8-153) - Otherwise (cIdx is not equal to 0), depending on the value of iFact, the following applies: predSamples[ x ][ y ] = ( ( 32 − iFact ) * ref[ y + iIdx + 1 ] + iFact * ref[ y + iIdx + 2 ] + 16 ) >> 5 (8-154)

3) Proposed method for handling the Intra_DM when current chroma block refer a luma block which is coded MODE_IBC in dual tree structure.

Derivation process for chroma intra prediction mode Input to this process are: - a luma location ( xCb, yCb ) specifying the top-left sample of the current chroma coding block relative to the top-left luma sample of the current picture, - a variable cbWidth specifying the width of the current coding block in luma samples, - a variable cbHeight specifying the height of the current coding block in luma samples. - a variable treeType specifying whether a single or a dual tree is used and if a dual tree is used, it specifies whether the current tree corresponds to the luma or chroma components. In this process, the chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is derived as following:. - If treeType is equal to DUAL_TREE_CHROMA_and CuPredMode[ xCb + (cbWidth /2) ][ yCb + (cbHeight /2) ][DUAL_TREE_LUMA] is equal to MODE_INTRA, the chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is derived using intra_chroma_pred_mode[ xCb ][ yCb ] and IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] as specified in Table 8-2 and Table 8-3. - Otherwise, if treeType is equal to SINGLE_TREE and CuPredMode[ xCb + (cbWidth /2)) ][ yCb + (cbHeight /2) ][SINGLE_TREE] is equal to MODE_INTRA, the chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is derived using intra_chroma_pred_mode[ xCb ][ yCb ] and IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] as specified in Table 8-2 and Table 8-3. - Otherwise, IntraPredModeC[ xCb ][ yCb ] is set equal to INTRA_DC. Table 8-2 – Specification of IntraPredModeC[ xCb ][ yCb ] depending on intra_chroma_pred_mode[ xCb ][ yCb ] and IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] when sps_cclm_enabled_flag is equal to 0 intra_chroma_pred_mode[ xCb ][ yCb ]; IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ]. 0; 50; 18; 1; X ( 0 <= X <= 66 ). 0; 66; 0; 0; 0; 0. 1; 50; 66; 50; 50; 50. 2; 18; 18; 66; 18; 18. 3; 1; 1; 1; 66; 1. 4; 0; 50; 18; 1; X. Table 8-3 – Specification of IntraPredModeC[ xCb ][ yCb ] depending on intra_chroma_pred_mode[ xCb ][ yCb ] and IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] when sps_cclm_enabled_flag is equal to 1 intra_chroma_pred_mode[ xCb ][ yCb ]; IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ]. 0; 50; 18; 1; X ( 0 <= X <= 66 ). 0; 66; 0; 0; 0; 0. 1; 50; 66; 50; 50; 50. 2; 18; 18; 66; 18; 18. 3; 1; 1; 1; 66; 1. 4; 81; 81; 81; 81; 81. 5; 82; 82; 82; 82; 82. 6; 83; 83; 83; 83; 83. 7; 0; 50; 18; 1; X.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).