| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0512 |
| --- | --- |

| Title: | AhG2 : Editorial Suggestion on the text specification for INTRA with interaction part between INTRA and IBC |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposed draft |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Jin Heo Junghak Nam Sunmi Yoo Jangwon Choi 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seunghwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution introduces editorial suggestions to VVC WD specification for clarifying derivation processes for INTRA and corner case handling when Intra DM for IBC mode is enabled at dual tree structure (i.e., tile_group_type is equal to Intra).

Problem statements

1) Current spec text doesn’t support for the Intra_DM when current chroma block refer a luma block which is coded MODE_IBC in dual tree structure.

2) Proposed editorial suggestion define addressed luma prediction mode for dual tree structure.

Proposed method

This contribution suggests editorial specification changes in order to align with reference SW.

The suggested editorial changes are described as bellow briefly.

1) Proposed method for handling the Intra_DM when current chroma block refer a luma block which is coded MODE_IBC in dual tree structure.

Derivation process for chroma intra prediction mode Input to this process are: - a luma location ( xCb, yCb ) specifying the top-left sample of the current chroma coding block relative to the top-left luma sample of the current picture, - a variable cbWidth specifying the width of the current coding block in luma samples, - a variable cbHeight specifying the height of the current coding block in luma samples. - a variable treeType specifying whether a single or a dual tree is used and if a dual tree is used, it specifies whether the current tree corresponds to the luma or chroma components. In this process, the chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is derived as following:. - If treeType is equal to DUAL_TREE_CHROMA_and CuPredMode[ xCb + (cbWidth /2) ][ yCb + (cbHeight /2) ][DUAL_TREE_LUMA] is equal to MODE_INTRA, the chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is derived using intra_chroma_pred_mode[ xCb ][ yCb ] and IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] as specified in Table 8-2 and Table 8-3. - Otherwise, if treeType is equal to SINGLE_TREE and CuPredMode[ xCb + (cbWidth /2)) ][ yCb + (cbHeight /2) ][SINGLE_TREE] is equal to MODE_INTRA, the chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is derived using intra_chroma_pred_mode[ xCb ][ yCb ] and IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] as specified in Table 8-2 and Table 8-3. - Otherwise, IntraPredModeC[ xCb ][ yCb ] is set equal to INTRA_DC. Table 8-2 – Specification of IntraPredModeC[ xCb ][ yCb ] depending on intra_chroma_pred_mode[ xCb ][ yCb ] and IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] when sps_cclm_enabled_flag is equal to 0 intra_chroma_pred_mode[ xCb ][ yCb ]; IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ]. 0; 50; 18; 1; X ( 0 <= X <= 66 ). 0; 66; 0; 0; 0; 0. 1; 50; 66; 50; 50; 50. 2; 18; 18; 66; 18; 18. 3; 1; 1; 1; 66; 1. 4; 0; 50; 18; 1; X. Table 8-3 – Specification of IntraPredModeC[ xCb ][ yCb ] depending on intra_chroma_pred_mode[ xCb ][ yCb ] and IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] when sps_cclm_enabled_flag is equal to 1 intra_chroma_pred_mode[ xCb ][ yCb ]; IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ]. 0; 50; 18; 1; X ( 0 <= X <= 66 ). 0; 66; 0; 0; 0; 0. 1; 50; 66; 50; 50; 50. 2; 18; 18; 66; 18; 18. 3; 1; 1; 1; 66; 1. 4; 81; 81; 81; 81; 81. 5; 82; 82; 82; 82; 82. 6; 83; 83; 83; 83; 83. 7; 0; 50; 18; 1; X.

2) Proposed editorial suggestion define addressed luma prediction mode for dual tree

Decoding process for coding units coded in IBC prediction mode General decoding process for coding units coded in IBC prediction mode The decoding process for coding units coded in ibc prediction mode consists of the following ordered steps: - The motion vector components of the current coding unit are derived as follows: - ….. - Otherwise, if treeType is equal to DUAL_TREE_CHROMA, the following applies: - The number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY are derived as follows: numSbX = ( cbWidth >> 2 ) (8-955) numSbY = ( cbHeight >> 2 ) (8-956) - The chroma motion vectors mvC[ xSbIdx ][ ySbIdx ] are derived as follows for xSbIdx = 0..numSbX − 1, ySbIdx = 0..numSbY − 1: - The luma motion vector mvL[ xSbIdx ][ ySbIdx ] is derived as follows: - The location ( xCuY, yCuY ) of the collocated luma coding unit is dervied as follows: xCuY = xCb + xSbIdx*4 (8-957) yCuY = yCb + ySbIdx*4 (8-958) - If CuPredMode[ xCuY ][ yCuY ][DUAL_TREE_LUMA] is equal to MODE_INTRA, the following applies. mvL[ xSbIdx ][ ySbIdx ][ 0 ] = 0 (8-959) mvL[ xSbIdx ][ ySbIdx ][ 1 ] = 0 (8-960) predFlagL0[ xSbIdx ][ ySbIdx ] = 0 (8-961) predFlagL1[ xSbIdx ][ ySbIdx ] = 0 (8-962) - Otherwise ( CuPredMode[ xCuY ][ yCuY ][DUAL_TREE_LUMA] is equal to MODE_IBC ), the following applies: mvL[ xSbIdx ][ ySbIdx ][ 0 ]=MvL0[ xCuY ][ yCuY ][ 0 ] (8-963) mvL[ xSbIdx ][ ySbIdx ][ 1 ]=MvL0[ xCuY ][ yCuY ][ 1 ] (8-964) predFlagL0[ xSbIdx ][ ySbIdx ] = 1 (8-965) predFlagL1[ xSbIdx ][ ySbIdx ] = 0 (8-966)

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).