| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0514-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-N0262 (CE9-related: Disabling DMVR for non equal weight BPWA) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yu-Ling Hsiao No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 yuling.hsiao@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The document reports the results of crosschecking JVET-N0262 proposed by Qualcomm. JVET-N0262 proposes a method to disable decoder-side motion vector refinement if the weights used in bi-prediction weighted average is not default weights. The verification task was done, and the BD-rate results match exactly with those in JVET-N0262.

# Verification

Qualcomm provided the source code of their proposed method described in JVET-N0262 [1]. One verification was carried out by setting the following parameters in the provided code:

#define GBI_DISABLE_DMVR                   1

The test is conducted under the the common test conditions (CTC) defined in [2] and are carried out by using the VTM4.0 software [3]. Table 1 shows the results using VTM4.0 as the anchor and enabling the proposed method in the test. The BD-rates of the test from our experimental results match exactly those in JVET-N0262. Please note that the runtime comparison may be not accurate.

Table 1. The results of JVET-N0262 using VTM4.0 as anchor

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.05% | 0.06% | 100% | 101% |
| Class A2 | 0.02% | 0.10% | 0.10% | 100% | 100% |
| Class B | -0.01% | 0.03% | -0.01% | 100% | 99% |
| Class C | 0.00% | -0.01% | -0.03% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.04% | 0.02% | 100% | 100% |
| Class D | 0.02% | -0.09% | -0.08% | 100% | 100% |
| Class F | -0.10% | -0.07% | -0.08% | 100% | 100% |

# References

- H. Huang, W.-J. Chien, M. Karczewicz, “CE9-related: Disabling DMVR for non equal weight BPWA,” Document of Joint Video Experts Team, JVET-N0262.

- F. Bossen, J. Boyce, K. Sühring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-M1010.

- VTM4.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).