| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0518 |
| --- | --- |

| Title: | Crosscheck of JVET-N0365 (CE6-related: MPM based non-separable secondary transform) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Pierrick Philippe | Email: | pierrick.philippe@orange.com |
| Source: | Orange, bcom |  |  |

_____________________________

# Abstract

This document reports the crosscheck of  JVET-N0365. It is reported that the average BD-rates match those provided by the proponents. No discrepancies have been found between the provided software and the description of the tests. The execution times are within the measurement error range.

# Description

The proposal aims at reducing the encoder search when NSST are used and also reducing the memory footprint for the second test method.

Two methods are proposed. The code is based on the code provided by the proponents of CE6-3.1d. The software modifications are concentrated in CabacWriter/Reader.cpp and in intraSearch.cpp.

Method 1, the NSST index is inferred for the prediction modes not part of the MPM list. The NSST index is set zero, and the selection of the best NSST index is avoided.

In  Method 2, Method 1 is used, but NSST is also inferred to zero when the MPMs are different from DC and PLANAR. Therefore, the secondary transforms for those mode are not needed anymore, some storage space is saved.

# Experimental Results

Experiments were performed based on VTM4.0, in All Intra configuration, using the common test conditions described in JVET-M1010. The average BD-rates match those provided by the proponents and the execution times are within the measurement error range.

## Method 1

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.59% | -0.30% | -0.26% | 121% | 98% |
| Class A2 | -1.04% | 0.38% | 0.68% | 125% | 99% |
| Class B | -0.78% | 0.03% | -0.86% | 123% | 99% |
| Class C | -0.90% | -0.12% | -0.87% | 121% | 101% |
| Class E | -1.07% | -1.05% | -1.78% | 114% | 101% |
| Overall | -1.03% | -0.18% | -0.66% | 121% | 100% |
| Class D | -0.64% | -0.24% | -0.79% | 124% | 102% |
| Class F | -0.91% | 0.04% | -0.25% | 114% | 100% |

## Method 2

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.00% | -0.53% | -0.31% | 114% | 100% |
| Class A2 | -0.91% | 0.26% | 0.61% | 117% | 100% |
| Class B | -0.62% | -0.13% | -0.99% | 115% | 100% |
| Class C | -0.84% | -0.20% | -0.89% | 114% | 105% |
| Class E | -0.85% | -0.98% | -1.81% | 109% | 102% |
| Overall | -0.82% | -0.29% | -0.72% | 114% | 101% |
| Class D | -0.61% | -0.37% | -1.01% | 115% | 106% |
| Class F | -0.84% | -0.07% | -0.30% | 110% | 100% |

# Patent rights declaration

Orange and b<>com may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).