| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0519 |
| --- | --- |

| Title: | Crosscheck of JVET-N0424 (CE6-related: An Explicit MTS Design with Fast Encoder) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Pierrick Philippe | Email: | pierrick.philippe@orange.com |
| Source: | Orange, bcom |  |  |

_____________________________

# Abstract

This document reports the crosscheck of JVET-N0424. It is reported that the average BD-rates match those provided by the proponents. No discrepancies have been found between the provided software and the description of the tests. The execution times are within the measurement error range.

# Description

In this proposal, the DCT-8 is replaced by the identity transform for the last mts indexes (3-5). The replacement depends on the transform length (the identity transforms is used for transform length less or equal than 16. This is only applied to intra CUs.

The software has been verified, it is based on VTM-4.0 with JVET_M0297_32PT_MTS_ZERO_OUT.

- RomTr.cpp, TrQuant_EMT.cpp and UnitTools.cpp are affected in order to introduce the Identity transform (IDT)

- TrQuant.cpp implements the logics of the IDT/DCT8 selection, also the L1 proposed modification is applied to affect the mts selection.

- UnitTools.cpp

- IntraSearch.cpp, further tests are performed to remove a speed-up of VTM-4.0 (

Consequently, the software matches the description made in JVET-N0424.

# Experimental Results

Experiments were performed based on VTM4.0, in All Intra configuration, using the common test conditions described in JVET-M1010. The items from Class TGM were also tested, based on proponent request.

The average BD-rates match those provided by the proponents and the execution times are within the measurement error range.

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | -0.05% | -0.06% | 89% | 101% |
| Class A2 | 0.17% | 0.05% | -0.10% | 89% | 101% |
| Class B | 0.10% | -0.21% | -0.12% | 88% | 101% |
| Class C | -0.04% | -0.29% | -0.27% | 90% | 103% |
| Class E | -0.12% | -0.27% | -0.28% | 88% | 103% |
| Overall | 0.04% | -0.17% | -0.17% | 89% | 102% |
| Class D | -0.07% | -0.30% | -0.34% | 90% | 101% |
| Class F | -0.26% | -0.23% | -0.15% | 98% | 102% |
| Class TGM | -0.27% | -0.07% | -0.06% | 104% | 98% |
|  |  |  | Random Access |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | -0.13% | -0.43% | 99% | 99% |
| Class C | 0.01% | 0.01% | -0.20% | 99% | 102% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.02% | -0.38% | -0.26% | 99% | 101% |
| Class F |  |  |  |  |  |

# Patent rights declaration

Orange and b<>com may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).