| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0524-v1 |
| --- | --- |

| Title: | Non-CE3: Modified beta derivation in CCLM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Masaru Ikeda Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81 50 3750 2782 masaru.ikeda@sony.com teruhiko.s@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This proposal presents a modification to derive the linear model parameter β using luma and chroma averaged values. The BD-Rate number for the modification is, for Y/U/V, reportedly shows -0.11% / -0.35% / -0.37% at AI over VTM-4.0, respectively.

Introduction

In the current VTM4.0 the cross-component linear model (CCLM) method is included. This method obtains the linear model parameters α and β by a straight line equation with two points. The two points (A, B) are the minimum and maximum values inside the set of neighboring Luma samples as shown in Figure 1.

Figure 1: Illustration of the straight line with minimum and maximum Luma vlaue

In the equation the linear model parameters α and β are derived according to the following equation [1]:

α=yB-yAxB-xA

β=yA-α*xA

The proposed method addresses the derivation of β in order to improve the coding efficiency.

Proposed method

In the proposed method, β is derived using both Luma and Chroma averaged values (xC, yC), which are calculated inside the set of neighboring samples together with the minimum and maximum values, instead of minimum values (xA, yA) as shown in Figure 2.

Figure 2: Illustration of the straight line with Luma and Chroma averaged values for β derivation

In the equation the linear model parameter α is not changed, but the parameter β is derived according to the following equation:

β=yC-α*xC

Experimental results

Proposed method was implemented on top of VTM-4.0 and tested based on CE3 test conditions [2].

Table 1: BD-Rate of AI

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.33% | -0.50% | -0.39% | #NUM! | #DIV/0! |
| Class A2 | -0.12% | -0.29% | -0.31% | #NUM! | #DIV/0! |
| Class B | -0.08% | -0.35% | -0.49% | #NUM! | #DIV/0! |
| Class C | -0.04% | -0.41% | -0.23% | #NUM! | #DIV/0! |
| Class E | -0.02% | -0.19% | -0.43% | #NUM! | #DIV/0! |
| Overall | -0.11% | -0.35% | -0.37% | #NUM! | #DIV/0! |
| Class D | 0.00% | -0.31% | -0.33% | #NUM! | #DIV/0! |
| Class F (optional) | -0.01% | -0.05% | 0.01% | #NUM! | #DIV/0! |

Conclusion

This proposal presents a modification to derive the linear model parameter β using luma and chroma averaged values. It gives us slightly increased complexity, but good coding gains for chroma as well. The proposed method can be applied to multi-model linear model as well. We recommend considering that the modification is included into next version of test model or studied in CE.

Reference

[1] G. Laroche, J. Taquet, C. Gisquet, P. Onno (Canon), “CE3-5.1: On cross-component linear model simplification”, JVET-L0191, October 2018

[2] G. Van der Auwera, L. Li, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding”, JVET-M01023, January 2019

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

WD changes

8.4.4.2.8 Specification of INTRA_LT_CCLM, INTRA_L_CCLM and INTRA_T_CCLM intra prediction mode

…

(skipped)

…

- The variables avgY, avgC, minY, maxY, minC and maxC are derived as follows:

    - The variable minY is set equal to 1 << (BitDepthY) + 1 and the variable maxY is set equal to −1.

    - If availT is equal to TRUE, the variables avgY, avgC, minY, maxY, minC and maxC with x = 0..nS − 1 are derived as follows:

    - avgY += pTopDsY[ x * xS ]

    - avgC += p[ x * xS ]

    - If minY is greater than pTopDsY[ x * xS ], the following applies:

minY = pTopDsY[ x * xS ] 	(8-200)

minC = p[ x * xS ][ −1 ] 	(8-201)

    - If maxY is less than pTopDsY[ x * xS ], the following applies:

maxY = pTopDsY[ x * xS ] 	(8-202)

maxC = p[ x * xS ][ −1 ] 	(8-203)

    - If availL is equal to TRUE, the variables avgY, avgC, minY, maxY, minC and maxC with y = 0..nS − 1 are derived as follows:

    - avgY += pLeftDsY[ y * yS ]

    - avgC += p[ −1 ][ y * yS ]

    - If minY is greater than pLeftDsY[ y * yS ], the following applies:

minY = pLeftDsY[ y * yS ] 	(8-204)

minC = p[ −1 ][ y * yS ] 	(8-205)

    - If maxY is less than pLeftDsY[ y * yS ], the following applies:

maxY = pLeftDsY[ y * yS ] 	(8-206)

maxC = p[ −1 ][ y * yS ] 	(8-207)

8. The variables a, b, and k are derived as follows:

    - If numSampL is equal to 0, and numSampT is equal to 0, the following applies:

k = 0		(8-208)

a = 0		(8-209)

b = 1 << ( BitDepthC − 1) 		(8-210)

    - Otherwise, the following applies:

    - If both availT and availL are equal to TRUE, the following applies:

countShift = Log2( nS ) + 1

    - Otherwise,

countShift = Log2( nS )

    - The variables aveY and aveC are derived as follows:

aveY = (aveY + (1 << (countShift - 1))) >> countShift

aveC = (aveC + (1 << (countShift - 1))) >> countShift

diff = maxY − minY		(8-211)

    - If diff is not equal to 0, the following applies:

diffC = maxC − minC	(8-212)

x = Floor( Log2( diff ) )	(8-213)

normDiff = ( ( diff << 4 ) >> x ) & 15	(8-214)

x += ( normDiff  !=  0 )  ?  1  :  0	(8-215)

y = Floor( Log2( Abs ( diffC ) ) ) + 1	(8-216)

a = ( diffC * ( divSigTable[ normDiff ] | 8 ) + 2y − 1 ) >> y	(8-217)

k = ( ( 3 + x − y ) < 1 )  ?  1  :  3 + x − y	(8-218)

a = ( ( 3 + x − y ) < 1 )  ?  Sign( a ) * 15  :  a	(8-219)

b = aveC − ( ( a * aveY ) >> k )	(8-220)

where divSigTable[ ] is specified as follows:

divSigTable[ ] = { 0, 7, 6, 5, 5, 4, 4, 3, 3, 2, 2, 1, 1, 1, 1, 0 }	(8-221)

    - Otherwise (diff is equal to 0), the following applies:

k = 0	(8-222)

a = 0	(8-223)

b = aveC	(8-224)