| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0525 v1 |
| --- | --- |

| Title: | CE4-related: CE4-4.5 pruning reduction in TPM using regular merge candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Robert, T. Poirier, F. Le Léannec, F. Galpin | Email: | antoine.robert@technicolor.com tangi.poirier@technicolor.com fabrice.leleannec@technicolor.com franck.galpin@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution describes two simpler motion predictor pruning processes to be applied during triangle merge list construction. In the first proposed version, pruning of uni-directional and averaged candidates is removed. In the second, more drastic, version, all pruning are removed. These two pruning reductions can then be applied on two sets of initial candidates, one based on the existing initial candidates where the spatial candidates are pruned as in regular merge mode and the temporal candidates are pruned together as in JVET-N0226, or one based on the regular merge spatial and temporal candidates.

The proposed processes have been implemented in the JVET VTM-4.0. Based on the regular merge candidates, the reported simulations, with the first version, keep the same BD-rate performances in RA and show an average luma BD-rate loss of 0.07% in LDB configuration, and, with the second version, show an average luma BD-rate loss of 0.02% in RA and of 0.17% in LDB configuration. The encoding and decoding times stay identical to the VTM-4.0 ones with both versions. The worth case complexity in terms of integer comparisons required by the pruning is decreased from 282 to 180 comparisons by using the regular merge candidates (-36%) then to 120 comparisons for the first version of pruning reduction (-57%) or to only 30 comparisons for the second version (-89%).
Compared to the existing initial candidates in JVET-N0226, regular merge candidates slightly improve the BD-rate performances in RA with both versions of pruning reduction (-0.02%) but decrease the BD-rate performances in LDB by 0.05% with first version of pruning reduction and 0.03% with the second version. However, the regular merge candidates involve less integer comparisons for the pruning than existing initial candidates. It is decreased from 282 comparisons to 120 (-57%) instead of 174 (-38%) for the first version and to only 30 (-89%) instead of 36 (-87%) for the second version.

# Introduction

In the current triangle merge motion predictor lists’ constructions, it is not possible to encounter the same predictor several times since a full and redundant pruning process is applied.

This contribution along with the response to CE4-4.5 [1] of JVET-N0226 [2] propose some simpler motion predictor pruning processes for the triangle merge mode that allow keeping low the redundancy in the lists of predictors.

# Description

In triangle merge mode, a motion predictor is one motion vector with its associated reference picture, and a list must contain 5 motion predictors.

The motion predictors are uni-directional parts picked in an initial list of candidates composed of spatial and temporal candidates. A full pruning process is applied when collecting the initial spatial candidates. Then, when constructing the triangle motion predictor list, a full pruning is also performed where motion vectors and POC of reference frames are compared.

This contribution then proposes, as in JVET-N0226, to:

- Remove the pruning of uni-directional candidates since the pre-list is already (partially) pruned,

- Remove the pruning of averaged candidates.

In a second, more drastic, version where all pruning of the triangle motion predictor list construction are removed, it is, in addition, proposed to:

- Remove the pruning of L0 parts of bi-directional candidates,

- Remove the pruning of L1 parts of bi-directional candidates.

These pruning reductions can be applied on two distinct sets of initial candidates:

- Based on the existing initial candidates as described in JVET-N0226 [2], or

- Based on the regular merge candidates until the temporal candidate, i.e. only spatial and temporal candidates of the regular merge list are considered.

Figure 1 represents the candidates considered during the triangle merge list creation based on the regular merge candidates. The worth case complexity analysis is also provided where the number of motion vectors and associated reference frame indexes involved in the pruning of motion predictors are counted to get the total number of integer comparisons.

The associated worth case complexity in terms of integer comparisons is reduced by:

- 102 comparisons and goes from 282 to 180 comparisons, using regular merge candidates (-36%),

- 162 comparisons and goes from 282 to 120 comparisons, for the first version (-57%),

- 252 comparisons and goes from 282 to only 30 comparisons, for the second version (-89%).

Figure 1: Proposed triangle merge list construction based on regular merge candidates (and complexity in comparisons)
[black: VTM-4.0 – blue: initial candidates – orange: proposed modification – green: for both lists]

| Merge list size | Max number of potential candidates | Max number of candidate comparison | Max number of MV scaling | Max number of temporal candidates | Max number of MV average | Max number of encoder MCs | Others |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 26 (-2) | v1: 5+30 (-5-44) v2: 5+0 (-5-74) | 2+5 (-2-2) | 1 (-1) | 5 (-2) | 5 (+0) | / |

# Simulation results

The simulations are conducted using the VTM configuration as described in JVET-M1010 [3]. VTM-4.0 reference software is used with VTM configuration files.

## First pruning reduction with regular merge candidates (CE4-4.5.c)

Table 1 and Table 2 show the simulation results of the first version of the proposed pruning reduction in triangle merge mode based on the regular merge candidates (spatial and temporal candidates) in random access and low delay B configurations. The proposed modifications allow keeping the BD-rate performances in random access and decrease the BD-rate performances in low delay B (0.07%) with the same complexity in encoding/decoding time and with the number of integer comparisons requested by pruning reduced by nearly 60% (120 comparisons instead of 282).

Table 1: Simulation results of the first proposed pruning reduction in the triangle merge mode
based on the regular merge candidates in random access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.08% | -0.04% | 100% | 100% |
| Class A2 | -0.05% | 0.01% | -0.05% | 100% | 100% |
| Class B | -0.01% | 0.00% | -0.06% | 100% | 100% |
| Class C | 0.00% | -0.12% | -0.07% | 101% | 102% |
| Overall | -0.01% | -0.04% | -0.06% | 100% | 101% |
| Class D | 0.01% | 0.03% | -0.11% | 100% | 101% |
| Class F (optional) | 0.00% | 0.04% | 0.01% | 100% | 100% |

Table 2: Simulation results of the first proposed pruning reduction in the triangle merge mode
based on the regular merge candidates in low delay B

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 0.04% | 0.04% | -0.18% | 100% | 99% |
| Class C | 0.07% | 0.36% | 0.35% | 100% | 101% |
| Class E | 0.12% | 0.33% | -0.87% | 101% | 101% |
| Overall | 0.07% | 0.22% | -0.17% | 100% | 100% |
| Class D | 0.07% | -0.07% | -0.66% | 100% | 101% |
| Class F (optional) | -0.12% | 0.08% | 0.68% | 100% | 101% |

## Second pruning reduction with regular merge candidates (CE4-4.5.d)

Table 3 and Table 4 show the simulation results in random access and low delay B configurations of the second version of the proposed pruning reduction in triangle merge mode based on the regular merge candidates (spatial and temporal candidates). The proposed pruning reduction slightly decreases the BD-rate performances in random access (0.02%) and even more in low delay B (0.17%) with the same complexity in encoding/decoding time but with the number of integer comparisons requested by pruning nearly removed (30 comparisons instead of 282).

Table 3: Simulation results of the second proposed pruning reduction in the triangle merge mode
based on the regular merge candidates in random access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.02% | 0.05% | 100% | 100% |
| Class A2 | -0.03% | -0.03% | 0.03% | 101% | 100% |
| Class B | 0.02% | -0.07% | 0.02% | 101% | 101% |
| Class C | 0.04% | 0.01% | -0.11% | 101% | 101% |
| Overall | 0.02% | -0.03% | 0.00% | 100% | 101% |
| Class D | 0.03% | -0.08% | -0.08% | 101% | 101% |
| Class F (optional) | 0.05% | 0.12% | 0.08% | 100% | 99% |

Table 4: Simulation results of the second proposed pruning reduction in the triangle merge mode
based on the regular merge candidates in low delay B

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 0.13% | 0.04% | -0.03% | 100% | 100% |
| Class C | 0.19% | 0.49% | 0.58% | 101% | 101% |
| Class E | 0.22% | 0.86% | -0.39% | 101% | 101% |
| Overall | 0.17% | 0.39% | 0.09% | 101% | 100% |
| Class D | 0.21% | 0.16% | 0.08% | 101% | 101% |
| Class F (optional) | 0.03% | -0.11% | -0.53% | 100% | 100% |

## Comparison between initial existing and regular merge candidates

Table 6, Table 7 and Table 6 show the comparison results between initial existing (JVET-N0226) and regular merge candidates with the first version of the proposed pruning reduction in random access and low delay B configurations. The regular merge candidates have slightly better BD-rate performances in random access (-0.02%) but decrease the BD-rate performances in low delay B (0.05%). The complexity in encoding/decoding time stays identical. But, the regular merge candidates reduce more the number of integer comparisons requested by pruning (120 (-57%) instead of 174 (-38%)).

Table 5: Simulation results of the first proposed pruning reduction with both initial candidates in random access

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  | Nb int comp reduction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  | Over VTM-4.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |  |
| Overall CE4-4.5.a | 0.01% | -0.01% | 0.01% | 100% | 100% | 0.02% | 0.19% | -0.15% | 101% | 100% | -38% |
| Overall CE4-4.5.c | -0.01% | -0.04% | -0.06% | 100% | 101% | 0.07% | 0.22% | -0.17% | 100% | 100% | -57% |
| Diff | -0.02% | -0.04% | -0.07% | 100% | 101% | 0.05% | 0.03% | -0.02% | 100% | 100% |  |

Table 6: Comparison results between initial existing (JVET-N0226 CE4-4.5.a) and regular merge candidates (CE4-4.5.c)
of the first proposed pruning reduction in the triangle merge mode in random access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4-4.5.a |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.18% | -0.03% | 99% | 100% |
| Class A2 | -0.05% | 0.05% | -0.08% | 100% | 101% |
| Class B | -0.02% | -0.01% | -0.11% | 100% | 100% |
| Class C | -0.02% | -0.04% | -0.03% | 100% | 101% |
| Overall | -0.02% | -0.04% | -0.07% | 100% | 101% |
| Class D | -0.02% | 0.09% | -0.13% | 100% | 102% |
| Class F (optional) | -0.01% | 0.03% | 0.04% | 100% | 100% |

Table 7: Comparison results between initial existing (JVET-N0226 CE4-4.5.a) and regular merge candidates (CE4-4.5.c)
of the first proposed pruning reduction in the triangle merge mode in low delay B

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4-4.5.a |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 0.02% | 0.13% | 0.05% | 99% | 99% |
| Class C | 0.06% | -0.04% | 0.25% | 100% | 100% |
| Class E | 0.10% | -0.06% | -0.52% | 100% | 102% |
| Overall | 0.05% | 0.03% | -0.02% | 100% | 100% |
| Class D | 0.02% | 0.28% | 0.30% | 100% | 102% |
| Class F (optional) | -0.06% | -0.25% | 0.01% | 100% | 99% |

Table 8, Table 9 and Table 10 show the comparison results between initial existing (JVET-N0226) and regular merge candidates with the second version of the proposed pruning reduction in random access and low delay B configurations. Both initial candidates have the same BD-rate performances in random access (-0.02% for regular merge candidates), but the regular merge candidates decrease the BD-rate performances in low delay B (0.03%). The complexity in encoding/decoding time stays identical. And, the regular merge candidates slightly reduce the number of integer comparisons requested by pruning (30 (-89%) instead of 36 (-87%)).

Table 8: Simulation results of the second proposed pruning reduction with both initial candidates in random access

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  | Nb int comp reduction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  | Over VTM-4.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |  |
| Overall CE4-4.5.b | 0.03% | 0.01% | 0.02% | 100% | 100% | 0.14% | 0.37% | -0.04% | 101% | 101% | -87% |
| Overall CE4-4.5.d | 0.02% | -0.03% | 0.00% | 100% | 101% | 0.17% | 0.39% | 0.09% | 101% | 100% | -89% |
| Diff | -0.02% | -0.04% | -0.02% | 100% | 101% | 0.03% | 0.02% | 0.12% | 100% | 100% |  |

Table 9: Comparison results between initial existing (JVET-N0226 CE4-4.5.b) and regular merge candidates (CE4-4.5.d)
of the second proposed pruning reduction in the triangle merge mode in random access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4-4.5.b |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.04% | -0.02% | 100% | 100% |
| Class A2 | -0.06% | 0.03% | -0.01% | 101% | 100% |
| Class B | -0.01% | -0.05% | 0.07% | 100% | 100% |
| Class C | -0.01% | -0.09% | -0.14% | 101% | 102% |
| Overall | -0.02% | -0.04% | -0.02% | 100% | 101% |
| Class D | -0.01% | -0.09% | -0.03% | 101% | 101% |
| Class F (optional) | 0.00% | 0.01% | 0.04% | 100% | 99% |

Table 10: Comparison results between initial existing (JVET-N0226 CE4-4.5.b) and regular merge candidates (CE4-4.5.d)
of the second proposed pruning reduction in the triangle merge mode in low delay B

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4-4.5.b |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 0.02% | -0.11% | 0.13% | 100% | 99% |
| Class C | 0.04% | 0.08% | 0.28% | 100% | 100% |
| Class E | 0.03% | 0.13% | -0.11% | 100% | 100% |
| Overall | 0.03% | 0.02% | 0.12% | 100% | 100% |
| Class D | 0.08% | -0.21% | -0.04% | 100% | 102% |
| Class F (optional) | 0.04% | -0.52% | -1.00% | 101% | 99% |

# Conclusion

This contribution describes two simpler motion predictor pruning processes to be applied during triangle merge list construction. In the first proposed version, pruning of uni-directional and averaged candidates is removed. In the second, more drastic, version, all pruning are removed. These pruning reductions can then be applied on two sets of initial candidates, one based on the existing initial candidates where the spatial candidates are pruned as in regular merge mode and the temporal candidates are pruned together (JVET-N0226), or one based on the regular merge spatial and temporal candidates.

For both versions of pruning reduction based on both sets of initial candidates, the BD-rate performances remain close to those of the VTM-4.0 in RA configuration (from -0.01% to 0.03%).
In LDB configuration, only the first version of pruning reduction based on initial existing candidates allows keeping the BD-rate performances (0.02%), with the second version, the losses are of 0.14%. And, based on the regular merge candidates, losses become 0.07% with the first version of pruning reduction and 0.17% with the second version.
The regular merge candidates compared to the existing initial candidates of JVET-N0226 slightly improve the BD-rate performances in RA with both versions of pruning reduction (-0.02%) but decrease the BD-rate performances in LDB by 0.05% with first version of pruning reduction and 0.03% with the second version. However, the regular merge candidates involve less integer comparisons for the pruning than existing initial candidates. It is decreased from 282 comparisons to 120 (-57%) instead of 174 (-38%) for the first version and to only 30 (-89%) instead of 36 (-87%) for the second version.

# References

[1] H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-M1024, 13th Meeting, Marrakesh, MO, January 2019.

[2] A. Robert, T. Poirier, F. Le Léannec, F. Galpin, “CE4-4.5: Motion predictor pruning reduction in Triangle Merge mode”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-N0226, 14th Meeting, Geneva, CH, March 2019.

[3] F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-M1010, 13th Meeting, Marrakesh, MO, January 2019.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).