| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0528 |
| --- | --- |

| Title: | AHG12/AHG19/SYS-VVC: Sub-picture decoding design: a comparison of proposed solutions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Emmanuel Thomas Anna van Buerenplein 1 2596 DA The Hague The Netherlands | Tel: Email: | +31 (0)88 866 42 61 emmanuel.thomas@tno.nl |
| Source: | TNO |  |  |

_____________________________

# Abstract

This contribution compares two proposed approaches to support decoding of independent coded regions which are 1) using tile groups as independent regions or 2) using a layered mechanism where each layer consists of a sub-picture. This contribution does not discuss the syntax details of any of these approaches but rather the technical merits of both approaches in addressing the need of real-world immersive applications.

It is asserted that the layered mechanism offers decisive advantages over the tile group-based approach when considering real-word applications. The simplicity of the layered approach which does not require inspection nor rewriting of parameters in the payload of VCL NAL units constitutes a significant advantage for the success of VCC in immersive application space.

Therefore, it is recommended to adopt the layered mechanism for supporting the decoding of independent regions. In addition, it is suggested to document at this meeting the use of this new mechanism in the context of viewport-based 360 video streaming which is currently an application considered by MPEG experts working on “Requirements on Immersive Media Delivery and Access” N18134.

# Introduction

At JVET 13th, experts have discussed the need of immersive applications such as 360 video streaming in terms of supporting the decoding of independent coded regions. These discussions were also motivated by the work carried out in MPEG on decoding interface for immersive applications. MPEG published the latest draft requirements under the document N18134 “Draft Requirements for Immersive Media Access and Delivery” [1].

This contribution discusses two fundamental design to support the decoding of independent coded picture regions. The first design is to extract within each picture the independent coded regions and generate the corresponding output picture. The second design is to operate outside of the picture concept and to extract sub-bitstreams based on a layered mechanism. However, this contribution does not aim at discussing the syntax details of both approaches but rather encourages the discussion on the merits of both approaches with respects to the intended goal. i.e. enabling immersive applications to leverage VCC features as easily as possible for decoding sub-pictures over time.

From the AHG12 High-level parallelism and coded picture regions (AHG12), this contribution addresses the following mandate:

- Study support of independently coded picture regions, including easy rewriting of such regions into a conforming sub-bitstream.

From the AHG19 Layered coding and resolution adaptivity (AHG19), this contribution addresses the flowing mandates:

- Study adaptive-resolution coding approaches for real-time communication, adaptive streaming, and 360-degree viewport-dependent streaming, including filters for resampling, reference picture management, and related scope and signaling

- Study potential approaches for support of layered coding scalability including spatial, temporal, quality, and view scalability

The contribution refrains from proposing technical syntax but recommends to first decide which approach will be chosen to support sub-picture decoding, especially in the context of immersive application as presented in N18134. In that regards, the analysis done shows that a solution based on a layering mechanism is more advantageous over a tile group-based approach.

Therefore, we recommend the adoption of a layered mechanism to support sub-picture mechanism and document the use of this new mechanism in the context of immersive applications in a spirit of collaboration with the MPEG experts working on the topic.

# Background - Navigation in panoramic and omnidirectional content

There are a several applications where the spatial subdivision of a large video content can be useful. Examples include applications when the user may want to only watch one area of the full video and/or pan. In this case, streaming and/or decoding the whole frame only to display a sub part of it is inneficient. It is desirable to be able to access only a region of the video.

VR 360 applications do not render the whole video but only a small part of it corresponding to the user's viewport. Figure 1 shows an example of how a cube map video tiled in 24 independent coded regions. Figure 2 shows a system that save bandwidth when streaming: when a user is looking in a certain direction then only the coded regions needed to render the user's viewport are decoded. In this case, this would allow for approximately 50% saving in transmitted bitrate compared to sending the whole bitstream. Note that this tilling of cube map video is merely an example and any other tiling scheme could be used to describe the VR application case.

Figure 1 – 360 video partitioned into 24 independent coded regions (red)

Figure 2 - Viewport-based selection of coded regions

# Problem Statement

## Extracting and rearranging independent coded regions

In the context of 360 video, applications would follow the following actions to implement a viewport-based decoding of the omnidirectional content as depicted in Figure 3. The first step is to select relevant parts of the content that suits the viewer needs. This application logic can be simple to very complex to make an optimised selection with respect to quality of experience under current resource constraints. The innovation in that domain is still very active, especially in the accuracy for predicting the viewport in the future. When the selection is made, the application then extracts the needed coded regions from the whole content. Very often, the resolution of the source content is 8K or higher which forces the application to reshape the selected content to make it fit the decoding capacity of the device. Even though 8K decoders may be expected in the future, this is a race the decoders cannot win as capture devices also increase their acquiring resolution and 16K resolution source content is not unthinkable for the near future. In this example, the application reshapes the 6x4 layout into a 6x2. Any other layouts of 12 coded region that fits the decoding capacity of the device would also be possible. At this stage, it is important to note that the application stores the specific layout it has chosen before feeding the bitstream to the decoder. This way, the samples of the output picture can be correctly mapped onto to the project frame by the GPU. Note that the GPU may not necessarily regenerate a picture in the original resolution but what is essential is that every sample of the output picture can be map onto a texture related to the spherical domain. This constitute the final stage of the pipeline. Next, new head motion sensor data are fed back to the beginning of the loop.

Figure 3 - Process of extracting coded regions for viewport-based decoding/streaming

## Time-locking the independent coded regions

The original content has been produced in such a way that every sample of a picture of the omnidirectional video have been captured at the same point in time (or with a neglectable difference). This property must be kept all the way to the rendered viewport. Otherwise, the user will notice edge artefact if some faces of the cube do not all come from the same original picture.

Figure 4 - Risk of incoherent textures with unsynchronised decoding processes

To this end, instructing the decoder to form a single output picture out of the selected coded region ensures the transmission of this property up to the output of the decoder and no further synchronisation by the application is needed.

# Possible approaches

Over the past meetings, there has been two fundamental approaches that have been discussed:

- Approach A: Independent coded regions within pictures to form the output picture

- Approach B: Independent coded pictures to form the output picture

## Approach A: Independent coded regions within pictures to form the output picture

In this approach, the idea is similar to HEVC MCTS. The tile groups are encoded in such a way they constitute independent coded regions over time. One measure is to constrain the motion vectors to the same tile groups over successive pictures similarly to HEVC MCTS.

Essentially, this option would require the inspection and possibly the rewriting of parameters inside the payload of NAL units and some case the data inside tile group payloads. The current VVC draft defines for instance the tile_group_id in the PPS and the tile_group_address the tile group data syntax element. The current definition of these elements is that their respective lengths are variable and dependent on other syntax elements. This means that any parsing or rewriting of these elements would require the localisation and the parsing of these dependent parameters. Even if improvements could be provided to the coding of these parameters, the position in bytes inside the NAL units of these parameters also variable since many other parameters with variable length and conditional presence are located before in the syntax.

Figure 5 visualize both problems of unpredictability of the byte position and length of relevant parameters for the application.

Figure 5 - Unpredictable byte position and length of relevant parameters in tile group-based approach

More generally, the fundamental issue with this option is that these useful parameters for the application are located inside the payload of NAL units and even worse in the payload of VCL NAL units. Real-word application are traditionally built around APIs and interfaces. Implementing an application that stretches from the retrieval of the content to the deep machinery of the core video codec syntax is a high burden for the implementers. It is believed that if such an option would be chosen, the sub-picture decoding features will remain in niche use cases at best or even not being used at all.

## Approach B: Independent coded pictures to form the output picture

In this approach, the different decoded pictures form a single output picture. Each of the coded picture would constitute an independent sub-bitstream similarly to layers in scalability design.

Several proposals have suggested either VPS or other non-VCL NAL unit for signaling the presence and the properties of these sub-picture layers. Currently, the VVC draft does not offer VPS syntax but it can be expected that either the layer information will be high up and at a predictable position in the VPS or that a completely dedicated new non-VCL NAL units will be created for signaling the layer information. In both cases, this makes the only introspection and possible rewriting operation limited to non-VCL NAL units which is an acceptable effort for the applications as it is already commonly done.

Figure 6 - Isolation in non-VCL NAL units of relevant parameters in the layered mechanism approach

## Comparison table of both approaches

|  | Tile Group (Approach A) | Layered sub-bitstream (Approach B) |
| --- | --- | --- |
| Relevant parameters location | In tile group header | In VPS or other Non-VCL NALU |
| Predictable position and length of parameter | No | Yes |
| Independent coded region | One or more tile group | One layer |
| Merging operation | Combine tile groups in successive pictures | Combine sub-bitstream of layers into one bitstream |
| Extracting operation | Extracting tile groups from successive pictures | Extracting the desired layers |
| Time-locking of regions | By inserting the coded region in the same coded picture | By outputting the decoded sub-picture layers in the same output picture |
| Application friendliness | Low | High |

# Proposal: Adopting the layered sub-bitstream mechanism since being the most application-friendly approach

Considering this comparison, it is recommended to adopt a layered mechanism for supporting sub-picture decoding.

As mentioned in introduction, this contribution does not propose any syntax details, but contributions have been submitted at the present and will be refined in the future meetings.

It is also proposed to document the use of the new mechanism in the context of viewport-dependent 360 video decoding in the spirit of collaboration with MPEG experts working on the “Requirements on Immersive Media Delivery and Access” N18134.

# References

- ISO/IEC JTC1/SC29/WG11 N18134, “Draft Requirements on Immersive Media Delivery and Access”, Emmanuel Thomas (TNO), Thomas Stockhammer (Qualcomm), Rob Koenen (Tiledmedia), January 2019, Marrakech, MA

# Patent rights declaration(s)

TNO may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).