| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: | Document: JVET-N0532r2 |
| --- | --- |

| Title: | AHG19: Resampling filters for adaptive resolution change (ARC) in VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Pankaj Topiwala Madhu P Krishnan Wei Dai | Email: | pankajtva@gmail.com madhupk83@gmail.com daisyagile@gmail.com |
| Source: | FastVDO LLC |  |  |

_____________________________

# Abstract

ARC has received increased attention this meeting cycle, and numerous documents consider ARC for VVC. While the signaling of ARC usage is relatively straightforward, this contribution focuses on the resampling filters used for the resolution adaptation. For comparison purposes, we use pair of a simple, zero-phase separable 5-tap downsampling filter and an 8-tap upscaling filter (the upsampler already part of the VVC design) for resolution change, which we refer to as Anchor, and believe were suggested in JVET-M0135. Following the same spirit, we introduce a 9-tap downsampling filter (complexity of 7-tap), and a very simple 4-tap upsampling filter, which is also part of the existing design of VVC. Gains of about 0.4dB in PSNR are reported without encoding, when compared to the anchor filters, on a subset of test data. An average BD-rate gain of about 2.35% is reported for the luma(Y) channel with compression. Further results and visual evidence will be presented at the meeting.

# Introduction

The concept of ARC was originally part of H.263 Annex P [2] , and was introduced in concept for HEVC by JCTVC-F158 [3]; the acronym “ARC” itself, which at the time stood for Adaptive Resolution Coding (where “Coding” is now “Change”), was actually invented by us in the joint paper [8]. Numerous methods can be used to signal the use of ARC in a bitstream, for which there are a number of contributions. JCTVC-F158 discussed the utility of adapting resolution within a single stream without introducing an IDR frame. The document JVET-M0135 cites that contribution and uses a zero phase 5-tap downsampling filter, and suggests for upsampling the half-pel motion compensation interpolation filter coefficients, which we believe is described in VVC WD [4] Table 8-9 for scaling operations.  In this contribution FastVDO introduces a zero phase 9-tap downsampling filter and a 4-tap upsampling filter, which is also listed in VVC WD Table 8-6: luma prediction interpolation filter, and compare it with the JVET-M0135 filters.

## Description of filters

JVET-M0135 lists the following filters for scaling by a factor of 2. In fact, the downsampling filter is explicitly given, but for the upsampling, they refer to “additional samples at odd grid positions are generated using the half-pel motion compensation interpolation filter coefficients in the latest VVC WD.” While that is descriptive though not very explicit, we infer (perhaps inaccurately) that this refers to the 0-phase filter in Table 8-9, which is for 1/16th pel accuracy motion compensation luma interpolation, given below. In any event, for definiteness, we compare our results with that obtained with the filters below as anchor.

Anchor:

- Downsampling: [ −1  9  16  9  −1 ]/ 32; which is a 5-tap zero phase filter

- Upsampling: [-1  4  -11  40  40  -11  4  -1]/64;  an 8-tap filter from Table 8-9 in VVC WD

This appears to have a modest downsampler, but an expensive upsampler. By contrast, FastVDO will use a pair of filters in which the downsampler is slightly more complicated, but the upsampler is only 4-tap.

FastVDO:

- Downsampling: [1 0 -8 16 46 16 -8 0 1] / 64    (complexity of 7-tap)

- Upsampling:  [ -1  9  9 -1 ] / 16.

First note that the 9-tap downsampler, due to the two zeros, really has the complexity of a 7-tap. Further, note that the 4-tap upsampling filter [-1 9 9 -1] / 16 is in fact the 0-phase filter in VVC WD Table 8-6. Also note that this filter is quite close to the simplest possible interpolator: the two-tap filter [1 1]/2. A tantalizing question, not studied yet, is whether we can get good results even with the simplest possible upsampler.  Importantly, note that since both sets of filters use upsamplers already in the draft specification, this is in part a comparison of downsamplers. See references [5-8] for more discussion.

## Results

One second clips from seven sequences (from the JVET test sequences list) were used for the filter study. This will actually suffice since in fact we are using All-Intra coding, and the gain are seen at every frame, as will be shown. Each of the sequences were downsampled by a factor of 2 using the JVET-M0135 and our filters. We report results for both cases: no compression, and with compression, using All Intra coding. The downsampled sequences were then encoded at QP 22,27,32 and 37 using BMS 2.2 with the All Intra Main10 profile. The binaries were then decoded and upsampled to generate the reconstructed videos. PSNR comparisons were made for all sequences at the source resolution with the original sequences as reference. A study was also conducted to evaluate the performance of the filters without encoding. Table 1 describes the results of the filter study without encoding between the downscaling and upscaling process. An average gain of ~0.4dB can be observed for the luma (Y) with the use of our filters. Furthermore, the gains increase with resolution, up to ~0.8dB for one 4K 10-bit sequence (“ParkRunning”). Table 2 plots the frame by frame gain for 1 second of the BasketballDrive sequence without compression. As described above the gain is consistent. Tables 3 and 4 do the same for encoding at Qp 22. Table 5 describes the results of the filter study with encoding with QP=22,27,32 & 37 introduced between the downscaling and upscaling process. An average BD-rate gain of ~2.3% can be observed for the luma (Y) with the use of our filters.  As an example result, Table 6 describes the Y-PSNR(dB) vs bitrate (KBps) plot for 1 second of the BasketballDrive sequence with compression. A consistent gain is observed for all bitrates. This is reassuring, in that after the initial loss due to resampling, the gain due to the better filters is essentially retained intact under compression. More extensive results and visuals will be presented at meeting.

Technicolor has kindly cross-checked these results in JVET-0669, for 1s clips at no encoding and one Qp setting, corresponding to Tables 1 – 4.

Table 1. Filter comparison without encoding

Table 2. Frame by frame gain (dB) for BasketballDrive sequence without compression

Table 3. Filter comparison with encoding

Table 4. Frame by frame gain (dB) for BasketballDrive sequence with compression

Table 5. BD-rate gain with encoding at 4 Qp values.

Table 4. Y-PSNR(dB) vs bitrate(KBps) plot for BasketballDrive sequence with compression

# Patent rights declaration(s)

FastVDO may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-M0135-v1, “On adaptive resolution change (ARC) for VVC”, Hendry, T. Davies, Y. Sun et al. , 13th Meeting: Marrakech, MA, 9–18 Jan. 2019

[2] ITU-T H.263 (01/2005): Video coding for low bit rate communication, Annex O “Temporal, SNR and Spatial Scalability mode” and Annex P “Reference Picture Resampling”

[3] JCTVC-F158, “Resolution switching for coding efficiency and resilience”, T. Davies (Cisco), JCTVC 6th Meeting, Turin, IT, 14-22 July 2011

[4] JVET-M1001v5, “Versatile Video Coding(Draft 4)”, B. Bross, J. Chen, S. Liu, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019

[5] JCTVC-F618, “Advanced Resampling Filters for Spatial Scalability and Chroma Sampling Applications in HEVC”, W. Dai, M. Krihnan, P. Topiwala (FastVDO LLC), JCTVC 6th Meeting, Turin, IT, 14-22 July 2011

[6] JVT-V030, “Advanced Extended Spatial Re-sampling Filters For SVC”, Trac D. Tran, L. Liu, P. Topiwala (FastVDO), JVT 22nd Meeting: Marrakech, Morocco, 13–19 January, 2007

[7] JCTVC-G862, “Advanced Resampling Filters for HEVC Applications”, W. Dai, M.Krihnan, P. Topiwala (FastVDO), JCT-VC 7th Meeting, Geneva, CH,  21-30 November, 2011.

[8] JCTVC-G264, “AHG18: Adaptive Resolution Coding,” T. Davies (Cisco), P. Topiwala (FastVDO), JCT-VC 7th Meeting, Geneva, CH,  21-30 November, 2011.