| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0548 |
| --- | --- |

| Title: | CE9-related: Additional experimental results of JVET-N0187 (CE9-2.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Kenji Kondo, Masaru Ikeda, Teruhiko Suzuki 2-10-1 Osaki, Shinagawa-ku, Tokyo Japan | Tel: Email: | +81-50-3750-2744 kenji.kondo@sony.com, Masaru.Ikeda@sony.com, teruhiko.s@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

In this contribution, the additional experiments are reported which relate to CE9-2.2. It’s introduced that the implementation is modified to get decoder speed up, and this modification is only software implementation and non-normative change. The decoding time was measured using the same bit streams as CE9-2.2. The decoding time is reported that 99% and 99% for CE9-2.2a and CE9-2.2b. For coding efficiency, it’s the same as JVET-N0187

Introduction

The flowchart of implementation in JVET-N0187 [1] is shown in Figure 1. The early termination is executed for 16x16 and 4x4 level at prediction process.

Figure 1: An implementation for early termination

In previous implementation, the gradient filter and finding optical flow parameters are always executed. In this contribution, the implementation slightly changes as shown in Figure 2

Figure 2: Modified flow chart

In Figure 2, when BDOF size is 16x16 and the SAD value is smaller than threshold, the early termination is executed before gradient filter; also the gradient filter and finding optical flow parameters are skipped.

Software modification

Above speed up method is implemented on CE9-2.2 branch current HEAD (fdbb1882ee). The difference of software is shown in Table 1, 7 lines are added.

Table 1: Difference between CE9-2.2 and modification

$ git diff fdbb1882ee diff --git a/source/Lib/CommonLib/InterPrediction.cpp b/source/Lib/CommonLib/InterPrediction.cpp index 04000bb..2d02c3d 100644 --- a/source/Lib/CommonLib/InterPrediction.cpp +++ b/source/Lib/CommonLib/InterPrediction.cpp @@ -1369,7 +1369,13 @@ bool InterPrediction::xCalcBiPredSubBlkDist(const PredictionUnit &pu, const Pel* m_bdofEt1Flag[etBlockIdx] = m_bdofEt1Dist[etBlockIdx] < m_bdofEt1DistThres; } } +#if CE9_2_2_ET16x16_SPEEDUP + if (width==16&&height==16) + { + return (m_bdofEt1Flag[0]==false); + } return true; +#endif #elif CE9_2_2_ET1ST_DISABLE return true; #else diff --git a/source/Lib/CommonLib/TypeDef.h b/source/Lib/CommonLib/TypeDef.h index c13b8e8..74a1b58 100644 --- a/source/Lib/CommonLib/TypeDef.h +++ b/source/Lib/CommonLib/TypeDef.h @@ -56,6 +56,7 @@ #define CE9_2_2_ET1ST_DISABLE 0 #endif #define CE9_2_2_ET4x4DISABLE 0 +#define CE9_2_2_ET16x16_SPEEDUP 1 #define JVET_M0055_DEBUG_CTU 1 // DebugCTU encoder debug option

Test cases

As described in [2], three cases are tested in the CE for CE9-2.2.

| # | description | Tester | Crosschecker |
| --- | --- | --- | --- |
| CE9-2.2a | The SAD region is divided for each 16x16 sub-blocks for 1st stage of early termination. | K. Kondo | T. Chujoh |
| CE9-2.2b | The SAD region is divided for each 16x16 sub-blocks for 1st stage of early termination. And an early termination for 2nd stage (4x4 block level) is disabled. | K. Kondo | T. Chujoh |
| CE9-2.2c | The early termination for 1st stage (CU level) is disabled. | K. Kondo | T. Chujoh |

Above implementation modification for speed up can be applied for CE9-2.2a and CE9-2.2b. In this report, the decoding run time is measured for those 2 tests using the modified software.

Experiment

As introduced above, three tests carried out. JVET common test condition [3] is followed. For the simulations, same platform machines as below in PC cluster were used.

- Encode

  - OS: CentOS 6.10

  - Compiler: g++ (GCC) 6.3.1

  - CPU: Intel Core i7-7700K 4.2GHz

    - SIMD: SSE42.

  - Memory: 32 GB

- Decode

  - OS: CentOS 6.10

  - Compiler: g++ (GCC) 6.3.1

  - CPU: Intel Core i5-2500K 3.3 GHz

    - SIMD: SSE42

  - Memory: 24 GB

For a runtime measurement, the user time in the output log was used. For a random-access case and an all intra case, a parallel encoding is used. To measure encoding time in parallel encoding, the total of all user time for each frame were calculated.

Confirmation for YUV file

The introduced modification is only software implementation. To check the software, it’s confirmed that the YUV files are the same in both implementations.

The summary of test results

Test results are shown in Table 1 and Table 2 for CE9-2.2a with the modified software and CE9-2.2b with the modified software. Since this test is just to check decoding time, the coding efficiency and encoding time is same as reports in JVET-N0187. The decoding time is updated.

Table 2 Test results for CE9-2.2a with modified software

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | -0.06% | 101% | 99% |
| Class A2 | 0.05% | -0.02% | -0.03% | 101% | 98% |
| Class B | 0.04% | 0.03% | 0.00% | 100% | 99% |
| Class C | 0.02% | -0.03% | -0.06% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -0.01% | -0.03% | 100% | 99% |
| Class D | 0.02% | 0.01% | -0.03% | 100% | 99% |
| Class F | 0.01% | -0.02% | -0.04% | 100% | 100% |

Table 3 Test results for CE9-2.2b with modified software

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.03% | 0.02% | 101% | 99% |
| Class A2 | 0.05% | 0.01% | 0.00% | 101% | 98% |
| Class B | 0.03% | 0.00% | -0.01% | 101% | 99% |
| Class C | 0.01% | -0.02% | -0.08% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.00% | -0.02% | 101% | 99% |
| Class D | 0.01% | -0.07% | -0.12% | 100% | 99% |
| Class F | 0.01% | -0.01% | -0.05% | 101% | 100% |

As shown in test results, if we compare with JVET-N0187, the decoding time gets faster about 2%.

Recommendation

Based on the experimental results, it is recommended that also results in this contribution are informed in CE9 discussion.

References

[1] Kenji Kondo, Masaru Ikeda, Teruhiko Suzuki, “CE9-2.2: On early termination of BDOF”, JVET-M0187, the 14th JVET meeting, Geneva, CH, March 2019.

[2] Semih Esenlik, Xiaoyu Xiu, “Description of Core Experiment 9 (CE9): Decoder Motion Vector Derivation”, JVET-M1029, the 13th JVET meeting, Marrakech, MA, Jan. 2019.

[3]	F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR”, Document: JVET-M0449

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).