| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0550 |
| --- | --- |

| Title: | CE8-related: Line-based CG Palette Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yung-Hsuan Chao, Ted Hsieh, Marta Karczewicz (Qualcomm) | Tel: Email: | yunghsua@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

# Abstract

In this proposal, a subblock based approach is proposed to simplify the palette mode in CE8-2.1. Similar to the coefficient group (CG) used in transform coefficient coding, a CU is divided into multiple segments, each consists of m samples, where index runs, palette index values, and quantized colors for escape mode are encoded/parsed. The results show only minor loss compared to CE8-2.1:  0.11% AI, 0.00% RA for class F and 0.29% AI, 0.16% RA for class TGM.

# Introduction

In the palette mode design in HEVC, the syntax element palette_index_idc and palette_escape_val are CABAC bypass coded. In order to improve CABAC throughput, syntax element palette_index_idc for the whole CU are grouped and parsed before the palette run coding, i.e., copy_above_palette_indices_flag and palette_run, which are CABAC context coded. The syntax element palette_escape_val for the whole CU, on the other hand, are grouped and parsed after palette run coding.  However, the buffer size needed to store the index values, index map, and escape colors can take up to the CU size in the worse case. In order to reduce the buffer size, a subblock based palette mode is proposed, which divided a CU into multiple segments with m samples, where index runs, palette index values, and quantized colors for escape mode are encoded/parsed. In addition, it is proposed to move the parsing of CABAC bypass coded syntax element, i.e., palette_index_idc, palette_escape_val, after palette run coding.

# Proposal

In this proposal, each CU of palette mode is divided into multiple segments of m samples (m=16 in this test) based on the traverse scan mode, as shown in Figure 1.

Figure 1: Sub-block-based index map scanning for palette

The encoding order for palette run coding in each segment is as follows: For each pixel, 1 context coded bins are signalled indicating if the pixel is of the same mode as the previous pixel, i.e., if the previous scanned pixel and the current pixel are both of run type COPY_ABOVE or if the previous scanned pixel and the current pixel are both of run type INDEX and the same index value. If the pixel and the previous pixel are of different mode, 1 context coded bin is signaled indicating the run type, i.e., INDEX or COPY_ABOVE, of the pixel. After palette run coding of pixels in one segment, the index values and escape modes, which are CABAC bypass coded, are parsed.

# Simulation results

The proposed method is evaluated for the intra-only (AI) and random access (RA) configurations on top of VTM4.0 and CE8-2.1. For class F and TGM sequences, HashME and IBC are enabled for both anchor and test. Palette mode is enabled (PLT=1) for test and disabled for anchor.

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  | Over CE8-2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class F | -2.28% | -2.18% | -2.21% | 105% | 110% | 0.11% | 0.03% | 0.08% | 105% | 108% |
| TGM | -6.59% | -5.49% | -5.12% | 97% | 99% | 0.29% | 0.19% | 0.19% | 106% | 106% |

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  | Over CE8-2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | c | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class F | -1.44% | -2.06% | -2.08% | 109% | 116% | 0.00% | 0.07% | 0.00% | 105% | 115% |
| TGM | -3.15% | -2.98% | -2.84% | 104% | 111% | 0.16% | 0.18% | 0.20% | 105% | 111% |

The encoding/decoding time in the results may not be accurate.

# Reference

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools”, JVET-M1028, 13th meeting, Marrakech, MA, January 2019.

- Y.-H. Chao, H. Wang, V. Seregin, M. Karczewicz, Y.-C. Sun, T.-S. Chang, J. Lou, “CE8: Palette mode in HEVC (CE8-2.1)”, JVET-N0344, 14th meeting, Geneva, CH, March 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).