| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0555-v3 |
| --- | --- |

| Title: | CE6 – related: Simplification of the Reduced Secondary Transform |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mischa Siekmann Martin Winken Heiko Schwarz Detlev Marpe | Tel: Email: | +49 30 31002-440 mischa.siekmann@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes a simplification of the Reduced Secondary Transform (RST) [CE6-3.1, JVET-N0193]. By also enforcing all the primary transform coefficients to be zero, that are outside the support of the RST transform matrix, the encoder and decoder runtime can be reduced significantly, while the coding gain keeps almost the same. Tests were performed on top of three different RST configurations of CE6-3.1 plus one additional configuration (test 1), which combines the reduced memory version with the worst-case multiplication per sample constraint of JVET-N0193.  Compared to CE6-3.1d the encoder runtime overhead is halved for AI (113% vs 126%) with a negligible effect on the performance (AI: -1.28% vs. -1.34%).

With configuration 1, a luma BD-rate of -0.97% (AI), -0.50% (RA), -0.14% (LB) at a encoder/decoder runtime of 93% / 99% (AI), 103% /98% (RA), 102% / 100% (LB) compared to VTM 4.0 is achieved.

With configuration 3 (CE6-3.1d), a luma BD-rate of -1.28% (AI), -0.67% (RA), -0.22% (LB) at a encoder/decoder runtime of 113% / 99% (AI), 107% /98% (RA), 105% / 100% (LB) compared to VTM 4.0 is achieved.

With configuration 4 (CE6-3.1e), a luma BD-rate of -1.16% (AI), -0.58% (RA), -0.20% (LB) at a encoder/decoder runtime of 94% / 100% (AI), 103% / 99% (RA), 102% / 99% (LB) compared to VTM 4.0 is achieved.

# Introduction

In JVET-K0099 [1] the Reduced Secondary Transform (RST) was introduced and different variants were proposed in [2,3,4]. From encoder perspective, RST is a secondary transform which is applied to the top-left sub-set of primary transform coefficients and has the specific property, that depending on the residual block size, only a sub-set of the secondary transform coefficients are retained, while the remaining secondary coefficients are inferred to be zero.

Depending on the actual configuration and transform block dimensions, the RST is given by either a 64x16, 48x16, 16x16, 64x8 or 16x4 transform matrix.

Figure 1. RST8x8 with 64x16 transform matrix. Orange: secondary coefficients, Blue: enforced non-significant coefficients, Green: unaltered primary-only transform coefficients.

The RST flag/index is signaled at CU level, after the transform coefficients have been coded, and only if none of the TUs have significant coefficients at the restricted (blue) coefficient positions. Otherwise, it can be inferred that RST was not used.

# Proposed Method

In this contribution, the number of coefficients that are allowed to be significant, when a secondary transform is applied, is further reduced compared to the RST version of JVET-N0193. This is done by also enforcing those primary transform coefficients to be zero, that are not altered by the RST transform matrix (here, referred to as primary-only coefficients).

Precisely, this means that when a secondary transform is applied, only the first coefficient sub-group (4x4) is allowed to have significant coefficients. When the worst-case handling of JVET-N0193 is enabled, transform blocks with sizes 8x8 and 4x4 have significant coefficients in the first 8 positions in scan-order only. In JVET-N0193 the RST4x4 can be applied to two consecutive regions of blocks with shapes 4xN and Nx4. In this proposal the RST4x4 is only applied to the top-left 4x4 region and the primary-only coefficients are inferred to be zero.

Figure 2. Proposed method. Also, all primary transform coefficients are inferred to be zero when RST is applied.

This extended restriction of significant coefficients positions has several advantages:

- As before, the signaling of the RST flag/index is only needed if none of the already decoded significant coefficient positions violates the given zero constraints. By also enforcing the primary-only coefficients to be zero, this violation check is simply given by the last significant positions of the transform blocks. Hence, no extra scanning of significant coefficient positions is needed at the decoder side.

- Furthermore, the complexity of transforming the primary coefficients (forward and inverse) is reduced, as all coefficients outside the support of the RST matrix are known to be zero. For block sizes with a height and width of at least 8, only 64 primary coefficients need to be processed. While for smaller block sizes a maximum of 16 primary coefficients are non-zero. The skip-width and height of the vertical/horizontal transforms can be set accordingly.

- When checking the R-D costs of applying a secondary transform in the encoder search loop, a maximum of 16 coefficients need to be quantized, which reduces the overall encoder runtime significantly.

# Test Configurations:

The proposed method is implanted on top of the CE6-3.1 software [4] and tests were performed for four different configurations, which can be found in Table 1.

Table 1: Tested RST-Configurations

# Number of multiplications per coefficient

Following  [7], the number of multiplications per coefficient for transform structures with zeroing (keep m x n out of M x N ) out are given by:

Muls_per_coeffdirectMatrixMultiply=mMn+m

Muls_per_coeffhalfButterFly=mM(n2+log2n2)+(m2+log2m2)

Table 2. Number of multiplications per coefficient for transforms with zeroing (keep m x n of M x N). Example sizes of VTM-4.

Table 2 summarizes the obtained figures for different block shapes in VTM-4. It can be found that in the worst case of 32x32 transforms, which are implemented in halfButterFly structure, 40 multiplications are needed per coefficient.

Table 3 lists the needed multiplications when RST with zeroing of primary-only transform coefficients is applied.

Table 3. Number of multiplication per sample for RST with primary-only coefficient zeroing.

Table 4. Number of multiplications per sample (primary+secondary transform)

When RST is combined with zeroing of primary only coefficients, the number of multiplications per sample (primary+secondary) increases only for block sizes of 8x8 and smaller, compared to applying the primary transform only. In the worst case of 8x8, 22 (Test 3) multiplications have to be performed per sample, which is still less than the worst-case (40, for 32x32) and second worst case (32, for 16x32) of all sizes we have in VTM-4. For 4x4 blocks 16 (Test 3) multiplications are needed per sample. For block sizes larger than 8x8 the number of multiplications per sample even decreases due to the zeroing.

# Experimental results

According to the common test conditions in JVET-M1010 [5], simulations were performed for all-intra (AI), random-access (RA) and low-delay B (LB) configuration with VTM-4.0 software. In addition to the CTC, also the mandatory configurations of CE-6, as described in JVET-M1026 [6], are tested.  The main parameters of the simulation environment are shown in Table 2.

Table 2 Simulation environment

| CPU | Turbo Boost | SIMD settings | Compiler | OS |
| --- | --- | --- | --- | --- |
| E5-2697A v4 | off | AVX2 | GCC 7.2.1 | Linux |

## Test 1

### Results with inter MTS off (CTC)

### Results with inter MTS on

### Results – Low QP

## Test 2

### Results with inter MTS off (CTC)

## Test 3

### Results with inter MTS off (CTC)

### Results with Inter MTS on

### Results – Low QP

## Test 4

### Results with inter MTS off (CTC)

### Results – Low QP

# Conclusion

With the proposed simplifications the encoder runtime of the Reduced Secondary Transform (RST) could be significantly decreased. Depending on the actual configuration, to a reasonable trade off, or even below the anchor runtime. With this speed-up, there is no increase in decoder runtime compared to the anchor for any configuration. It is proposed to adopt the Reduced Secondary Transform in combination with the described zeroing of primary-only coefficients.

# Reference

- M. Salehifar et al., “CE 6.2.6: Reduced Secondary Transform (RST)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K0099, 11th meeting, Ljubljana, SI, 10-18 July 2018.

- M. Koo et al., “CE 6-2.1: Reduced Secondary Transform (RST)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-L0133, 12th meeting, Macao, CN, 3-12 Oct. 2018.

- M. Koo et al., “CE 6: Reduced Secondary Transform (RST) (test 6.5.1)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-M0292, 13th meeting, Marrakech, MA, 9–18 Jan. 2019.

- M. Koo et al., “CE6: Reduced Secondary Transform (RST)(CE6-3.1)”, JVET-N0193, 14th meeting, Geneva, CH, 19-27 March 2019.

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, 13th JVET meeting, Marrakech, MA, 9–18 Jan. 2019.

- X. Zhao and H. E. Egilmez, “Description of Core Experiment 6 (CE6): Transforms and transform signalling”, JVET-M1026, 13th JVET meeting, Marrakech, MA, 9–18 Jan. 2019.

- M. Zhou, Y. Hu, “CE6-related: A study of primary transforms,” JVET-M0046, 13th meeting, Marrakesh, MA, 9–18 Jan. 2019.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).