| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0570 |
| --- | --- |

| Title: | Crosscheck of JVET-N0206 (CE1-related: Improvement of the neighboring reference samples restriction on LIC test) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Vadim Seregin | Tel: Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents cross-check report of the JVET-N0206, where modification to the CE1-5 test is proposed by excluding non inter coded samples from the LIC parameters derivation. The partial available results are matching the results provided by the proponent.

# Introduction

In CE1-5, the restriction of using only neighbor samples of the current CU coded in intra mode is tested. In the CE1-5 implementation, the samples, which are not inter coded, are replaced with the corresponding samples from the reference block.

In the cross-checked contribution, such samples are excluded from being used in the LIC parameter derivation. Since the number of samples is changed, different normalization should be used, which is done by reusing existed look-up table.

The described modification was tested on top of CE1-5.1 and CE1-5.7.

The partial available results match to those provided by the proponent.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.63% | -0.11% | -0.27% | 119% | 99% |
| Class A2 |  |  |  |  |  |
| Class B | -0.35% | -0.31% | -0.33% | 114% | 102% |
| Class C | -0.28% | -0.32% | -0.37% | 118% | 104% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.12% | -0.21% | -0.14% | 115% | 102% |
| Class F | -0.78% | -0.49% | -0.56% | 109% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.56% | -0.12% | -0.45% | 116% | 102% |
| Class C | -0.43% | -0.27% | -0.32% | 120% | 100% |
| Class E | 0.03% | 0.18% | -0.11% | 102% | 97% |
| Overall | -0.37% | -0.10% | -0.32% | 113% | 100% |
| Class D | -0.13% | -0.24% | -1.04% | 117% | 100% |
| Class F | -2.61% | -2.79% | -2.55% | 114% | 98% |

Table 1. Excluding neighbor samples on top of CE1-5.1b

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.49% | -0.18% | -0.46% | 120% | 97% |
| Class A2 |  |  |  |  |  |
| Class B | -0.31% | -0.27% | -0.20% | 114% | 101% |
| Class C | -0.25% | -0.32% | -0.25% | 118% | 103% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.11% | -0.19% | -0.25% | 115% | 102% |
| Class F | -0.75% | -0.45% | -0.52% | 110% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.44% | -0.38% | -0.12% | 114% | 101% |
| Class C | -0.38% | -0.11% | -0.45% | 119% | 98% |
| Class E | 0.04% | 0.00% | 0.07% | 102% | 99% |
| Overall | -0.30% | -0.20% | -0.18% | 112% | 100% |
| Class D | -0.15% | -0.30% | -1.02% | 116% | 102% |
| Class F | -2.35% | -2.66% | -2.52% | 114% | 98% |

Table 2. Excluding neighbor samples on top of CE1-5.7b