| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0583 |
| --- | --- |

| Title: | Cross-check of JVET-N0394: CE3-related: Unified MPM list based on CE3-3.3 and CE3-3.5.1 |  |  |
| --- | --- | --- | --- |
| Status: | Information |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | 1-858-209-3851 xiaoyuxiu@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Verification

The document reports the results of crosschecking JVET-N0394 where a combination of CE3-3.3.2/CE3-3.3.3 and CE3-3.5.1 is proposed.

In the proposed combination, only one unified MPM list is generated, wherein Planar and DC modes are always first two MPMs in this unified MPM list. All the MPM candidates in this unified MPM list are used for zero reference line with ISP off, MPM candidates excluding DC mode in this unified MPM list are used for ISP on, and only the angular MPM candidates in this unified MPM list are used for non-zero reference lines. 5 or 6 angular MPM candidates are derived by the method in CE3-3.5.1, and the number of the unified MPM list is 7 or 8. For combined test 1 and combined test 3, there is one context for each of the first two bins of MPM index, which is the same as that in CE3-3.3.2. For combined test 2 and test 4, there are 3 and 1 contexts for the first two bins of MPM index respectively, which is the same as that in CE3-3.3.3.

## Method One

## Method Two

## Method Three