| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0586-v1 |
| --- | --- |

| Title: | Cross-check of JVET-N0087: CE4-related: Simplification of triangle merging candidate list derivation on top of CE4-4.1c |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Hongtao Wang 5775 Morehouse Drive, San Diego, CA, U.S.A. 92121 | Tel: Email: | 1-858-658-4535 hongtaow@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution provides cross-check results of JVET-N0087: CE4-related: An improved method for triangle merge list construction. The complete test is yet to be finished and the partial results match the test reported in JVET-N0087.

# Introduction

In JVET-N0087, it is proposed to derive the TPM merge candidate list as follows. Firstly, the regular merge candidate list is generated. For the first triangular merge candidate index, L0 part of regular merge candidate with the candidate index is tested and if it exists, the L0 MV is selected, otherwise L1 MV is selected. For the second triangular merge candidate index, L1 part of regular merge candidate with the index is tested, if it exists, the L1 MV is selected and otherwise, the L0 part of the candidate is selected.

Furthermore, pruning is performed among the 2 selected MVs. If they are same, the following logic apply:

- If the regular merge candidate of the 2nd triangular PU is bi-MV, then the L0 part is used for the 2nd triangular PU (Instead of the L1 part).

- Otherwise

  - If the regular merge candidate of the 1st triangular PU is bi-MV, then the L1 part is used for the 1st triangular PU (Instead of the L0 part).

  - Otherwise, an offset of (1/4 pel, 0) is added to the MV of the 2nd triangular PU.

The complete test is yet to be finished and the partial results match the test reported in JVET-N0087.

# Simulation results

The cross-check results are shown in the following table.

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.08% | 0.10% | -0.07% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.06% | 0.01% | 0.18% | 101% | 101% |
| Class F | 0.10% | 0.09% | 0.03% | 101% | 101% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.09% | 0.14% | -0.05% | 101% | 102% |
| Class E | 0.04% | 0.82% | 0.31% | 100% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.13% | -0.20% | 0.12% | 99% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusion

This contribution provides cross-check results of JVET-N0087: CE4-related: An improved method for triangle merge list construction. The complete test is yet to be finished and the partial results match the test reported in JVET-N0087.

# Patent rights declaration(s)

Qualcomm does not have any current or pending patent rights relating to the technology described in this contribution.