| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0588 |
| --- | --- |

| Title: | Cross-check of JVET-N0465 (CE3-related : Restrict 2xN, Nx2 chroma processing for dual tree structure) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | TianYang Zhou Tomohiro Ikai | Tel: Email: | +81-50-5213-6338 zhou.tianyang @sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution is a cross-check report of JVET-N0465: CE3-related: Restrict 2xN, Nx2 chroma processing for dual tree structure. The code has been checked and the crosscheck was carried out under the JVET common test condition (CTC). Experimental results (partial) match with those provided by the proponent.

Introduction

In JVET-N0465[1], it has two tests as below.

#1. Extend Chroma block partitioning restriction(test2 in JVET-N0465).

The proposed restriction for chroma block partitioning is that 2xN and Nx2 split is disabled.

#2. Proposed IBC Chroma motion vector derivation based 4x4 sub-block(test3 in JVET-N0465).

The proposed IBC Chroma motion vector derivation process is applied base on 4x4 sub-block size instead of 2x2 sub-block and luma motion vector which is corresponded with top-left position of the 4x4 Chroma sub-block.

Simulation results

The software on top of VTM-4.0 was provided by the proponents [1]. Crosscheck experiments were performed under the JVET common test conditions. Following tables show the results of the cross-check experiments under VTM-4.0 configuration.

## Table1-M0465_test2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0_SHARP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.50% | 0.27% | 98% | 99% |
| Class A2 | 0.21% | 1.48% | 1.11% | 97% | 98% |
| Class B | 0.05% | 1.09% | 1.64% | 99% | 99% |
| Class C | 0.25% | 2.81% | 3.90% | 98% | 97% |
| Class E | -0.01% | 0.29% | 0.56% | 100% | 100% |
| Overall | 0.12% | 1.31% | 1.65% | 98% | 99% |
| Class D | 0.33% | 4.08% | 5.39% | 97% | 96% |
| Class F (optional) | 0.60% | 4.00% | 4.90% | 99% | 97% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0_SHARP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 1.20% | 1.47% | 101% | 100% |
| Class C | 0.08% | 1.70% | 2.20% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.08% | 2.49% | 3.50% | 101% | 100% |
| Class F (optional) | 0.30% | 2.55% | 3.09% | 100% | 100% |

Table1-M0465_test3

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0_SHARP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.50% | 0.27% | 99% | 100% |
| Class A2 | 0.21% | 1.48% | 1.11% | 97% | 98% |
| Class B | 0.05% | 1.09% | 1.64% | 99% | 99% |
| Class C | 0.25% | 2.81% | 3.90% | 98% | 97% |
| Class E | -0.01% | 0.29% | 0.56% | 99% | 100% |
| Overall | 0.12% | 1.31% | 1.65% | 98% | 99% |
| Class D | 0.33% | 4.08% | 5.39% | 97% | 96% |
| Class F (optional) | 0.63% | 4.02% | 4.97% | 99% | 97% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0_SHARP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 1.20% | 1.47% | 101% | 100% |
| Class C | 0.08% | 1.70% | 2.20% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.08% | 2.49% | 3.50% | 101% | 100% |
| Class F (optional) | 0.37% | 2.68% | 3.18% | 100% | 100% |

Conclusion

It was confirmed that the the simulation results (partial) of bitrate and PSNR match with those provided by the proponent.

References

[1] Hyeongmun Jang, Naeri Park, Junghak Nam, Jaehyun Lim “CE3-related : Restrict 2xN, Nx2 chroma processing for dual tree structure”, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 14th Meeting: Geneva, CH, 19–27 March 2019