| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0590 |
| --- | --- |

| Title: | Crosscheck of JVET-N0342 (Decoder-side Intra Mode Derivation with Prediction Fusion) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Cross-check Report |  |  |
| Author(s) or Contact(s): | Christophe Chevance | Tel: Email: | Christophe.chevance@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution reports cross-checking results for contribution JVET-N0342 (Decoder-side Intra Mode Derivation with Prediction Fusion). The simulations were performed in AI configuration. Simulation results reportedly match those provided by the proponents. Encoding and decoding time are reported to be in line with those reported in JVET-N0342.

# Introduction

JVET-N0342 presents a modified version of the Decoder-side Intra Mode Derivation (DIMD) on top of the previously proposed contribution JVET-M0094 [1]. The proposal infers the intra prediction mode based on the analysis of 3 lines/columns neighboring the current block. A CU-level DIMD flag is sent before the multi-reference line (MRL) and Intra Sub-Partitioning (ISP) indexes. When it is true, the intra mode is not signaled but instead derived at the decoder.

# Experimental Results

The cross-checking was achieved using the following computing specifications:

- CPU type:	Xeon

- Hyperthreading:	off

- Turbo mode:	on

- Compiler:	gcc 6.3.1

- OS:	CentOS7

- Decoding options

  - MD5 SEI:	no

  - YUV output:	yes

- SIMD options:	SSE42

Results are reported below for AI configurations.

For all the simulations, a perfect match is obtained compared to the proponent results.

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.51% | -0.35% | -0.41% | 124% | 104% |
| Class A2 | -0.30% | -0.13% | -0.14% | 122% | 104% |
| Class B | -0.34% | -0.16% | -0.18% | 123% | 104% |
| Class C | -0.40% | -0.29% | -0.21% | 123% | 108% |
| Class E | -0.52% | -0.27% | -0.42% | 124% | 107% |
| Overall | -0.40% | -0.24% | -0.26% | 123% | 105% |
| Class D | -0.49% | -0.35% | -0.49% | 122% | 113% |
| Class F | -0.25% | -0.23% | -0.08% | 112% | 105% |