| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0600 |
| --- | --- |

| Title: | Context Reduction for Inter and Split Syntax Elements |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Y. Chen F. Le Léannec T. Poirier | Tel: Email: | +33(0)299273198 ya.chen@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution proposes to simplify the context modelling and reduces the number of contexts for CABAC in VVC. In this contribution, the CABAC context modelling for the regular coded bins affine_flag, amvr_flag, triangle_flag and split_qt_flag are investigated. With 3, 4, 7 and maximum 83 contexts reduction, the BD-Rate increase is reported as 0.02%, -0.01%, 0.01% and 0.08% for RA, respectively.

# Introduction

In VTM-4.0, the context derivation process of some syntax elements uses left and above neighboring syntax elements. For example, affine_flag, which is signaled to indicate whether the affine model based motion compensation is used to generate the prediction samples of the current CU or not, is regular coded with 3 context models and the context model is derived with the sum of the affine_flag of the left block L and above block A as depicted in Figure 1.

Figure 1: neighboring blocks used in context model derivation for affine_flag

And the CABAC context ctx derivation process is formulated as the follow:

ctx = (L is available && L is affine) + (A is available && A is affine)

CABAC context of other syntax elements amvr_flag (to signal AMVR mode), triangle_flag (to signal triangular prediction mode) and split_qt_flag (to signal quadtree partitioning) are also derived in quite the similar way.

# Proposed Solution

## affine_flag and split_qt_flag coding

Method 1: derive context model with the OR information of the left and above syntax element for affine_flag and split_qt_flag

In this contribution, the context modelling derivation of affine_flag is modified as the follow:

ctx = (L is available && L is affine) ∥ (A is available && A is affine)

with 1 context reduction.

And the context modeling derivation of split_qt_flag is modified as the follow:

ctx = (L is available && cqDepth_L > cqDepth) ∥ (A is available && cqDepth_A > cqDepth)

+ (cqDepth < 2 ? 0: 2)

with 2 contexts reduction.

## amvr_flag and triangle_flag coding

Method 2: derive context model without using the left and above syntax element for amvr_flag and triangle_flag

In this contribution, it is also proposed to use only one context model for coding amvr_flag and triangle_flag to avoid using the left and above syntax elements. This modification can reduce the line buffer size as well as the parsing complexity, since the syntax elements of the neighbouring blocks is not used in the context derivation process. And in total, 4 contexts can be reduced for amvr_flag and triangle_flag.

# Simulation results

The proposed method is implemented on the top of VTM-4.0. The common test conditions defined in [3] is used in the simulation. The tables below show the BD-Rate of the proposed methods of transform coefficients coding relative to VTM-4.0. Please note that the timing is not reliable.

Table 1 gives the performances of the method 1 of affine_flag and split_qt_flag coding compared to the VTM-4.0 anchor.

Table 1: method 1 performances over VTM-4.0.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | 0.04% | 102% | 99% |
| Class A2 | 0.00% | 0.09% | -0.02% | 100% | 98% |
| Class B | 0.00% | -0.01% | 0.07% | 99% | 99% |
| Class C | 0.03% | -0.04% | 0.03% | 101% | 99% |
| Class E | -0.01% | -0.01% | 0.01% | 102% | 100% |
| Overall | 0.00% | 0.01% | 0.03% | 101% | 99% |
| Class D | 0.03% | 0.06% | -0.11% | 102% | 98% |
| Class F (optional) | 0.06% | 0.02% | -0.03% | 102% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | 0.04% | 104% | 103% |
| Class A2 | 0.04% | -0.08% | 0.01% | 103% | 102% |
| Class B | 0.02% | 0.00% | -0.12% | 102% | 101% |
| Class C | 0.02% | -0.12% | -0.04% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.04% | -0.04% | 102% | 101% |
| Class D | 0.01% | 0.02% | -0.21% | 101% | 103% |
| Class F (optional) | 0.04% | -0.10% | -0.07% | 102% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.09% | -0.44% | 102% | 101% |
| Class C | 0.00% | 0.28% | -0.02% | 102% | 99% |
| Class E | 0.10% | 0.50% | 0.26% | 101% | 100% |
| Overall | 0.03% | 0.18% | -0.13% | 102% | 100% |
| Class D | 0.04% | 0.33% | -0.23% | 101% | 101% |
| Class F (optional) | -0.04% | 0.15% | 0.35% | 99% | 100% |

Next table provides the results of method 2 over VTM-4.0, which simplify and reduce the context modeling of amvr_flag and triangle_flag coding

Table 2: method 2 performances over VTM-4.0.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.01% | 0.03% | 104% | 98% |
| Class A2 | -0.02% | 0.00% | 0.03% | 102% | 98% |
| Class B | -0.01% | -0.02% | -0.13% | 102% | 98% |
| Class C | 0.00% | -0.12% | -0.03% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.04% | -0.04% | 102% | 98% |
| Class D | -0.02% | -0.03% | -0.09% | 100% | 99% |
| Class F (optional) | -0.01% | 0.04% | -0.02% | 102% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.12% | -0.23% | 101% | 98% |
| Class C | -0.02% | 0.08% | 0.07% | 101% | 98% |
| Class E | -0.03% | 0.14% | -0.32% | 101% | 101% |
| Overall | -0.01% | 0.01% | -0.15% | 101% | 99% |
| Class D | -0.01% | -0.04% | -0.40% | 99% | 99% |
| Class F (optional) | -0.07% | -0.34% | -0.15% | 99% | 100% |

Table 3 provides the results of combining method 1 and method 2 over VTM-4.0.

Table 3: method 1 and method 2 performances over VTM-4.0.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | 0.04% | 102% | 100% |
| Class A2 | 0.00% | 0.09% | -0.02% | 100% | 98% |
| Class B | 0.00% | -0.01% | 0.07% | 100% | 100% |
| Class C | 0.03% | -0.04% | 0.03% | 102% | 102% |
| Class E | -0.01% | -0.01% | 0.01% | 101% | 100% |
| Overall | 0.00% | 0.01% | 0.03% | 101% | 100% |
| Class D | 0.03% | 0.06% | -0.11% | 101% | 100% |
| Class F (optional) | 0.06% | 0.02% | -0.03% | 102% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.01% | 0.03% | 104% | 102% |
| Class A2 | -0.02% | 0.00% | 0.03% | 102% | 102% |
| Class B | 0.01% | 0.00% | -0.08% | 101% | 100% |
| Class C | 0.03% | -0.02% | -0.08% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.00% | -0.04% | 102% | 101% |
| Class D | 0.03% | 0.00% | -0.01% | 100% | 104% |
| Class F (optional) | 0.04% | -0.09% | -0.13% | 103% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.15% | -0.26% | 103% | 105% |
| Class C | -0.01% | 0.21% | -0.11% | 103% | 100% |
| Class E | 0.15% | 0.90% | 0.81% | 101% | 101% |
| Overall | 0.03% | 0.23% | 0.06% | 102% | 102% |
| Class D | -0.01% | -0.16% | -0.60% | 99% | 99% |
| Class F (optional) | -0.01% | 0.03% | -0.39% | 99% | 99% |

Table 4 provides all the context simplification and reduction proposals in this contribution and JVET-N0311[4] over VTM-4.0.

Table 4: all proposed methods in this contribution and JVET-N0311 performances over VTM-4.0.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.26% | 0.33% | 106% | 101% |
| Class A2 | 0.12% | 0.51% | 0.35% | 103% | 99% |
| Class B | 0.08% | 0.15% | 0.13% | 105% | 101% |
| Class C | 0.10% | -0.02% | 0.16% | 105% | 101% |
| Class E | 0.06% | 0.19% | 0.05% | 105% | 99% |
| Overall | 0.09% | 0.20% | 0.19% | 105% | 100% |
| Class D | 0.10% | 0.14% | -0.15% | 105% | 99% |
| Class F (optional) | 0.04% | 0.12% | 0.15% | 108% | 105% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.24% | 0.21% | 104% | 103% |
| Class A2 | 0.11% | 0.13% | 0.02% | 103% | 103% |
| Class B | 0.05% | 0.21% | 0.06% | 102% | 101% |
| Class C | 0.09% | 0.09% | -0.15% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | 0.17% | 0.03% | 103% | 102% |
| Class D | 0.07% | -0.08% | -0.10% | 102% | 102% |
| Class F (optional) | 0.04% | 0.02% | 0.26% | 104% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.27% | -0.13% | 106% | 106% |
| Class C | -0.04% | 0.12% | -0.09% | 103% | 100% |
| Class E | 0.09% | 0.65% | 0.63% | 103% | 102% |
| Overall | 0.00% | 0.09% | 0.07% | 104% | 103% |
| Class D | -0.10% | 0.21% | -0.04% | 103% | 102% |
| Class F (optional) | -0.05% | 0.02% | -0.57% | 101% | 101% |

# Conclusions

This contribution proposes some ways to reduce the complexity of the regular coded bins entropy coding and the number of CABAC contexts in the same time. This is achieved through the removal of the contexts for the affine_flag, amvr_flag, triangle_flag and split_qt_flag. The results of different combinations of the proposed methods in this contribution are given by the following table.

As can be seen, a significant reduction of the context used in VTM-4.0 can be achieved without major penalty in terms of coding efficiency.

Table 14: summary of the performances and CABAC context reduction of the proposed methods

| Proposed method | Nb contexts/VTM-4.0 | RA (Y/Cb/Cr) |
| --- | --- | --- |
| method 1 | -3 | 0.02% -0.04% -0.04% |
| method 2 | -4 | -0.01% -0.04% -0.04% |
| method 1 + method 2 | -7 | 0.01% 0.00% -0.04% |
| all proposed methods in this contribution and JVET-N0311 | -83 | 0.08% 0.17% 0.03% |

# Reference

[1] F. Le Léannec, Y. Chen, JVET-M0107, “CE7-related: reduced local neighborhood usage for transform coefficients coding”, 13th meeting, Marrakech, MA, 9–18 Jan. 2019.

[2] M. Gao, X. Li, X. Zhao and S. Liu, JVET-M0489, “CE7-related: Reduced context models for transform coefficients coding”, 13th meeting, Marrakech, MA, 9–18 Jan. 2019.

[3] F. Bossen, J. Boyce, K. Sühring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, 13th meeting, Marrakech, MA, 9–18 Jan. 2019.

[4] Y. Chen, F. Le Léannec, T. Poirier, JVET-N0311, “CE7-related: Context modeling simplification and reduction in VVC”, 14th meeting, Geneva, CH, 19–27 March 2019.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).