| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0604 |
| --- | --- |

| Title: | Crosscheck of JVET-N0467 (Non-CE8: Experimental result for various size of IBC block with optimized syntax signaling) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu Han, Wei-Jung Chien 5775 Morehouse Drive, San Diego, CA, U.S.A. 92121 | Tel: Email: | +1-858-845-1793 yuhan@qti.qualcomm.com wchien@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution is a crosscheck report of JVET-N0467 (Non-CE8: Experimental result for various size of IBC block with optimized syntax signaling). The simulations were carried out based on the common test condition (CTC) for Class F. Based on the partial results, the test results are matched.

# Introduction

In JVET-N0467, IBC mode is signalled from 4x4 to 128x128 luma block while Rate-Distortion Optimization process is only applied on a block which is smaller and equal to 16x16. IBC coding efficiency on the various block size restriction is investigated as follows:

#2. IBC mode for 4x4 ~ 16x16 and syntax Signaling optimization

#3. IBC mode for 4x4 ~ 32x32 and syntax Signaling optimization

#4. IBC mode for 4x4 ~ 64x64 and syntax Signaling optimization

# Simulation results

The simulation is based on VTM4.0 software. The anchor is generated by using VTM4.0 default configurations with IBC mode. Common test conditions for Class F are followed. Results are shown in the following tables:

## Test 2: IBC mode for 4x4 ~ 16x16 and syntax Signaling optimization

|  | All Intra Main10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4 + IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.05% | -0.08% | -0.05% | 100% | 99% |
| Class SCC 1080p | -0.13% | -0.11% | -0.11% | 101% | 100% |

|  | Random Access Main 10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4 + IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.09% | -0.13% | -0.07% | 100% | 100% |
| Class SCC 1080p | -0.14% | -0.14% | -0.12% | 100% | 99% |

## Test 3: IBC mode for 4x4 ~ 32x32 and syntax Signaling optimization

|  | All Intra Main10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4 + IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.10% | -0.06% | -0.11% | 103% | 101% |
| Class SCC 1080p | -0.77% | -0.87% | -0.88% | 108% | 101% |

|  | Random Access Main 10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4 + IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.11% | -0.21% | -0.25% | 103% | 99% |
| Class SCC 1080p | -0.80% | -1.02% | -1.03% | 105% | 98% |

## Test 4: IBC mode for 4x4 ~ 64x64 and syntax Signaling optimization

|  | All Intra Main10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4 + IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.08% | -0.09% | -0.17% | 103% | 100% |
| Class SCC 1080p | -0.79% | -0.87% | -0.90% | 108% | 101% |

|  | Random Access Main 10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4 + IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.08% | -0.19% | -0.26% | 104% | 103% |
| Class SCC 1080p | -0.87% | -0.97% | -1.10% | 107% | 99% |

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET-M1010, “JVET common test conditions and software reference configurations for SDR video”, Marrakech, January 2019.