| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0605r1 |
| --- | --- |

| Title: | Comparative study of video coding solutions VVC, AV1, EVC versus HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ya Chen, Edouard François, Franck Galpin, Rémi Jullian, Michel Kerdranvat Technicolor 975 Avenue des champs blancs 35576 Cesson-Sévigné, FRANCE | Tel: Email: | +33 2 99 27 33 95 first-name.last-name@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution reports the results of a comparative study between video coding solutions either standardized or stable such as HEVC (IEC/ISO and ITU-T) and AV1 (Alliance for Open Media), or still in development such as VVC in the JVET group of IEC/ISO and ITU-T, or EVC (Essential Video Coding) within IEC/MPEG video group.

The methodology is based on the common test conditions (CTC) defined in JVET by using the same configuration parameters on all the sequences of classes A1, A2, B, C and D. Specific investigations were made on AV1 configuration parameters to be as close as possible to CTC for a fair comparison. However, to limit the processing time only one second of every sequence was coded by assuming that the relative performances between codecs are not changed compared to complete sequences. This assumption was validated.

The performances according the average BD-rate PSNR metrics of VTM-4.0 (VVC after Marrakech meeting January 2019), AV1, and ETM 1.0 (EVC after Marrakech meeting) versus HM-16.18 are respectively 29.5%, 21%, and 15% gains for all the CTC clips. When considering only UHD and HD clips the gains are higher, 33.5%, 22.4% and 19.8% respectively. The run times versus HM-16.18 for encoding are respectively 7.4x, 6.7x, 3.6x and for decoding 1.2x, 2.5x, 1.4x. As VVC is still in development the question could be asked on its performance versus AV1 and potentially EVC: is it significant enough to ensure its commercial success?

# Introduction

Currently, the development of new video codecs knows a busy time in standards organizations ISO/IEC MPEG and ITU-T VCEG, but also in AOM. The state-of-the-art in video compression is HEVC/H.265, which was published in 2015. MPEG and VCEG have joint again their resources in 2018 to develop a new video codec, named VVC (Versatile Video Coding), targeting 50% gain over HEVC by 2020. However, a competitive project is about to be launched in MPEG for a new standard, named is EVC (Essential Video Coding), with the objective to be royalty-free in a baseline profile and with managed IPR in a main profile. Furthermore, AOM has developed another video codec, named AV1, claimed to be royalty free in order to compete with HEVC.

This contribution presents the results of a comparison between these codecs: HEVC, AV1, VVC, and EVC. After an overview of the algorithms, and a description of the simulation conditions, the relative performances between these codecs are provided.

# Overview of the algorithms

In the Table 1 below, the video coding solutions are compared per category of coding tools based on existing specifications and other input sources [10]. More detailed description can be found in the cited references.

| Category | HEVC [1] | VVC [3] | AV1 [6] | EVC (Main) [4] |
| --- | --- | --- | --- | --- |
| Partition size | - Coding Unit: 8x8 to 64x64 | - Coding Unit: 4x4 to 128x128 - Pipeline size 64x64 | - Superblock: 4x4 to 128x128 | - Coding Unit: 4x4 to 128x128 - Pipeline size 128x128 |
| Partitioning | - Transform: QT - Prediction: QTBT | - QTBT + TT (NS, QT, BTx2, TTx2= 6 partitions) | - Recursive tree (10 partitions) | - QTBT + TT (NS, QT, BTx2, TTx2= 6 partitions) - Split Unit Coding Order |
| Intra Prediction | - 35 predictors | - 81 modes: 65 angular + planar + DC + Sub - 6-MPM | - 60 modes: 56 angles + 3 smooths + Paeth | - 30 modes (angular, bilinear, DC, plane), Inspired from HEVC & AVS2 - 2MPM + 2nd 8MPM |
|  |  | - Chroma from Luma, PDPC, combined, Wide Angle, Sub-partitions | - Chroma from Luma | - Close to HEVC |
| Inter Prediction | - 1/4 Pixel luma MV - 2 interpolation filters for MC | - 1/16 Pixel luma MV - 2 interpolation filters for MC - DMVR | - 1/8 Pixel luma MV - 5 interpolation filters for MC | - 1/16 Pixel luma MV - 2 interpolation filters for MC - DMVR |
|  |  | - Sub-PU based MV Pred (ATMVP, Affine) - Enhanced Pred: GBi, BDOF, Triangular, Multi-hypothesis | - Extended reference frames (7 chosen per frame) | - Many similarities for inter prediction with VVC |
|  |  | - Enhanced MV pred: MMVD, HMVP, Pairwise Enhanced MV coding, AMVR, SMVD | - Enhanced MV pred: compound pred, wedge pred, OBMC, Global motion | - Many similarities for MV predictors with VVC: AMVR, MMVD, Affine, HMVP |
| Transform | - square DCT-based transforms - 1 DST4x4-based transform | - DCT2 (up to 64x64) + MTS (<=32x32, DST7, DCT8, independent H/V) - Coefficient coding: HEVC + DQ design | - DCT, ADST, Flipped ADST, Identity, H/V Independently & Rectangular | - floating DCT2 converted into 8 bits, up to 128x128) - coefficient coding: run-level-last |
| Entropy coding | - CABAC | - CABAC + dependent scalar quantization - LUT based CAE | - Multi-symbol Arithmetic Coder | - Multiplier-based CAE |
| Loop filters | - Deblocking - SAO | - Deblocking - SAO - ALF | - Deblocking - Constrained Directional Enhancement Filter - Loop restoration | - Hadamard transform domain filter - Deblocking (H.263+) - ALF |
| Screen Content Coding |  | - Intra Block Copy | - Palette mode - Intra Block Copy |  |
| Others |  |  | - Frame Super-resolution - Frame Grain Synthesis |  |

Table 1: Video coding algorithms comparison

Terminology used in Table 1 :

- QTBT: Quad Tree Binary Tree

- BT: Binary Tree

- TT: Ternary Tree

- MPM: Most Probable Mode (Intra)

- BDOF: Bi Directional Optical Flow

- MV: Motion Vector

- MC: Motion compensation

- GBi: Generalized Bi Prediction

- MMVD: Merge with Motion Vector Difference

- HMVP: History based Motion Vector Prediction

- AMVR: Adaptive Motion Vector Resolution

- ATMVP: Advanced Temporal Motion Vector Prediction

- SMVD: Symmetrical Motion Vector Difference

- SAO: Sample Adaptive Offset

- ALF: Adaptive Loop Filter

- MTS: Multiple Transform Set

- DCT2: Discrete Cosine Transform 2

- DCT8: Discrete Cosine Transform 8

- DST7: Discrete Sine Transform

- CAE: Context Adaptive Entropy

- OBMC: Overlapped Block Motion Compensation

- PDPC: Position Dependent Prediction Combination (Intra)

- Paeth: kind of planar prediction for AV1

# Simulation conditions

The simulation conditions follow the Random Access 10-bit mode specified in the common test conditions of JVET [1] for the HM-16.18, VTM-4.0, which are also used for EVC. The software configuration requires not more than 16 frames of structural delay, e.g. 16 pictures “group of pictures (GOP)”, and random-access intervals of 1.1 seconds or less. In order to reduce the simulation times only one second of each sequence was processed which means 32 pictures for a video sequence with a frame rate of 30 Hz, 48 pictures for a video sequence with a frame rate of 50 Hz, 64 pictures for a video sequence of 60 Hz. However, the complete set of JVET mandatory sequences was used.

All the simulations were run in single thread on the same platform in order to compare run times at encoder and decoder.

The AV1 version of January 8th, 2019 (1.0.0) [6] was used. The configuration parameters of AV1 software are different from the HM, VTM, and EVC. They were fixed according to published information either in conference [11], journals or web sites from AOM, but also by running several tests in order to be as close as possible as JVET CTC. However, AOM recommends running two passes of encoding to reach the best performance because the first pass computes important statistics on the full sequence to guide the second pass. A test was made on the Class D sequences, the gain is 20.60% with two passes but only 7.04% with one pass. The encoder/decoder running time versus HM-16.18 for two-passes is 739% / 209%, and for one pass 676% / 193%. Two passes clearly provide a look ahead on the encoding parameters the other coding solutions have not. Nevertheless, two passes were used at the best quality (slowest encoding) to not disadvantage AV1.

Another important parameter is the “End-usage” which fixes the quantization method. A constant quantizer value was chosen again to be close to JVET CTC.

The impact of fixing the golden frame distance (AV1 specific feature) to stick to the 16 pictures GOP size of JVET CTC was also investigated on one second sequences. As a result, there was no observed impact. It was also the case when the key frame distance was fixed to one second or let without constraint.

| AV1 configuration |  |  |
| --- | --- | --- |
| Passes | 2 | 1st pass used to build statistics, 2nd to encode |
| End-usage | q | - q: constant quality, achieves constant quality level without regard to bitrate - cq: constrained quality, achieves constant quality level below a specified upper bitrate - vbr: variable citrate - cbr: constant bitrate |
| cq-level | 32, 40, 48, 56 | Constant Quality level (QP value) |
| cpu-used | 0 | Encoder speed up: 0, 1, 2 cpu-used=0 is the slowest one with the best quality |

Table 2: AV1 configuration

The full command line for AV1 encoding is the following for a Class D:

./aomenc seq.yuv -o bitstream.webm --kf-min-dist=0 --kf-max-dist=1000 --auto-alt-ref=2 --lag-in-frames=25 -v --fpf=stats.fpf --i420 --skip=0 --codec=av1 --passes=2 --threads=1 --minsection-pct=0 --maxsection-pct=2000 --static-thresh=0 --drop-frame=0 --end-usage=q --cq-level=56 --tune=psnr --q-hist=0 --rate-hist=0 --psnr --bit-depth=10 --arnr-maxframes=7 --arnr-strength=3 --width=832 --height=480 --fps=30/1 --limit=33 --input-bit-depth=8 --cpu-used=0

# Results

## BD-rate

In the following tables the results are provided using the following objective metrics: the Bjøntegaard PSNR Delta-Rate [2].

For 10-bit video, PSNR is calculated as 10*(log10((255 << 2) / MSE). 8-bit content is converted to 10-bit input in the encoder by shifting 2 bits to the left and 10-bit PSNR calculation is used to report testing results.

In order to have a better overview of the performances a combined PSNR is used in this study:

- PSNRYUV = (6xPSNRY + PSNRU + PSNRV) / 8.

It allows to mitigate the encoding strategy regarding the bit budget balance between luminance and chrominance.

It has been validated on AV1 versus HM-16.18 that encoding one second (one intra period) of a clip provides performance figures (see Table 3) that are very close to a complete clip with some variations depending on classes, but the overall mean value provides a good estimation:

| BD-Rate PSNR YUV | AV1 full length clips | AV1 one second clips |
| --- | --- | --- |
| Class: A1+A2+B | -21.1% | -22.4% |
| Class: C+D | -20.1% | -19.2% |
| Overall clips | -20.7% | -21.0% |

Table 3: Full length Vs One second clips for AV1 Vs HM-16.18

The tables show the performance according to the PSNR BD-rate metrics in percentage with regards to a reference that can be any of the codecs. A negative percentage means a gain while a positive percentage means a loss regarding the reference.

| BD-Rate PSNR YUV* | Reference |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test |  | HM-16.18 | VTM-4.0 | AV1 | ETM-1.0 |
|  | HM-16.18 |  | 50.8% | 29.5% | 25.1% |
|  | VTM-4.0 | -33.5% |  | -11.4% | -17.4% |
|  | AV1 | -22.4% | 13.5% |  | -4.0% |
|  | ETM-1.0 | -19.8% | 21.3% | 5.4% |  |

Table 4: results on UHD and HD sequences (Classes A1, A2 and B)

| BD-Rate PSNR YUV* | Reference |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test |  | HM-16.18 | VTM-4.0 | AV1 | ETM-1.0 |
|  | HM-16.18 |  | 31.6% | 25.1% | 9.4% |
|  | VTM-4.0 | -23.9% |  | -5.3% | -16.8% |
|  | AV1 | -19.2% | 6.0% |  | -11.4% |
|  | ETM-1.0 | -8.6% | 20.5% | 14.6% |  |

Table 5: results on WVGA and WQVGA formats (Classes C and D)

| BD-Rate PSNR YUV* | Reference |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test |  | HM-16.18 | VTM-4.0 | AV1 | ETM-1.0 |
|  | HM-16.18 |  | 42.7% | 27.6% | 18.5% |
|  | VTM-4.0 | -29.5% |  | -8.8% | -17.1% |
|  | AV1 | -21.0% | 10.4% |  | -7.1% |
|  | ETM-1.0 | -15.0% | 21.0% | 9.3% |  |

Table 6: Overall results (all sequences)

## Run time

The Table 7 reports the run time factor for the encoding and the decoding versus the HM-16.18 for UHD and HD sequences (Classes A1, A2, and B). The run time provides an estimate of the complexity. “Nx” means N times the HM-16.18 run time.

| Run time Vs HM-16.18 | VTM-4.0 | AV1 | ETM-1.0 |
| --- | --- | --- | --- |
| Encoding | 7.36x | 6.71x | 3.62x |
| Decoding | 1.23x | 2.48x | 1.36x |

Table 7: Run times on UHD and HD versus HM-16.18

The platform characteristics are:

| CPU type | Intel(R) Xeon(R) Gold 6142 |
| --- | --- |
| Hyper threading | Off |
| Turbo mode | On |
| Compiler | gcc 6.3.0 |
| OS | CentOS7 |

Table 8: Platform characteristics

## Summary

# Conclusion

This comparative study on video coding solutions already specified HEVC (IEC/ISO, ITU-T), and AV1 (AOM), or still in development VVC (IEC/ISO, ITU-T), and EVC (IEC/ISO) has been conducted in the same test conditions. Some adaptations were made for AV1 as the software in not configurable in the same way. Indeed, AV1 software is designed as a product capable of encoding full movies with several parameters to fix the encoding time, the rate control, the number of passes … IEC/ISO and ITU-T software are test models designed to develop algorithms in a collaborative environment. The configuration parameters of AV1 were fixed according to published information either in conferences, journals or web sites, but also by running several tests. The approach has been driven to put simulation conditions of AV1 as close as possible to CTC JVET ones. It was estimated that keeping the 2-passes configuration was required, as the 1-pass only configuration is clearly not optimized in the AV1 codec.

The performances according the BD-rate metrics of VTM-4.0 (VVC after Marrakech meeting January 2019), AV1, and ETM 1.0 (EVC after Marrakech meeting) versus HM-16.18 are respectively 29.5%, 21%, and 15% on all frame sizes, but by considering only UHD and HD the gains are higher 33.5%, 22.4% and 19.8%. The run times versus HM-16.18 for encoding are respectively 7.4x, 6.7x, 3.6x and for decoding 1.2x, 2.5x, 1.4x. As VVC is still in development, the question could be asked on its performance versus AV1 and potentially EVC: is it significant enough to ensure its commercial success?

# References

- H.265 (04/2015), Coding of moving video: High Efficiency Video Coding, ITU-T

- JVET-M1010, “JVET common test conditions and software reference configurations for SDR video”, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019

- JVET-M1002, “Algorithm description for Versatile Video Coding and Test Model 4 (VTM 4)”, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019

- ISO/IEC JTC1/SC29/WG11 N18283, “Working Draft 1 of Essential Video Coding”, Jan. 2019, Marrakech

- ISO/IEC JTC1/SC29/WG11 MPEG2018/N18284, “Test Model of Essential Video Coding (ETM 1.0)”, Jan. 2019, Marrakech

- AV1 specification: https://aomediacodec.github.io/av1-spec/

- AV1 software (commit 52ea88f)

- Gisle Bjontegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6 VCEG 13th meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33 (available at http://wftp3.itu.int/av-arch/video-site/0104_Aus/).

- Gisle Bjontegaard, “Improvements of the BD-PSNR model”, ITU-T SG16/Q6 VCEG 35th meeting, Berlin, Germany, 16–18 July, 2008, Doc. VCEG-AI11 (available at http://wftp3.itu.int/av-arch/video-site/0807_Ber/).

- ISO/IEC JTC1/SC29/WG11 MPEG2019/ m46554, “Tool comparison between VVC (VTM3.0) and NVC”, Jan. 2019, Marrakech

- “Analysis of Emerging Video Codecs: Coding Tools, Compression Efficiency and Complexity”, MediaKind (Ericsson), SMPTE 2018 annual conference