| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0617 |
| --- | --- |

| Title: | Crosscheck of JVET-N0478 (CE1: Hadamard transform domain filter): additional test results for HTDF at the “loop filter” position (CE1-2.3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | D.Rusanovskyy 10160 Pacific Mesa Blvd San Diego, CA 92121 (USA) | Email: | dmytror@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This document reports the crosscheck results for the Hadamard transform domain filter (HTDF) at the “loop filter” position, test CE1-2.3 of JVET-N0478. Additional test results are reported by cross-checker for CE1-2.3 w/o filtering during the RDO process.

# Results

The method proposed in N0478 and tested in CE1-2.3 deploys the HTDF filter at the loop filter position, HTDF is applied as conventional in-loop filter (no restriction on block size or block type, samples outside the block can be used). Proponent’s reported coding gain of 0.18% / 0.45% / 0.34% of BD-rate reduction for AI/RA/LDB. Cross-checker confirms simulation results reported by the proponent for completed QP points.

At the same time, method of CE1-2.3 utilizes HTDF filter during the RDO process. This is an encoder optimization algorithm for HTDF, which is similar to the method proposed in JVET-M0428 for deblocking filter. Simulation results reported in JVET-M0428, shows that coding gain 0.58%/0.71%/0.66% was achieved by this encoder optimization technique. The method of JVET-M0428 was adopted to the VTM4.0 software but was not enabled in the CTC. It is argued by the cross-checker, that for fairness of comparisons, simulation results reported in CE1-2.3 with encoder optimization should be compared against the encoder optimization for deblocking. Alternatively, RDO for HTDF can be disabled for comparison against the VTM4.0 anchor.

To assess performance of the HTDF in conditions similar to deblocking filter (no filtering during the RDO optimization), cross-checker modified CE1-2.3 software by disabling encoder HTDF filtering in RDO.

Achieved simulation results of HTDF w/o RDO, are reported below.

Note: reported time estimates are in-accurate.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.09% | -0.04% | -0.06% | 100% | 88% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.12% | -0.01% | -0.01% | 101% | 94% |
| Class F (optional) |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 | 0.25% | -0.24% | -0.14% | 79% | 74% |
| Class B | 0.19% | -0.24% | -0.24% | 99% | 81% |
| Class C | 0.15% | -0.20% | -0.30% | 98% | 84% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.06% | -0.11% | -0.24% | 97% | 85% |
| Class F (optional) | -0.19% | -0.05% | -0.09% | 98% | 81% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.23% | 0.44% | -0.07% | 79% | 71% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.12% | 0.30% | -0.10% | 98% | 87% |
| Class F (optional) |  |  |  |  |  |