| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0621-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-N0335 (CE2/4-related: unification of MV rounding) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chia-Ming Tsai No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 chia-ming.tsai@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The document reports the results of crosschecking JVET-N0335 proposed by Kwai. JVET-N0335 proposes a method to unify all the motion vector (MV) rounding operations in the VVC to be the same as that used by the MV scaling of the adaptive motion vector prediction (AMVP), subblock based temporal motion vector prediction (SbTMVP), merge mode with motion vector difference (MMVD) and triangle prediction. The verification task was done, and the BD-rate results match exactly with those in JVET-N0335.

# Verification

Kwai provided the source code of their proposed method described in JVET-N0335 [1]. One verification was carried out by setting the following parameters in the provided code:

#define MV_ROUNDING_BASED_ON_TMVP_ROUNDING                   1

The test was conducted under the the common test conditions (CTC) defined in [2] using the VTM4.0 software [3]. Table 1 shows the results using VTM4.0 as the anchor and enabling the proposed method in the test. The BD-rates of the test from our experimental results match exactly those in JVET-N0335. Our runtime comparison may be not accurate.

Table 1.  The results of JVET-N0335 using VTM4.0 as anchor

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.01% | 0.03% | 101% | 101% |
| Class A2 | -0.03% | -0.01% | 0.00% | 101% | 102% |
| Class B | -0.02% | -0.03% | -0.04% | 103% | 101% |
| Class C | -0.04% | -0.07% | -0.06% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.03% | -0.02% | 101% | 101% |
| Class D | -0.07% | 0.03% | -0.01% | 99% | 102% |
| Class F | 0.01% | -0.04% | -0.07% | 99% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | 0.07% | -0.26% | 96% | 98% |
| Class C | -0.04% | 0.06% | -0.40% | 98% | 100% |
| Class E | -0.09% | -0.14% | -0.38% | 98% | 101% |
| Overall | -0.05% | 0.01% | -0.34% | 97% | 99% |
| Class D | -0.14% | -0.26% | -0.74% | 100% | 102% |
| Class F | -0.21% | 0.05% | 0.45% | 97% | 98% |

# References

- X. Xiu, Y.-W. Chen, T.-C. Ma, and X. Wang, “CE2/4-related: unification of MV rounding,” Document of Joint Video Experts Team, JVET-N0335.

- F. Bossen, J. Boyce, K. Sühring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-M1010.

- VTM4.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).