| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0636-v1 |
| --- | --- |

| Title: | Crosscheck of CE1-5.6: Pipeline restriction and DBF modification on LIC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Saurav Bandyopadhyay | Tel: Email: | +1 858 210 4813 saurav.bandyopadhyay@interdigital.com |
| Source: | InterDigital Communications Inc. |  |  |

_____________________________

# Abstract

The document reports the cross-checking results of CE1-5.6 (JVET-N0205) proposed by Panasonic. The CE1-5.6 test proposes to set the block strength to 1 for LIC boundary if at-least one of the blocks uses LIC. The cross-checker reviewed the code as well validated the experimental results and found them to match with CE1-5.6 (JVET-N0205).

# Test1-5.6

Test1-5.6 [1] proposes to set the block strength to 1 for LIC boundary, if at-least one of the blocks uses LIC.

# Experimental Results

Panasonic shared the source code of their proposed method described in CE1-5.6 in git branch CE1-5 [1].

The test is conducted under the the common test conditions (CTC) defined in [2] and are carried out by using the provided software. Table 1 and Table 2 shows the results using VTM4.0 as the anchor [3] and enabling the proposed method in the tests CE1-5.6a and CE1-5.6b respectively. The BD-rates of the test from our experimental results match exactly those in JVET-N0205.

Table 1: The results of CE1-5.6a using VTM4.0 as anchor

|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.64% | 0.24% | 0.04% | 117% | 101% |  |  |  |  |  |
| Class A2 | -0.33% | -0.45% | -0.36% | 111% | 103% |  |  |  |  |  |
| Class B | -0.46% | -0.38% | -0.25% | 111% | 101% | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.32% | -0.28% | -0.30% | 113% | 101% | -0.51% | 0.16% | -0.27% | 116% | 101% |
| Class E |  |  |  |  |  | 0.00% | 0.15% | -0.18% | 103% | 104% |
| Overall | -0.43% | -0.25% | -0.23% | 113% | 101% | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.14% | -0.10% | -0.31% | 112% | 101% | -0.12% | 0.06% | -0.84% | 114% | 102% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 2: The results of CE1-5.6b using VTM4.0 as anchor

|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.54% | -0.34% | -0.64% | 119% | 102% |  |  |  |  |  |
| Class A2 | -0.26% | -0.30% | -0.33% | 111% | 102% |  |  |  |  |  |
| Class B | -0.33% | -0.24% | -0.27% | 113% | 101% | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.29% | -0.30% | -0.33% | 115% | 101% | -0.46% | 0.31% | -0.01% | 119% | 102% |
| Class E |  |  |  |  |  | 0.07% | 0.51% | -0.45% | 104% | 104% |
| Overall | -0.35% | -0.29% | -0.37% | 114% | 102% | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.16% | -0.37% | -0.22% | 114% | 101% | -0.15% | -0.01% | -0.65% | 113% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# References

- J. Ström, S. Ikonin and V. Seregin, “Description of Core Experiment 1 (CE1): Post-prediction and post-reconstruction filtering”, JVET-M1021, Marrakech, MA, 9–18 Jan. 2019.

- F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, Marrakech, MA, 9–18 Jan. 2019.

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 4),” JVET-M1001, Marrakech, MA, 9–18 Jan. 2019.

- K. Abe, T. Toma, J. Li C.-W. Kuo, V. Drugeon, “CE-4 related: Low pipeline latency LIC”, JVET-N0206, Geneva, CH, March 2019.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).